| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0369 WG11 Number: m24616 |
| --- | --- |

| Title: | Compact representation method for quantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianhua Zheng, HiSilicon Jianwen Chen, UCLA Q20, Shi Chuang Ke Ji Shi Fan Yuan No.156 Beiqing Rd.,Hai-Dian District Beijing 100095, P.R. China | Tel: Email: | +86-10-60612141 zhengjianhua@huawei.com |
| Source: | HiSilicon Technologies University of California, Los Angeles |  |  |

_____________________________

# Abstract

The subjective test in the last San Jose meeting shown some proofs that it is possible to apply sub-sampling methods to the larger quantization matrices and represent the larger matrices as smaller sub-sampled matrices. Current HEVC design use a downsampled 8x8 matrix to hold the sub-sampled values of 16x16 or 32x32 matrix. Sub-sampling according to the important of frequency band is considered in this proposal, and a further simple compact representation method for quantization matrices is proposed. Quantization matrix is spitted into one smaller size of values and a simple frequency pattern. This method is further reduce the larger size quantization matrices bits but with small reconstructed error. It can further reduce the APS data sets.

# Description

Current HEVC design use a downsampled 8x8 matrix to hold the sub-sampled values of 16x16 or 32x32 matrix. In decoder, 16x16 and 32x32 matrices are reconstructed from the downsampled 8x8 matrices. One sub-sampled values in the 8x8 matrices stand for a 2x2 neighbor frequency components in 16x16, and 4x4 neighbor frequency components in 32x32. This scheme has not considered the properties difference between 32x32/16x16 transformed blocks and 8x8 transformed blocks when applying the sub-sampling the larger block. In the other hand, the frequency components values are much close to those of neighboring in quantization matrices, especially in the larger size quantization matrix. It is possible to group these frequency components and represent the matrix as a smaller serial of components and its frequency positions. Hence the quantization matrix can be spitted into one smaller size of values and a simple frequency pattern. In additional, frequency band properties for the 8x8 and 16x16, 32x32 quantization matrices can be considered in the frequency components grouping.

## Parameterized representation method

Based on the description above, the parameterized representation method is proposed. The idea is to use a small size of parameters set and simple frequency pattern to represent the large quantization matrices (16x16, 32x32). In low frequency band, high density sub-sampling is used to protect low frequency components values and reduce frequency band mapping errors. While in high frequency band, sparse sub-sampling is used to reduce the matrix coding bits. So for low frequency band, more parameters are used, for low frequency band, few parameters are used. The proposed method can reduce the quantization matrix coding bits and reduce the reconstructed matrix error comparing to the original input user defined matrices.

### Encoding and Decoding

In the encoder, the large matrix is partitioned to several areas, and each area is assigned with a specific parameter. QMs are represented by a few numbers of frequency parameters and a frequency pattern to signal the positions of these frequency parameters.

Fig. 1 encoder

Fig. 2  decoder

The parameterized model can be used to reconstruct the large QM. The frequency pattern can be fixed one or be flexible to the input user matrices. In the following, fixed and flexible pattern method are investigated.

### Method 1

Fixed Parameterized Pattern

Figure 3: Fixed Parameterized Pattern

One parameterization pattern is defined in encoder and decoder sides.

16x16 QM Coding

For 16x16 quantization matrix, each block in pattern (Fig.3) will cover a 2x2 area of the QM. 7 parameters are totally required to represent the QM.

In this case, 16x16 quantization matrix coefficients are represented by 7 parameters. Hence, the number of weighting values in 16x16 matrix is reduced from 256 to 7.

32x32 QM Coding

For 32x32 quantization matrix, each block in pattern (Fig.3) will cover a 4x4 area of the QM. 7 parameters are totally required to represent the QM.

In this case, 32x32 quantization matrix coefficients are represented by 7parameters. Hence, the number of weighting values in 32x32 matrix is reduced from 1024 to 7.

### Method 2

Multiple Parameterized Patterns

Three parameterized patterns are tested.

16x16 QM Coding

For 16x16 quantization matrix, each block in pattern (Fig.3) will cover a 2x2 area of the QM. 7 coefficients and 1 pattern parameters are required to represent the QM.

In this case, 16x16 quantization matrix coefficients are represented by 7 parameters and one 2bit index. Hence, the number of weighting values in 16x16 matrix is reduced from 256 to 8.

32x32 QM Coding

For 32x32 quantization matrix, each block in pattern (Fig.3) will cover a 4x4 area of the QM. 7 coefficients and 1 pattern parameters are totally required to represent the QM.

In this case, 32x32 quantization matrix coefficients are represented by 7 parameters and one 2bit index. Hence, the number of weighting values in 32x32 matrix is reduced from 1024 to 8.

### Parameter Derivation

For one input QM, for the different areas, the average value of the coefficients in the area is used as the parameter value.

For example, for one 16x16 QM with pattern 1, the parameter pl is the average of the coefficients in the red area. For other coefficients, the same method can be used.

## Test software

The simulation software is implemented on the top of HM6 software.

# Experimental Results

The experiments use the sets of quantization matrices available in CE4 test condition, asymmetry and symmetry case are tested.

Table 1  Number of samples

| Num of samples | Ref. | 8x8 downsampled Ref. | Method1 | Method2 |
| --- | --- | --- | --- | --- |
| 16x16 | 256*6= 1536 | 64*6= 384 | 7 | 8 |
| 32x32 | 1024*2= 2048 | 64*2= 128 | 7 | 8 |
| Total | 3584 | 512 | 14 | 8 |

Table 2  QM coded results

|  | Ref. | Method1 | Method2 +pattern1 | Method2 +pattern2 | Method2 ++pattern3 |
| --- | --- | --- | --- | --- | --- |
| Total Bit(4x4,8x8,16x16,32x32) | 9959.667 | 1127 | 1128 | 1138.667 | 1140.333 |
| AvgError |  | 8.983534 | 8.983534 | 9.024791 | 8.390482 |

# Conclusion

Simple compact representation method for quantization matrices is proposed, Sub-sampling according to the important of frequency band is considered. The quantization matrices bits reduce to a few numbers of parameters. It further reduces the larger size quantization matrices bits but with small reconstructed error increasing.

It is suggested to include this simple compact representation method in HM.

# Patent rights declaration(s)

HiSilicon Technologies, Huawei Technologies, UCLA may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).