| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0370 |
| --- | --- |

| Title: | Removal of zigzag scan from quantization matrices coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masato Shima 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of quantization matrices coding using existing non-zigzag scans.  Using the test matrices used in former CE4, the simulation results shows that replacing zigzag scan with raster scan led to about 25% quantization matrices coding bit increase.  In contrast, replacing zigzag scan with diagonal scan (with 4x4 sub-block partitions) led to about 1.1% quantization matrices coding bit increase, and the diagonal scan (including 8x8) led to about 0.6% quantization matrices coding bit reduction.

# Introduction

In most of the cases in former video coding standards, both transformed coefficients and quantization matrix coefficients are scanned using a zigzag scan.  However, that commonality is broken in HM6, as transformed coefficients are scanned with non-zigzag scans (horizontal, vertical, or up-right diagonal scans) while quantization matrix coefficients are scanned with a zigzag scan.  Accordingly, this contribution investigates the effect of applying existing non-zigzag scans for quantization matrix coefficients scanning and coding.

# Description of the tested methods

In this contribution, the following three alternative options to zigzag scan were tested:

Option 1: Raster (horizontal) scan

Option 2: Diagonal scan (with 4x4 sub-block partitions as described in subclause 6.5.3)

Option 3: Diagonal scan (including 8x8, which needs to be newly defined)

Figure 1 shows the 4x4 and 8x8 zigzag scans used for scanning quantization matrices coefficients in HM6.

Figure 1: 4x4 and 8x8 zigzag scans used to scan quantization matrix coefficients in HM6.

## Option 1: Raster scan

The first option is to replace zigzag scan by raster scan described in Figure 2.

Figure 2: 4x4 and 8x8 raster scans which can be used for scanning quantization matrix coefficients

## Option 2: Diagonal scan with 4x4 sub-block partitions

The second option is to replace zigzag scan by diagonal scan with 4x4 sub-block partition as described in Figure 3.

Figure 3: 4x4 and 8x8 diagonal scan with 4x4 sub-block partitions

## Option 3: Diagonal scan including 8x8 scan pattern

The first option is to replace zigzag scan by diagonal scan including 8x8 scan pattern as described in Figure 4.

Figure 4: 4x4 and 8x8 diagonal scan including 8x8 scan pattern

# Experimental results

Table 1 presents the bits needed to code each quantization matrix in the test set used in former CE4.  In order to perform the simulation with HM6.1, the coefficients in every two rows/columns of 16x16 matrices and the coefficients in every four rows/columns of 32x32 matrices are used to create 8x8 samples.

Table 1: Bits needed to code quantization matrices using different scans

| Coded bits | HM6.1 (zigzag) | Option 1 (raster) | Option 2 (diagonal w/ 4x4 sub-block partitions) | Option 3 (diagonal including 8x8 scan pattern) |
| --- | --- | --- | --- | --- |
| Symmetry1 | 2067 | 2731 (132.1%) | 2167 (104.8%) | 2067 (100.0%) |
| Symmetry2 | 1255 | 1667 (132.8%) | 1321 (105.3%) | 1255 (100.0%) |
| Symmetry3 | 2593 | 3321 (128.1%) | 2681 (103.4%) | 2593 (100.0%) |
| Symmetry4 | 2565 | 2993 (116.7%) | 2541 (99.1%) | 2565 (100.0%) |
| Symmetry5 | 1899 | 2491 (131.2%) | 1983 (104.4%) | 1899 (100.0%) |
| Symmetry6 | 2089 | 2709 (129.7%) | 2167 (103.7%) | 2089 (100.0%) |
| Asymmetry1 | 2377 | 3147 (132.4%) | 2375 (99.9%) | 2345 (98.7%) |
| Asymmetry2 | 1397 | 1873 (134.1%) | 1373 (98.3%) | 1371 (98.1%) |
| Asymmetry3 | 2873 | 3679 (128.1%) | 2885 (100.4%) | 2861 (99.6%) |
| Asymmetry4 | 3859 | 3803 (98.5%) | 3813 (98.8%) | 3829 (99.2%) |
| Asymmetry5 | 2165 | 2879 (133.0%) | 2149 (99.3%) | 2127 (98.2%) |
| Asymmetry6 | 2315 | 3031 (130.9%) | 2313 (99.9%) | 2293 (99.0%) |
| Total | 27454 | 34324 (125.0%) | 27768 (101.1%) | 27294 (99.4%) |

# Conclusion

Several options for scanning quantization matrix coefficients are presented.  The first option is to replace zigzag scan by raster scan.  The second one is to replace zigzag scan by diagonal scan with 4x4 sub-block partitions.  The last one is to replace zigzag scan by diagonal scan including 8x8 scan pattern.  Experimental results show that at the cost of quantization matrices coding bits increase of 25%, 1.1%, and reduction of 0.6% respectively, each of the options may remove the zigzag scan table/operation from HEVC design.

It is recommended that one of the presented options to replace zigzag scan be incorporated into the next version of HM as quantization matrix coefficients scanning method of HEVC.

# Patent rights declaration(s)

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Draft 6

The modified or added portions are highlighted with yellow colour.  All of the below options will remove subclause 6.5.4 “Zig-zag scanning array initialization process”

### A.1. Change of subclause 7.4.2.4 “Scaling list semantics” when zigzag scan is replaced by raster scan

The elements of the quantization matrix of size 4x4, ScalingFactor[ 0 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 0 ][ MatrixID ][ 0 ][ y*4+x ] = ScalingList[ 0 ][ MatrixID ][ i ]	(7-1)
with i=0..15 and MatrixID=0..5

where ScanOrder is an array specified in subclause  with 4 as input, x=ScanOrder[1][ i ][ 0 ] and y=ScanOrder[1][ i ][ 1 ]

The elements of the quantization matrix of size 8x8, ScalingFactor[ 1 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 1 ][ MatrixID ][ 0 ][ y*8+x ] = ScalingList[ 1 ][ MatrixID ][ i ]	(7-2)
with i=0..63 and MatrixID=0..5

where ScanOrder is an array specified in subclause  with 8 as input, x=ScanOrder[1][ i ][ 0 ] and y=ScanOrder[1][ i ][ 1 ]

The elements of the quantization matrix of size 16x16, ScalingFactor[ 2 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 2 ][ MatrixID ][ 0 ][ (y*2+j)*16+(x*2+k) ] = ScalingList[ 2 ][ MatrixID ][ i ]	(7-3)
with i=0..63, j=0..1, k=0..1 and MatrixID=0..5

where ScanOrder is an array specified in subclause  with 8 as input, x=ScanOrder[1][ i ][ 0 ] and y=ScanOrder[1][ i ][ 1 ]

ScalingFactor[ 2 ][ MatrixID ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 0 ][ MatrixID ] + 8	(7-4)
with MatrixID=0..5

The elements of the quantization matrix of size 32x32, ScalingFactor[ 3 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 3 ][ MatrixID ][ 0 ][ (y*4+j)*32+(x*4+k) ] = ScalingList[ 3 ][ MatrixID ][ i ]	(7-5)
with i=0..63, j=0..3, k=0..3 and MatrixID=0..1

where ScanOrder is an array specified in subclause  with 8 as input, x=ScanOrder[1][ i ][ 0 ] and y=ScanOrder[1][ i ][ 1 ]

ScalingFactor[ 3 ][ MatrixID ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 1 ][ MatrixID ] + 8	(7-6)
with MatrixID=0..1

### A.2. Change of subclause 7.4.2.4 “Scaling list semantics” when zigzag scan is replaced by diagonal scan

The elements of the quantization matrix of size 4x4, ScalingFactor[ 0 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 0 ][ MatrixID ][ 0 ][ y*4+x ] = ScalingList[ 0 ][ MatrixID ][ i ]	(7-7)
with i=0..15 and MatrixID=0..5

where DiagScan is an array specified in subclause  with 4 as input, x=DiagScan[ i ][ 0 ] and y=DiagScan[ i ][ 1 ]

The elements of the quantization matrix of size 8x8, ScalingFactor[ 1 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 1 ][ MatrixID ][ 0 ][ y*8+x ] = ScalingList[ 1 ][ MatrixID ][ i ]	(7-8)
with i=0..63 and MatrixID=0..5

where DiagScan is an array specified in subclause  with 8 as input, x=DiagScan[ i ][ 0 ] and y=DiagScan[ i ][ 1 ]

The elements of the quantization matrix of size 16x16, ScalingFactor[ 2 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 2 ][ MatrixID ][ 0 ][ (y*2+j)*16+(x*2+k) ] = ScalingList[ 2 ][ MatrixID ][ i ]	(7-9)
with i=0..63, j=0..1, k=0..1 and MatrixID=0..5

where DiagScan is an array specified in subclause  with 8 as input, x=DiagScan[ i ][ 0 ] and y=DiagScan[ i ][ 1 ]

ScalingFactor[ 2 ][ MatrixID ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 0 ][ MatrixID ] + 8	(7-10)
with MatrixID=0..5

The elements of the quantization matrix of size 32x32, ScalingFactor[ 3 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 3 ][ MatrixID ][ 0 ][ (y*4+j)*32+(x*4+k) ] = ScalingList[ 3 ][ MatrixID ][ i ]	(7-11)
with i=0..63, j=0..3, k=0..3 and MatrixID=0..1

where DiagScan is an array specified in subclause  with 8 as input, x=DiagScan[ i ][ 0 ] and y=DiagScan[ i ][ 1 ]

ScalingFactor[ 3 ][ MatrixID ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 1 ][ MatrixID ] + 8	(7-12)
with MatrixID=0..1

### A.3.1. Change of subclause 7.4.2.4 “Scaling list semantics” when zigzag scan is replaced by diagonal scan (including 8x8)

The elements of the quantization matrix of size 4x4, ScalingFactor[ 0 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 0 ][ MatrixID ][ 0 ][ y*4+x ] = ScalingList[ 0 ][ MatrixID ][ i ]	(7-13)
with i=0..15 and MatrixID=0..5

where Diagonal is an array specified in subclause  with 4 as input, x=Diagonal[ i ][ 0 ] and y=Diagonal[ i ][ 1 ]

The elements of the quantization matrix of size 8x8, ScalingFactor[ 1 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 1 ][ MatrixID ][ 0 ][ y*8+x ] = ScalingList[ 1 ][ MatrixID ][ i ]	(7-14)
with i=0..63 and MatrixID=0..5

where Diagonal is an array specified in subclause  with 8 as input, x=Diagonal[ i ][ 0 ] and y=Diagonal[ i ][ 1 ]

The elements of the quantization matrix of size 16x16, ScalingFactor[ 2 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 2 ][ MatrixID ][ 0 ][ (y*2+j)*16+(x*2+k) ] = ScalingList[ 2 ][ MatrixID ][ i ]	(7-15)
with i=0..63, j=0..1, k=0..1 and MatrixID=0..5

where Diagonal is an array specified in subclause  with 8 as input, x=Diagonal[ i ][ 0 ] and y=Diagonal[ i ][ 1 ]

ScalingFactor[ 2 ][ MatrixID ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 0 ][ MatrixID ] + 8	(7-16)
with MatrixID=0..5

The elements of the quantization matrix of size 32x32, ScalingFactor[ 3 ][ MatrixID ][ 0 ][ ] is derived as follows:

ScalingFactor[ 3 ][ MatrixID ][ 0 ][ (y*4+j)*32+(x*4+k) ] = ScalingList[ 3 ][ MatrixID ][ i ]	(7-17)
with i=0..63, j=0..3, k=0..3 and MatrixID=0..1

where Diagonal is an array specified in subclause  with 8 as input, x=Diagonal[ i ][ 0 ] and y=Diagonal[ i ][ 1 ]

ScalingFactor[ 3 ][ MatrixID ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 1 ][ MatrixID ] + 8	(7-18)
with MatrixID=0..1

### A.3.2. Change of subclause 6.5.4 “Zig-zag scanning array initialization process” when zigzag scan is replaced by diagonal scan (including 8x8)

### 6.5.4  Diagonal scanning array initialization process

Input to this process is a block size blkSize.

Output of this process is the array Diagonal[ sPos ][ sComp ]. The array index sPos specify the scan position ranging from 0 to ( blkSize * blkSize ) − 1. The array index sComp equal to 0 specifies the horizontal component and the array index sComp equal to 1 specifies the vertical component. The array Diagonal is derived as follows.

ZigZag[ 0 ][ 0 ]  =  0
ZigZag[ 0 ][ 1 ]  =  0
i  =  0
x  =  0
y  =  0
stopLoop  =  ( blkSize  = = 1 ) ? TRUE : FALSE
while( !stopLoop ) {
	while( y  >=  0 ) {
		if(x  >=  0  &&  x  <  blkSize  &&  y  >=  0  &&  y  <  blkSize ) {
			Diagonal[ i ][ 0 ]  =  x
			Diagonal[ i ][ 1 ]  =  y
			i++
		}
		x− −
		y++
	}
	y  =  x

x  =  0
 	while( y  >=  0 ) {
		if(x  >=  0  &&  x  <  blkSize  &&  y  >=  0  &&  y  <  blkSize ) {
			ZigZag[ i ][ 0 ]  =  x
			ZigZag[ i ][ 1 ]  =  y
			i++
		}
		x++
		y− −
	}
	y  =  0
	if( i  >=  blkSize  *  blkSize ) 
		stopLoop  =  TRUE
}