| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, April 27 – May 4, 2012 | Document: JCTVC-I371 |
| --- | --- |

| Title: | Luma Context reduction for Last Coefficient Position Coding in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xue Fang Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2723 X.Fang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document describes a proposal related to the luma context reduction for last coefficient position coding. Several context index assignments are proposed. The first version uses same contexts for 16x16 and 32x32 Luma TUs. The total number of contexts for Luma (X,Y) is reduced from 30 to 20 with 0.01% Y-BD rate change. On the top of the first version, the second version further reduces the number of contexts for Luma (X,Y) from 30 to less than 18 by sharing some contexts for TUs of 4x4 and 8x8 with the similar Y-BD rate change. Cross-Check will be provided by Qualcomm’s JCTVC-I0486.

# Introduction

Last coefficient coding process codes the position (X, Y) of last nonzero transform coefficient of a TU with respect to a pre-determining scan pattern/coding order, so decoder can skip decoding of consecutive number of zero coefficients at the end of TU.

In the current HEVC CD [1] (Table 1, Table 2 show below), there are a total of 18 contexts per X or Y per picture type, 15 contexts for luma highlighted as gray and 3 contexts for chroma as show in orange. Specifically, contexts 0 -17 are used for picture type I, 18-35 for picture type P, and 36-53 for picture type B.

Table 1 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 57 | 72 | 55 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 54 |
|  | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

Table 2 – Values of variable initValue for last_significant_coeff_y_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_y_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 57 | 72 | 55 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 54 |
|  | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

Specifically for luma context assignment, 4x4 TU uses 3 contexts of 0-2 for X or Y, 8x8 TU uses 4 contexts of 2 and 3-5 for X or Y, and 16x16 TU has 5 contexts of 2 and 6-9 for X or Y, and 32x32 TU has 6 contexts of 2 and 10-14 for X or Y. Context indices are assigned to each bin of X or Y in luma as follows;

Table 3 - Context Assignment for Luma

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 4 | 5 | 5 | 2 |  |  |  |  |
| TU 16x16 | 6 | 7 | 8 | 8 | 9 | 9 | 2 |  |  |
| TU 32x32 | 10 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 2 |

Note that in order to obtain last_significant_coefficient_x_prefix_ctxIdx  and last_significant_coefficient_y_prefix_ctxIdx, context indices in Table 3 need to be added to the corresponding picture type offsets.

# Proposal

We propose to reduce the total number of contexts used for last significance coefficient coding of a luma TU. This can be achieved by assigning the same contexts to a few consecutive bins of X or Y per TU size. For example, the first and the second bins of X or Y for a TU size may be assigned a same context. The context reduction can also be achieved by assigning the same contexts to certain bins of X or Y for different TU sizes. These certain bins may be in relatively similar significant bit positions of X or Y for different TU sizes. For example, the first bin of X or Y in 4x4 TU and 8x8 TU may use a same context.

Four examples of context index assignments are proposed as described below.

## First Proposal of context reduction from 30 to 20

### Proposal

The first proposal uses 6 contexts (0-5) for 4x4 TU and 8x8 TU as in the current CD, and 5 (2, 6-9) contexts for 16x16 TU and 32x32 TU, thereby reducing the total number of contexts from 15 to 10 for each picture type’s luma as show in table 4.  Gray areas of ctxIdx are the contexts used by luma, and orange areas of ctxIdx are assigned for chroma. The total number of contexts for Luma (X,Y) is reduced from 30 to 20.

Table 4 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 54 | 70 | 53 |
|  | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| initValue | 57 | 72 | 55 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 54 | 70 | 54 |
|  | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 54 | 70 | 69 |

Note: In table 4, original column 10-14 of table 1 or 2 are removed and the original column 15-17 of table 1 or 2 are shifted left accordingly.  Contexts initializations are not changed.

Context index assignment for each bin of X or Y in luma is as follows;

Table 5 - Context Assignment for Luma

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 4 | 5 | 5 | 2 |  |  |  |  |
| TU 16x16 | 6 | 7 | 8 | 8 | 9 | 9 | 2 |  |  |
| TU 32x32 | 6 | 6 | 7 | 7 | 8 | 8 | 9 | 9 | 2 |

### Test Results of Proposal 1

Table 6 - Proposal 1 Main Profile for Context Reduction From 30 to 20 on (X,Y)

| BD-rate | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.02% | 0.01% | -0.01% | 0.00% | -0.09% |  |  |  |
| Class B | 0.02% | 0.00% | -0.01% | 0.03% | -0.11% | 0.07% | 0.03% | -0.32% | 0.20% |
| Class C | 0.00% | 0.00% | 0.05% | -0.02% | -0.03% | -0.03% | 0.01% | 0.19% | 0.01% |
| Class D | 0.01% | 0.03% | 0.00% | 0.01% | -0.08% | 0.14% | 0.05% | 0.21% | 0.35% |
| Class E | 0.00% | 0.04% | 0.00% |  |  |  | 0.11% | -0.74% | 0.10% |
| All | 0.01% | 0.01% | 0.01% | 0.01% | -0.06% | 0.04% | 0.05% | -0.14% | 0.17% |
| Class F | 0.01% | 0.05% | -0.01% | -0.03% | -0.03% | -0.13% | -0.12% | 0.49% | -0.15% |

Table 7 – Proposal 1 HE10 Profile for Context Reduction From 30 to 20 on (X,Y)

| BD-rate | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.04% | -0.08% | 0.01% | 0.10% | -0.01% |  |  |  |
| Class B | 0.02% | -0.03% | -0.03% | 0.00% | -0.01% | 0.01% | -0.02% | -0.43% | 0.26% |
| Class C | 0.01% | -0.02% | 0.05% | 0.01% | -0.12% | -0.10% | -0.01% | 0.02% | 0.07% |
| Class D | 0.01% | -0.02% | 0.02% | -0.01% | -0.05% | -0.23% | 0.00% | -0.61% | -0.34% |
| Class E | 0.00% | -0.05% | 0.02% |  |  |  | -0.02% | 0.66% | -0.51% |
| All | 0.01% | -0.03% | 0.00% | 0.00% | -0.03% | -0.09% | -0.01% | -0.16% | -0.08% |
| Class F | 0.01% | 0.05% | -0.04% | 0.01% | -0.16% | 0.06% | 0.08% | 0.18% | 0.71% |

## Second Proposal of context reduction from 30 to lest then 18

### Proposal 2A of context reduction from 30 to 18

This proposal uses 5 contexts (0-4) for 4x4 TU, 8x8 TU, and 6 contexts (2, 5-8) for 16x16 TU and 32x32 TU, thereby reducing the total number of contexts from 15 to 9 of each picture type for luma. The second version further reduces the number of contexts for Luma (X,Y) from 30 to 18 by sharing some contexts for TUs of 4x4 and 8x8.

Table 8 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx

| Initialisation variable | last_significant_coeff_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 72 | 72 | 71 | 72 | 104 | 88 | 89 | 59 | 73 | 54 | 70 | 53 |
|  | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| initValue | 57 | 72 | 55 | 72 | 57 | 88 | 73 | 73 | 72 | 54 | 70 | 54 |
|  | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 88 | 72 | 71 | 72 | 57 | 88 | 73 | 73 | 72 | 54 | 70 | 69 |

Note: In table 8, original columns of 5 and 10-14 of table 1 or 2 are removed and the rest of original column of table 1 or 2 are shifted left accordingly.  No change on initialization of contexts.

Context index assignment for each bin of X or Y in luma is shown in table 9 as follows;

Table 9 Context Assignment for Luma

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 3 | 4 | 4 | 2 |  |  |  |  |
| TU 16x16 | 5 | 6 | 7 | 7 | 8 | 8 | 2 |  |  |
| TU 32x32 | 5 | 5 | 6 | 6 | 7 | 7 | 8 | 8 | 2 |

### Test Results of Proposal 2A

Table 10 - Proposal 2A Main Profile for Context Reduction From 30 to 18 on (X,Y)

| BD-rate | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.03% | 0.00% | 0.00% | -0.14% |  |  |  |
| Class B | 0.01% | 0.01% | 0.00% | 0.05% | -0.07% | 0.26% | 0.05% | -0.27% | -0.17% |
| Class C | 0.00% | 0.01% | 0.06% | 0.01% | 0.04% | 0.03% | 0.01% | 0.28% | 0.25% |
| Class D | 0.00% | 0.06% | -0.02% | -0.03% | -0.18% | -0.02% | 0.01% | 0.45% | -0.03% |
| Class E | 0.00% | -0.01% | 0.03% |  |  |  | 0.13% | -0.06% | -0.24% |
| All | 0.00% | 0.02% | 0.02% | 0.01% | -0.06% | 0.07% | 0.04% | 0.09% | -0.04% |
| Class F | 0.01% | -0.02% | 0.01% | -0.01% | -0.14% | -0.05% | -0.14% | -0.04% | -0.53% |

Table 11 – Proposal 2A HE10 Profile for Context Reduction From 30 to 18 on (X,Y)

| BD-rate | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.05% | -0.04% | 0.00% | 0.16% | 0.03% |  |  |  |
| Class B | 0.01% | -0.02% | -0.04% | 0.00% | 0.05% | -0.14% | 0.01% | -0.17% | 0.41% |
| Class C | 0.01% | 0.02% | 0.07% | -0.03% | 0.03% | -0.03% | -0.01% | 0.28% | 0.14% |
| Class D | 0.00% | 0.00% | 0.05% | 0.03% | 0.23% | -0.13% | 0.02% | 0.04% | 0.41% |
| Class E | 0.00% | 0.02% | 0.01% |  |  |  | -0.04% | -0.17% | 0.62% |
| All | 0.00% | 0.00% | 0.01% | 0.00% | 0.11% | -0.09% | 0.00% | 0.00% | 0.38% |
| Class F | 0.02% | 0.06% | 0.16% | 0.00% | -0.14% | -0.06% | 0.04% | -1.37% | 0.12% |

### Proposal 2B of context reduction from 30 to 14

This proposal uses 3 contexts (0-2) for 4x4 TU, 8x8 TU, and 5 (2, 3-6) contests for 16x16 TU and 32x32 TU, thereby reducing the total number of contexts  from 15 to 7 of each picture type for luma. The second version further reduces the number of contexts for Luma (X,Y) from 30 to 14 by sharing some contexts for TUs of 4x4 and 8x8.

Table 12 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx

| Initialisation variable | last_significant_coeff_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| initValue | 72 | 72 | 71 | 88 | 89 | 59 | 73 | 54 | 70 | 53 |
|  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| initValue | 57 | 72 | 55 | 88 | 73 | 73 | 72 | 54 | 70 | 54 |
|  | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| initValue | 88 | 72 | 71 | 88 | 73 | 73 | 72 | 54 | 70 | 69 |

Note: In table 12, original columns of 3-6 and 10-14 of table 1 or 2 are removed and the rest of original column of table 1 or 2 are shifted left accordingly.  No change on initialization of contexts.

Context index assignment for each bin of X or Y in luma is shown in table 7 as follows;

Table 13 Context Assignment for Luma

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 0 | 0 | 1 | 1 | 2 |  |  |  |  |
| TU 16x16 | 3 | 4 | 5 | 5 | 6 | 6 | 2 |  |  |
| TU 32x32 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 2 |

### Test Results of Proposal 2B

Table 14 – Proposal 2B Main Profile for Context Reduction From 30 to 14 on (X,Y)

| BD-rate | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.06% | -0.01% | 0.02% | 0.08% | 0.14% | 0.02% |  |  |  |
| Class B | 0.07% | -0.05% | -0.03% | 0.05% | -0.09% | 0.08% | 0.02% | -0.20% | -0.07% |
| Class C | 0.00% | -0.02% | 0.03% | 0.01% | -0.03% | -0.15% | 0.01% | 0.05% | -0.06% |
| Class D | 0.01% | 0.07% | -0.02% | -0.04% | 0.01% | 0.07% | 0.01% | 0.34% | 0.32% |
| Class E | 0.05% | 0.00% | -0.02% |  |  |  | 0.05% | -0.08% | 0.25% |
| All | 0.04% | 0.00% | -0.01% | 0.02% | -0.02% | 0.01% | 0.02% | 0.02% | 0.09% |
| Class F | 0.13% | 0.13% | 0.09% | 0.07% | 0.07% | 0.04% | -0.05% | 0.12% | 0.05% |

Table 15 – Proposal 2B HE10 Profile for Context Reduction From 30 to 14 on (X,Y)

| BD-rate | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.06% | -0.06% | -0.04% | 0.02% | 0.15% | 0.11% |  |  |  |
| Class B | 0.05% | -0.06% | -0.05% | 0.00% | -0.14% | -0.13% | -0.03% | -0.03% | -0.09% |
| Class C | 0.00% | -0.01% | 0.00% | -0.01% | -0.09% | -0.04% | -0.02% | 0.09% | 0.18% |
| Class D | 0.01% | -0.03% | 0.04% | 0.04% | 0.12% | 0.18% | -0.01% | 0.09% | 0.29% |
| Class E | 0.05% | -0.07% | -0.04% |  |  |  | -0.07% | -0.03% | -0.42% |
| All | 0.03% | -0.04% | -0.01% | 0.01% | -0.02% | 0.01% | -0.03% | 0.03% | 0.01% |
| Class F | 0.13% | 0.15% | 0.09% | 0.10% | -0.03% | 0.12% | 0.06% | 1.26% | 1.65% |

# Draft Text for Proposal 2A of context reduction from 30 to 18

#### 9.2.1.1 Initialization process for context variables

Table 9-26 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 57 | 72 | 55 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 54 |
|  | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

New Table 9-26 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx

| Initialisation variable | last_significant_coeff_x_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 72 | 72 | 71 | 72 | 104 | 88 | 89 | 59 | 73 | 54 | 70 | 53 |
|  | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| initValue | 57 | 72 | 55 | 72 | 57 | 88 | 73 | 73 | 72 | 54 | 70 | 54 |
|  | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 88 | 72 | 71 | 72 | 57 | 88 | 73 | 73 | 72 | 54 | 70 | 69 |

Table 9-27 – Values of variable initValue for last_significant_coeff_y_prefix ctxIdx

| Initialisation variable | last_significant_coefficient_y_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 72 | 72 | 71 | 72 | 104 | 89 | 88 | 89 | 59 | 73 | 89 | 106 | 60 | 59 | 43 | 54 | 70 | 53 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 57 | 72 | 55 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 54 |
|  | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| initValue | 88 | 72 | 71 | 72 | 57 | 72 | 88 | 73 | 73 | 72 | 103 | 73 | 89 | 73 | 57 | 54 | 70 | 69 |

New Table 9-27 – Values of variable initValue for last_significant_coeff_y_prefix ctxIdx

| Initialisation variable | last_significant_coeff_y_prefix ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| initValue | 72 | 72 | 71 | 72 | 104 | 88 | 89 | 59 | 73 | 54 | 70 | 53 |
|  | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| initValue | 57 | 72 | 55 | 72 | 57 | 88 | 73 | 73 | 72 | 54 | 70 | 54 |
|  | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| initValue | 88 | 72 | 71 | 72 | 57 | 88 | 73 | 73 | 72 | 54 | 70 | 69 |

##### 9.2.3.1.2 Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

Inputs to this process are the binIdx, the colour component index cIdx, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

Table 9-1 – Specifcation of lastCtx[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 2 | 2 | 6 | 7 | 8 | 8 |
| i | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| lastCtx[ i ] | 9 | 9 | 2 | 2 | 10 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 2 | 2 |

When cIdx is equal to 0, ctxIdxInc is spefified in Table 9-39

Table 9-2 – Specification of ctxIdxInc for cIdx equal to 0

| binIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 ctxIdxInc | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 ctxIdxInc | 3 | 3 | 4 | 4 | 2 |  |  |  |  |
| TU 16x16 ctxIdxInc | 5 | 6 | 7 | 7 | 8 | 8 | 2 |  |  |
| TU 32x32 ctxIdxInc | 5 | 5 | 6 | 6 | 7 | 7 | 8 | 8 | 2 |

The vector lastCtx[ i ] is specified in           Table 9-1 and ctxIdxInc is derived as follows.

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[( ( log2TrafoWidth − 2 )*( log2TrafoWidth + 1 ) ) + binIdx ]	(9-1)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( ( log2TrafoHeight − 2 )*( log2TrafoHeight + 1 ) ) + binIdx ]	(9-2)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  (binIdx >>( log2TrafoWidth − 2 ) )+ 15 9	(9-3)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  (binIdx >>( log2TrafoWidth − 2 ) )+ 15 9	(9-4)

# Conclusions

This contribution proposes to reduce the number of context used for last coefficient position coding of luma TU. Three specific proposals were given and their simulation results were reported. Those proposals reduce the number of contexts for luma (X, Y) from 30 to 20 (33%), 18 (40%), 14 (50%), with 0.01% Y-BD rate change. Therefore, it is recommended to adopt one of the proposed context reduction methods into the next HEVC CD. Thanks Qualcomm for the help of cross check!

# Reference

[1] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification Working Draft 6” JCTVC-H1003.

# Patent Rights Declaration(s)

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).