| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0372 |
| --- | --- |

| Title: | Zone Partition for Significance Map Coding of Large TU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Ying Kung Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2350 viviankung@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution document proposes to merging zone 1 and zone 2 for level 0 significance map coding of luma transform units (TU). The experimental results show that there is less than 0.1% loss in both intra and inter conditions.

# Introduction

In the current JCT-VC reference code HM6.0 [1], the positions of non-zero coefficients in a TU are specified by significance maps, one significance map per component. Further, in HM6.0[1], a two–level significance map (level zero L0 and level one L1) is used for large TU of size 16x16, 32x32, 16x4, 4x16, 32x8, and 8x32. Level 1 significance map is used to code significance element at subblock level within a TU and level 0 significance map is used to code significance element at pixel level within a subblock.

In HM6.0, significance map of luma L0 is further partitioned into four zones and significance map of chroma L0 into three zones, where each zone is assigned with 1 or 3 contexts. Fig. 1 (a) shows four zones for significance map of luma L0.  The detailed context assignment for luma L0 is described as follows:

- 1 context for zone 0.

- 3 contexts for zone 1 (dc subblock):

  - a first context is used if there is no non-zero coefficient in neighborhood;

  - a second context is used if there are 1 or 2 non-zero coefficients in neighborhood;

  - a third context is used if there are more than 3 non-zero coefficients in neighborhood.

- 3 contexts for zone 2: the same derivation rule as with (2), but these 3 contexts are different from those in (2).

- 1 context for zone 3, the context is same as the first context in zone 2

- Threshold for partitioning of zone 1 and zone 2 is determined based on whether or not it is a first DC sub-block:

  - Zone 1 is determined if subBlockX+subBlockY = 0

  - Zone 2 is determined if subBlockX+subBlockY > 0

- Threshold for partitioning of zone 2 and zone 3 (thresholdHighFreq) is determined based on the formula:

  - Zone 3 is determined if subBlockX+subBlockY  >=  thresholdHighFreq, 
    where thresholdHighFreq = 3*max(width, height)>>4

# Zone Partition for Significance Map Coding of Large TU

We propose to merging zone 1 and zone 2 for significance map coding of luma L0, as shown in Fig 1 (b). There are three advantages associated with this merging. The first is zone 1 and zone 2 for significance map coding of luma L0 can utilize the same contexts, reducing the number of contexts. The second is no need to check whether an element is in zone 1 and zone 2 during significance map coding. The third is harmonization of the zone separation of luma TU and chroma TU, which makes it possible to use the same logic for both luma TU and chroma TU. The detailed context assignment of luma L0 is described as follows:

- 1 context for zone 0.

- 3 contexts for zone 1:

- a first context is used if there is no non-zero coefficient in neighborhood;

- a second context is used if there are 1 or 2 non-zero coefficients in neighborhood;

- a third context is used if there are more than 3 non-zero coefficients in neighborhood.

- 1 context for zone 2, the context is same as the first context in zone 1

- Threshold for partitioning of zone 1 and zone 2 is determined based on whether or not it is a first DC sub-block:

  - Zone 1 is determined if subBlockX+subBlockY = 0

- Threshold for partitioning of zone 2 and zone 3 (thresholdHighFreq) is determined based on the formula:

  - Zone 3 is determined if subBlockX+subBlockY  >=  thresholdHighFreq, 
    where thresholdHighFreq = 3*max(width, height)>>4

  - Original					(b) Proposed

Fig. 1 Contexts for significance map coding of large transform unit, showing four zones with the original separation between zone 1 and 2 in (a) and the proposed separation between zone 1 and 2 in (b)

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 1. Detailed results are included in the attachment (JCTVC-Ixxx.xls).

Table 1 Simulation results

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.09% | 0.04% | 0.08% | 0.10% | 0.02% | 0.06% |
| Class B | 0.10% | 0.02% | 0.05% | 0.11% | 0.06% | 0.04% |
| Class C | 0.02% | -0.02% | 0.03% | 0.03% | 0.00% | 0.00% |
| Class D | 0.02% | 0.02% | 0.02% | 0.02% | -0.02% | 0.04% |
| Class E | 0.04% | -0.04% | -0.01% | 0.03% | -0.01% | -0.02% |
| Overall | 0.06% | 0.01% | 0.04% | 0.06% | 0.01% | 0.03% |
|  | 0.06% | 0.01% | 0.04% | 0.06% | 0.02% | 0.03% |
| Class F | 0.01% | 0.26% | 0.09% | 0.03% | -0.01% | -0.12% |
| Enc Time[%] | 71% |  |  | 70% |  |  |
| Dec Time[%] | 75% |  |  | 72% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.27% | -0.65% | -0.49% | 0.19% | 0.32% | 0.01% |
| Class B | 0.06% | -0.15% | 0.02% | 0.06% | -0.05% | -0.01% |
| Class C | -0.02% | 0.06% | -0.10% | 0.02% | -0.15% | -0.15% |
| Class D | -0.02% | -0.19% | 0.03% | 0.01% | -0.02% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | 0.07% | -0.23% | -0.13% | 0.07% | 0.02% | -0.04% |
|  | -0.12% | -0.23% | -0.18% | 0.05% | 0.01% | -0.02% |
| Class F | -0.02% | 0.00% | -0.07% | -0.04% | -0.35% | 0.00% |
| Enc Time[%] | 80% |  |  | 80% |  |  |
| Dec Time[%] | 76% |  |  | 72% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.05% | -0.43% | 0.14% | -0.03% | -0.29% | -0.07% |
| Class C | 0.00% | 0.11% | -0.19% | 0.01% | 0.09% | 0.11% |
| Class D | 0.07% | -0.18% | 0.35% | -0.01% | -0.50% | -0.18% |
| Class E | 0.05% | -1.01% | -1.35% | -0.03% | 0.09% | -0.02% |
| Overall | 0.04% | -0.34% | -0.17% | -0.02% | -0.18% | -0.04% |
|  | 0.04% | -0.34% | -0.16% | -0.01% | -0.14% | -0.10% |
| Class F | -0.15% | -0.06% | -0.02% | 0.03% | 0.37% | 0.75% |
| Enc Time[%] | 81% |  |  | 83% |  |  |
| Dec Time[%] | 79% |  |  | 75% |  |  |

Motorola Mobility Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).