| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0373 |
| --- | --- |

| Title: | AHG5- context derivation clean-up for significance map |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen, Joel Sole, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-651-8028 cjianle@qualcomm.com joels@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal provides a simplified context derivation for significance map coding by 1) applying the HM6.0 4×4 luma look-up table to all small TUs (4×4, 8×8,  luma and chroma); 2) sharing DC component context for all TU sizes. Experimental results show that BD-rate changes in common test conditions are 0.02% for AI-Main and AI-HE10, 0.00% for RA-Main and 0.01%RA-HE10, 0.01% for LB-Main and 0.00% for LB-HE10. The results also show that there is no performance change at the low QP test condition (12, 17, 22, and 27).

# Technical description

In current HM6.0, the contexts of 4×4 and 8×8 significant map are derived by using three look-up tables: one for 4×4 luma, one for 4×4 chroma and one shared by 8×8 luma and chroma [1].

This contribution proposes to reuse the current HM6.0 4×4 luma look-up table to 4×4 chroma, 8×8 luma and 8×8 chroma. To map the position between 8×8 block and 4×4 look-up table, down-scaling is applied as it is currently done in HM6.0, where a 2:1 down-scaling is performed to the position coordinates of the 8×8 block in both horizontal and vertical directions to get the corresponding position of a 4×4 block. This down-scaled position is used as an input of the 4×4 luma look-up table to derive the context.

This contribution also proposes the sharing of the DC component significant map context among all TU sizes. Four context models are saved with the DC component context sharing and no BD-rate loss is observed.

In summary, this contribution proposes two modifications to the significant map context derivation:

- Reusing HM6.0 4×4 luma look-up table for 4×4 chroma, 8×8 luma and 8×8 chroma blocks

- Sharing context of DC among all block sizes (4×4 to 32×32), respectively for luma and chroma

# Experimental results

The proposed method is implemented on HM6.0 and simulations are run under the common test conditions [2]. The performance is compared to HM6.0 anchor in terms of Bjøntegaard Delta (BD) bit-rate. Tab.1 shows the experimental results of common test configuration and Tab.2 shows the results of low QP test condition (QP = 12, 17, 22, 27).

Tab.1. Common condition results

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.01% | 0.07% | 0.11% | -0.01% | 0.26% | 0.30% |
| Class B | 0.02% | 0.13% | 0.15% | 0.01% | 0.12% | 0.10% |
| Class C | 0.02% | 0.07% | 0.09% | 0.02% | -0.03% | 0.04% |
| Class D | 0.03% | 0.09% | 0.06% | 0.03% | -0.05% | 0.02% |
| Class E | 0.04% | -0.02% | 0.03% | 0.05% | -0.08% | 0.10% |
| Overall | 0.02% | 0.07% | 0.09% | 0.02% | 0.05% | 0.11% |
|  | 0.02% | 0.08% | 0.09% | 0.02% | 0.06% | 0.11% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.02% | 0.09% | 0.01% | -0.01% | 0.72% | 1.11% |
| Class B | 0.02% | 0.00% | 0.30% | 0.02% | -0.01% | 0.09% |
| Class C | -0.02% | 0.10% | 0.12% | -0.01% | -0.17% | -0.03% |
| Class D | -0.01% | 0.11% | 0.21% | 0.04% | 0.02% | -0.01% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.07% | 0.19% | 0.01% | 0.13% | 0.28% |
|  | 0.00% | 0.06% | 0.17% | 0.02% | 0.13% | 0.24% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.04% | -0.37% | 0.10% | 0.02% | -0.03% | 0.35% |
| Class C | 0.00% | 0.15% | 0.19% | -0.01% | 0.19% | -0.10% |
| Class D | 0.01% | -0.13% | -0.24% | 0.01% | 0.08% | 0.32% |
| Class E | -0.02% | -1.30% | 0.98% | -0.01% | -0.06% | 0.54% |
| Overall | 0.01% | -0.35% | 0.20% | 0.00% | 0.05% | 0.26% |
|  | 0.01% | -0.29% | 0.10% | 0.00% | 0.06% | 0.18% |

Tab.2. Results of QPs(12, 17, 22, 27)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.03% | 0.12% | 0.20% | -0.09% | 0.34% | 0.51% |
| Class B | 0.04% | 0.21% | 0.21% | 0.03% | 0.23% | 0.21% |
| Class C | 0.01% | 0.06% | 0.05% | 0.01% | 0.02% | 0.03% |
| Class D | 0.01% | 0.06% | 0.08% | 0.02% | 0.02% | 0.03% |
| Class E | 0.02% | 0.03% | 0.07% | 0.01% | 0.01% | 0.07% |
| Overall | 0.01% | 0.10% | 0.12% | 0.00% | 0.13% | 0.18% |
|  | 0.02% | 0.11% | 0.12% | 0.00% | 0.10% | 0.13% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.01% | 0.09% | 0.15% | -0.02% | 0.48% | 0.53% |
| Class B | 0.01% | 0.13% | 0.21% | 0.03% | 0.04% | 0.10% |
| Class C | -0.01% | 0.08% | 0.11% | -0.01% | 0.11% | 0.10% |
| Class D | -0.02% | 0.07% | 0.04% | 0.00% | 0.04% | 0.05% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | 0.10% | 0.13% | 0.00% | 0.16% | 0.19% |
|  | -0.01% | 0.08% | 0.13% | 0.00% | 0.13% | 0.20% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.15% | 0.13% | -0.01% | 0.06% | 0.34% |
| Class C | 0.00% | 0.17% | -0.02% | 0.00% | 0.18% | 0.13% |
| Class D | 0.02% | 0.11% | -0.07% | 0.02% | 0.11% | 0.12% |
| Class E | -0.01% | -0.51% | -0.09% | -0.06% | -0.62% | 0.88% |
| Overall | 0.00% | 0.02% | 0.00% | -0.01% | -0.02% | 0.33% |
|  | 0.00% | -0.01% | -0.01% | 0.00% | 0.00% | 0.39% |

# Conclusion

Comparing to the current HM6.0, a single look-up table instead of three is used for 4×4 and 8×8 blocks for both luma and chroma and 3 contexts are reduced in the proposed method. Experimental results show no noticeable change in performance. It is proposed to adopt the simplified significant map context derivation method in HM7.0 and HEVC CD/DIS.

# References

[1] B. Bross, W.-J. Han, J. R. Ohm, G. J. Sullivan, and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6” Doc. JCTVC-H1003, San Jose, Feb, 2012.

[2] F. Bossen, “Common test conditions”, JCTVC-H1100, San Jose, Feb, 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Change to the CD text

Table 9-1 – Values of variable initValue for significant_coeff_flag ctxIdx

| Initialization variable | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 111 | 111 | 125 | 110 | 110 | 94 | 124 | 108 | 124 | 139 | 139 | 139 | 168 | 124 | 138 | 124 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initValue | 138 | 107 | 107 | 125 | 141 | 179 | 153 | 125 | 140 | 139 | 182 | 182 | 152 | 136 | 152 | 136 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initValue | 153 | 182 | 137 | 149 | 192 | 152 | 224 | 136 | 31 | 136 | 136 | 139 | 111 | 155 | 154 | 139 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initValue | 153 | 139 | 123 | 123 | 63 | 153 | 153 | 153 | 152 | 152 | 152 | 137 | 152 | 137 | 122 | 166 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| initValue | 183 | 140 | 136 | 153 | 154 | 170 | 153 | 123 | 123 | 107 | 121 | 107 | 121 | 167 | 153 | 167 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| initValue | 136 | 149 | 107 | 136 | 121 | 122 | 91 | 151 | 183 | 140 | 170 | 154 | 139 | 153 | 139 | 123 |
|  | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| initValue | 123 | 63 | 124 | 153 | 153 | 152 | 152 | 152 | 137 | 152 | 137 | 137 | 166 | 183 | 140 | 136 |
|  | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| initValue | 153 | 154 | 170 | 153 | 138 | 138 | 122 | 121 | 122 | 121 | 167 | 153 | 167 | 136 | 121 | 122 |
|  | 128 | 129 | 130 | 131 | 132 | 133 | 134 |  |  |  |  |  |  |  |  |  |
| initValue | 136 | 121 | 122 | 91 | 151 | 183 | 140 |  |  |  |  |  |  |  |  |  |

##### Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap [ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap [ (yC << 2) + xC ]	(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap [ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 0 : 9 + ctxIdxMap [ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

-

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  0		(9-4)

- Otherwise, if (xC>>2) + (yC>>2) is less than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-5)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-6)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-7)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-8)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-9)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-10)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 21		(9-11)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 18( (cIdx > 0) ? 18 : 21 )	(9-12)

- Otherwise( (xC>>2) + (yC>>2) is equal to or greater than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)) ), sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 18: 21

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-13)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  24  +  sigCtx		(9-14)

Table 9-2 – Specifcation of ctxIdxMap[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap[ i ] | 0 | 1 | 4 | 5 | 2 | 3 | 4 | 5 | 6 | 6 | 8 | 8 | 7 | 7 | 8 |

-