| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0374 |
| --- | --- |

| Title: | 4 Bin Parallel Scheme for Significance Map Coding and Level Coding (H0228) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Ying Kung Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2350 viviankung@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution document proposes a 4 bin parallel scheme for H0228. In this proposal, diagonal scan is maintained for coding across subblock. However, horizontal scan is followed for coding within a 4x4 subblock. A 4 bin parallel friendly mask for context index derivation is also given.

# Introduction

H0228 [1] proposed a cleanup for transform coefficient coding of the current HEVC design. It achieves the simplification and harmonization by coding the transform coefficients level and significance value in a single loop and applying the same simple rules for context selection and the Rice parameter derivation for different sizes of transform units (TU).  In addition, H0228 shows the performance advantage over the latest HM6.0.  However, it has a main drawback, which is that it does not provide any parallel capacity.

# 4 Bin Parallel Scheme for H0228

We propose a simple 4 bin parallel scheme without introducing additional complexity.  It consists two parts.  The first one is 4 bin friendly scan, as shown in Fig 1. For a TU with diagonal scan, we keep the diagonal scan for the sub-block level, but have the horizontal scan, instead of diagonal scan for coefficients inside the sub-block.  The second one is a 4 bin friendly mask for context index derivation.

For each coefficient in a subblock with horizontal scan, an appropriate mask which excludes the immediate four previous neighbors in the scanning pattern is employed to derive the context index. One example of such mask is shown in Fig. 2. Fig. 2 (a) shows an example of a 5-neighbor mask when vertical scan is used and Fig. 2 (b) shows an example of a 5-neighbor mask when horizontal scan is used.

Fig. 1 Proposed scan pattern for 4 bin parallel scheme

(a) vertical scan            (b) horizontal scan

Fig. 2 An example of 5-neighbor mask with 4 bin parallel support for vertical scan and horizontal scan, where “x” is the current element and “o” is the neighbor.

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 1. Detailed results are included in the attachment (JCTVC-Ixxx.xls).

Table 1 Simulation results

Reference

- JCT-VC, “Non-CE11: Proposed cleanup for Transform Coefficient Coding”, JCTVC-H0228, 8th JCT-VC Meeting, San Jose, CA, USA, 1-10 February, 2012

Motorola Mobility Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).