| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0375 |
| --- | --- |

| Title: | Simplification of Significance Map Coding Process for Small TU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Ying Kung Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2350 viviankung@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution document proposes to replace fixed contexts with adaptive context for small TUs in order to harmonize and unify the context selection for TUs with different sizes. The experimental results show that there is within 0.1% gain or loss in both intra and inter conditions.

# Introduction

In current HM6.0, the significant bits of small transform units (TUs) are using fixed contexts according to its physical locations.  For 4x4 blocks, luma significant map utilizes 9 contexts while the chroma significant map uses 6 contexts, as Fig. 1 (a) shows.  For 8x8 blocks, luma and chroma are using 10 contexts for AC components and 1 additional context for DC component, as shown in Fig 1 (b). There are total 20 (9 for 4x4 and 11 for 8x8) contexts are required to code significant map of luma small block TUs and 17 (6 for 4x4 and 11 for 8x8) contexts are required to code significance map of chroma block TUs.

(a)

(b)

Fig. 1 HM-6.0 significance map context index assignment of small TU.

HM6.0 also utilizes adaptive scan method for intra TU of 4x4 and 8x8. Depending on intra prediction direction selected, an intra TU may use horizontal, vertical, or diagonal scan pattern.

For significance bits of large TUs (TU of size 16x16, 32x32, 16x4, 4x16, 32x8, 8x32), HM6.0 employs a combination of fixed contexts for low and high frequency elements and adaptive context for middle frequency elements. Adaptive context index is derived from the non-zero significant neighbors, as shown in Fig 2.  Usually five neighbors are used.  In order to achieve 2 bin parallel processing, only four neighbors are used for the elements in the upper left corner and the one right above the right bottom corner of a subblock.

Fig. 2 Neighbors of significant bits, where “x” is the current element and “o” is the neighbor.

A luma TU is first separated into 4 different zones, as shown in Fig. 3(a). Zone 0 only contains a DC element, and is coded with a fixed context; Zone 1 consists of the first 4x4 subblock, except the DC element, and each element is coded with a mask based adaptive context model; Zone 2 consists of the middle frequency subblocks, and each element is coded the same way as the one in Zone 1, but with different context offset. Zone 1 and Zone 2 occupies 3 contexts each.  Zone 3 consists of the high frequency subblocks, and each element is coded with the fixed context, which is the first context in Zone2. Zone 2 and Zone 3 are separated based on the threshold equals 3(TU size >> 4).  An element is belongs to Zone 3 if (posX>>2) + (posY>>2) >= 3(TU size >> 4). Otherwise, it belongs to Zone 2 if it is not in the first DC subblock, which mathematically means (posX>>2) + (posY>>2) > 0.

The similar zone separation approach is applied to a large chroma TU except that there is no separation between zone 1 and zone 2 like luma TU. Specifically, A chroma TU is separated into 3 different zones, as shown in Fig. 3(b). Zone 0 only contains a DC element, and is coded with a fixed context; Zone 1 consists of the middle frequency subblocks including the DC subblock, but excluding the DC element, and each element is coded with a mask based adaptive context model; Zone 1 occupies 3 contexts.  Zone 2 consists of the high frequency subblocks, and each element is coded with the fixed context, which is the first context in Zone1. Zone 1 and Zone 2 is separated based on the threshold equals 3(TU size >> 4).  An element is belongs to Zone 2 if (posX>>2) + (posY>>2) >= 3(TU size >> 4). Otherwise, it belongs to Zone 1 if it is not the first DC element, which mathematically means posX + posY > 0.

(a) Luma 					(b) Chroma

Fig. 3 HM-6.0 significance map context index zoning for large TUs.

# Simplification of Significance Map Coding Process for Small TU

We propose to harmonize the luma L0 significance map coding by extending the mask based adaptive context modeling and TU zoning to small TUs. In other words, the fixed contexts with the mapping tables are removed for coding the significance bits of the small TUs.  Instead, an adaptive context modeling with the appropriate zoning is applied to 4x4 and 8x8 TUs.

This proposal proposes to separate both 4x4 luma and chroma TUs into two zones, as shown in Fig. 4(a). Zone 0 only contains a DC element, and is coded with a fixed context; Zone 1 consists of the rest elements, and each element is coded with a mask based adaptive context model. Zone 1 of a small luma TU shares the same contexts with the one of a large luma TU;  Zone 1 of a small chroma TU shares the same contexts with the one of a large chroma TU;  Zone 0 can share the fixed context with the one in a large TU or the one in an 8x8 TU.

This proposal also proposes to separate an 8x8 luma TU is separated into three zones and an 8x8 chroma TU is separate into two zones, as shown in Fig 4(b). Zone 0 only contains a DC element, and is coded with a fixed context; Zone 1 of a luma TU consists of the first 4x4 subblock, except the DC element. Zone 1 of a chroma TU consist the rest of elements in a TU except the DC element. Each element in zone 1 is coded with a mask based adaptive context model; Zone 2 consists of the rest subblocks, and each element is coded the same way as the one in Zone 1, but with different context offset. Zone 0 can share the fixed context with the one in a large TU or the one in a 4x4 TU. Zone 1 and Zone 2 occupies 3 contexts each.  These contexts are shared with those of a large TU.

HM6.0 allows horizontal, vertical as well as diagonal scans adaptively selected for intra code 4x4 and 8x8 TUs.  The subblock size is normally 4x4.  However, there are two special cases. One is 8x2 for an intra coded 8x8 TU with horizontal scans.  The other is 2x8 for an intra-coded 8x8 TU with vertical scans.  An alternative example to separate zone 1 and zone 2 in intra-coded 8x8 luma TUs with horizontal/vertical scans is to set the boundaries along the subblock boundaries, as depicted in Fig. 5.

In the proposed method, a total number of contexts used to code L0 luma significance map can be reduced by 18; from 27 to 9.

In order to achieve 2 bin parallel processing, the same neighbor setting as a large TU is used for a small TU with diagonal scans. When horizontal scan is selected for the small TU,   the neighbor immediately to the right should always not be referenced.  When vertical scan is selected for the small TU, the neighbor immediately to the bottom should always not be referenced. One example of the 4 and 5 neighbor setting with 2 bin parallel processing support is demonstrated in Fig 6 (a) and (b).

(a)

(b)

Fig. 4 Proposed significance map context index zoning for small TUs

Fig. 5 Alternative zoning for 8x8 luma TUs with horizontal/vertical scans

Fig. 6 Neighbors of significant bits for a small TU with horizontal/vertical scan

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 1. Detailed results are included in the attachment (JCTVC-Ixxx.xls).

Table 1 Simulation results

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.13% | 1.01% | 0.89% | 0.11% | 0.67% | 0.87% |
| Class B | 0.17% | 1.19% | 1.03% | 0.13% | 1.05% | 0.93% |
| Class C | 0.00% | 1.22% | 1.25% | -0.04% | 0.97% | 0.80% |
| Class D | -0.05% | 0.99% | 0.85% | -0.08% | 0.77% | 0.72% |
| Class E | 0.33% | 0.42% | 0.43% | 0.30% | 0.20% | 0.39% |
| Overall | 0.11% | 1.00% | 0.92% | 0.08% | 0.77% | 0.77% |
|  | 0.11% | 1.01% | 0.93% | 0.08% | 0.78% | 0.79% |
| Class F | -0.07% | 1.20% | 0.89% | -0.12% | 0.79% | 0.54% |
| Enc Time[%] | 72.16% |  |  | 110.29% |  |  |
| Dec Time[%] | 73.13% |  |  | 128.42% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.12% | 0.98% | 0.98% | 0.01% | 0.56% | 0.63% |
| Class B | 0.03% | 0.78% | 1.05% | 0.03% | 0.80% | 0.89% |
| Class C | -0.13% | 1.06% | 0.71% | -0.16% | 0.85% | 0.61% |
| Class D | -0.12% | 0.85% | 0.85% | -0.05% | 0.76% | 0.75% |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | 0.91% | 0.91% | -0.04% | 0.75% | 0.73% |
|  | -0.02% | 0.90% | 0.89% | -0.04% | 0.76% | 0.72% |
| Class F | -0.18% | 0.72% | 0.84% | -0.24% | 0.55% | 0.69% |
| Enc Time[%] | 80.98% |  |  | 99.77% |  |  |
| Dec Time[%] | 73.34% |  |  | 146.89% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.04% | 0.28% | 0.33% | 0.00% | 0.71% | 0.57% |
| Class C | -0.09% | 0.79% | 0.80% | -0.16% | 1.05% | 0.68% |
| Class D | 0.04% | 0.79% | 0.92% | -0.06% | 0.39% | 0.86% |
| Class E | -0.12% | -0.52% | 0.37% | -0.11% | 0.28% | 0.38% |
| Overall | -0.05% | 0.39% | 0.60% | -0.08% | 0.64% | 0.63% |
|  | -0.05% | 0.43% | 0.61% | -0.08% | 0.68% | 0.65% |
| Class F | -0.41% | 1.20% | 0.18% | -0.35% | -0.50% | 0.76% |
| Enc Time[%] | 81.75% |  |  | 100.66% |  |  |
| Dec Time[%] | 72.87% |  |  | 140.03% |  |  |

Motorola Mobility Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).