| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0376 |
| --- | --- |

| Title: | Sub-block Based Zoning for Significant Map Coding and Level Coding (H0228) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Ying Kung Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2350 viviankung@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

We propose to set the zone separation line along the sub-block boundaries in H0228 so that the zone determination is performed once for a whole subblock, which saves the logics and reduces the complexity.

# Introduction

H0228 [1] proposed a cleanup for transform coefficient coding of the current HEVC design. It achieves the simplification and harmonization by coding the transform coefficients level and significance bit in a single loop and applying the same simple rules for context selection and the Rice parameter derivation for different sizes of transform units (TU).

Fig. 1 shows the context index zoning for significance map and level bin 1 and 2 of luma TUs in H0228.  Assume posX and posY are the x and y coordinates of a coefficient in a TU.

In case of significance map of a luma TU, as shown in Fig 1(a), zoning is determined as the following formula:

Zone0if((posX+posY)<2)Zone1elseif((posX+posY)<5)Zone2else

In case of level bin1 and bin2 of a luma TU, as shown in Fig 1(b), zoning is determined as the following formula:

Zone0if((posX+posY)<3)Zone1elseif((posX+posY)<10)Zone2else

For chroma TUs, the context index zoning is the same except that there is no zone 1 and zone 2 differentiation (i.e., they are combined), as shown in Fig 2. Note that the separation bounds between zone 0 and zone1 and between zone 1 and zone 2 have to be checked for each transform coefficient.

(a) Significance map					 (b) level bin 1 and bin 2

Fig. 1 Context index zoning for a luma TU

(a) Significance map					 (b) level bin 1 and bin 2

Fig. 2 Context index zoning for a chroma TU

# Sub-block Based Zoning for Significant Coefficients in H228

We propose to set the zone separation line along the sub-block boundaries so that the zone determination is performed once for a whole subblock, which saves the logics and reduces the complexity.  Fig 3 shows an example of the zone separation along the sub-block boundaries for zone 1 and zone 2 in a luma TU.

In case of significance map of a luma TU, as shown in Fig 3(a), zoning is determined as the following formula:

Zone0if((posX+posY)<2)Zone1elseif((posX>>2+posY>>2)==0)Zone2else

In case of level bin1 and bin2 of a luma TU, as shown in Fig 3(b), zoning is determined as the following formula:

Zone0if((posX+posY)<3)Zone1elseif((posX>>2+posY>>2)<2)Zone2else

(a) Significance map					 (b) level bin 1 and bin 2

Fig. 3 Proposed zone 1 and zone 2 separation along sub-block boundaries for a luma TU

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 1. Detailed results are included in the attachment (JCTVC-Ixxx.xls).

Table 1 Simulation results

Reference

- JCT-VC, “Non-CE11: Proposed cleanup for Transform Coefficient Coding”, JCTVC-H0228, 8th JCT-VC Meeting, San Jose, CA, USA, 1-10 February, 2012

Motorola Mobility Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).