| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0377 |
| --- | --- |

| Title: | CE1 Test 5.1/5.2: Modified EO offset operation and coding of offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo and David Baylon 6450 Sequence Drive San Diego, CA 92121 USA Elena Alshina, Alexander Alshin, JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +1 (858) 404-2521 kminoo@motorola.com david.baylon@motorola.com +82 10 30261305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com |
| Source: | Motorola Mobility, Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This document is a Core Experiment report for CE1 Test 5.1/5.2 corresponding to [1,2].  In the HM6.0 process, an offset can be added to each pixel in attempt to reduce the distortion of the reconstructed pixel relative to the original pixel.  In this CE1 Test 5.1/5.2, the EO operation in which the offset is applied to the pixel is modified.  In addition, this CE also examines reducing the number of offsets and the coding of the offsets.

# Introduction

In HM6.0, the four possible EO types indicate the direction along which pixel classification will take place.  The possible EO classes per type are defined based on whether neighboring pixels are greater than, less than, or equal to the current pixel.  The offset for a given class is then applied to all pixels in the class by adding the respective offset to the pixel.  In this CE1 Test 5.1/5.2, the operation in which the EO offset is applied to the pixel is modified.  Also, the number of offsets and the coding of the offsets are modified.

# Modified EO offset operation

It is proposed that the EO offset is applied as in HM6.0 but with the following modification.  Consider an interpolated current pixel value that is generated using a weighted combination of its two neighbors.  For example, let the current pixel value be C, and refer to Figure 1.  If the neighbors are L and R, and 2*C > (L + R), let an interpolated value I be computed as I = (L + R) >> 1.  If 2*C < (L + R), let the interpolated value I can be computed as I = (L + R + 1) >> 1.  (Note that for the case of 2*C = (L + R), C is effectively already on the interpolated line so that I = C and this corresponds to the class where no EO is applied.)

In this proposal we apply the offset O for EO class 1 – 4 depending on the relative position of C and I.  When C is not equal to I, the following is applied:

- If the offset O applied to C does not pass I, then the offset is applied (as in HM6.0) and the reconstructed output value V = C + O.

- If the offset O applied to C brings the pixel value towards I and it passes I, then the output pixel value is set to I, that is, V = I.

The second condition above prevents a high frequency edge from passing the interpolated value I and creating another high frequency edge.  That is, it prevents for example a class 1 pixel from becoming a class 3 or 4 pixel after application of the offset.  In the modified EO offset operation, the offset can be applied until the reconstructed output value reaches the bilinear prediction value I.  For any offset that generates the value past the bilinear interpolation, the interpolated value is used for the reconstructed output value.

Figure 1.  Example modified SAO EO operation

# Modification in the number and coding of edge offsets

In this CE, the number of EO offset classes shown in Figure 2 is reduced from 4 to 2 and 1.  For the case of 2 classes, class 1 and 4 can be merged into one class 1’, and class 2 and 3 can be merged into another class 2’.  By combining classes, it has been observed that for the new class 1’ it is beneficial to swap the codewords for offset index 0 and 1 as the frequency of occurrence for the index 0 is less.  For the case where 4 classes are merged into 1 class, all 4 classes use the same offset.  In this case, the offset index 0 corresponds to a non-zero offset value.

Figure2.  SAO EO HM6 classes

# Simulation Results

Both described abode ideas were tested as part of CE1 (Test 5.1 and Test 5.2). Test results vs CE1 anchor are summarized in Tables 1-2. According to CE1 description we testes SAO APS mode for Random Access test case (both main and HE10) [3] with LCU size 64x74. SAO Interleaving mode test was done for Low-Delay-B test case (both main and HE10) and LCU sixes 64x64, 32x32 and 16x16.

# .

Table 1. Summary of CE1 Test5.x results for 1 EO offset: in average 0.1% gain for Luma and 0.14% avrage gain dn Chroma (all in interleaving mode).

|  |  | Low delay B Main LCU 16x16 |  |  |  |  |  | Low delay B HE10 LCU 16x16 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  |  | U |  | V | Y |  | U |  | V |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% |  |  | 0.9% |  | 1.7% | -0.2% |  | 0.1% |  | -0.1% |  |
| Class C | 0.0% |  |  | 0.5% |  | 0.3% | -0.2% |  | -0.2% |  | 0.2% |  |
| Class D | 0.3% |  |  | 1.3% |  | -0.1% | 0.0% |  | 0.2% |  | 0.6% |  |
| Class E | -0.1% |  |  | 0.6% |  | 0.2% | -0.2% |  | -0.2% |  | -0.3% |  |
| Overall | 0.0% |  |  | 0.8% |  | 0.6% | -0.1% |  | 0.0% |  | 0.1% |  |
|  | 0.0% |  |  | 0.8% |  | 0.6% | -0.1% |  | 0.0% |  | 0.1% |  |
| Class F | -0.1% |  |  | -0.5% |  | 0.7% | -0.7% |  | -2.1% |  | -1.5% |  |
| Enc Time[%] | 102% |  |  |  |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 90% |  |  |  |  |  | 85% |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Random Access Main |  |  |  |  |  | Random Access HE10 |  |  |  |  |
|  |  | Y | U |  | V |  |  | Y |  | U |  | V |
| Class A |  | 0.4% | 1.6% |  | 1.7% |  |  | 0.0% |  | -0.1% |  | -0.1% |
| Class B |  | 0.6% | 0.8% |  | 0.7% |  |  | 0.0% |  | -0.4% |  | -0.5% |
| Class C |  | 0.2% | 0.2% |  | 0.3% |  |  | -0.1% |  | -0.5% |  | -0.6% |
| Class D |  | 0.1% | 0.0% |  | 0.2% |  |  | 0.0% |  | -0.2% |  | -0.5% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | 0.3% | 0.7% |  | 0.7% |  |  | 0.0% |  | -0.3% |  | -0.4% |
|  |  | 0.3% | 0.7% |  | 0.7% |  |  | 0.0% |  | -0.3% |  | -0.4% |
| Class F |  | 0.4% | 0.6% |  | 0.6% |  |  | -0.5% |  | -0.3% |  | -0.3% |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |  |  |  |

|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 1.3% | 1.2% | -0.3% | -0.5% | -0.3% |
| Class C | -0.1% | 0.2% | 0.0% | -0.6% | -0.2% | 0.4% |
| Class D | -0.1% | -0.1% | 0.4% | -0.3% | -0.1% | -0.6% |
| Class E | 0.2% | -0.3% | -0.6% | -0.4% | -0.1% | -0.4% |
| Overall | 0.0% | 0.4% | 0.4% | -0.4% | -0.2% | -0.2% |
|  | 0.0% | 0.4% | 0.3% | -0.4% | -0.2% | -0.3% |
| Class F | -0.2% | 1.8% | 1.3% | -0.8% | -0.7% | 0.1% |
| Enc Time[%] | 95% |  |  | 97% |  |  |
| Dec Time[%] | 83% |  |  | 86% |  |  |

|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 1.6% | 1.4% | -0.2% | -0.6% | -0.7% |
| Class C | 0.1% | 0.3% | 0.6% | -0.5% | -1.1% | -1.2% |
| Class D | 0.0% | 0.0% | 0.5% | -0.3% | -1.3% | -0.6% |
| Class E | 0.2% | 1.0% | 1.8% | -0.5% | -1.0% | -1.1% |
| Overall | 0.1% | 0.8% | 1.0% | -0.4% | -1.0% | -0.9% |
|  | 0.1% | 0.7% | 1.0% | -0.4% | -1.0% | -1.0% |
| Class F | 0.2% | 0.6% | 1.8% | -0.6% | 1.8% | 1.2% |
| Enc Time[%] | 90% |  |  | 89% |  |  |
| Dec Time[%] | 80% |  |  | 83% |  |  |

Table 2. Summary of CE1 Test5.x results for 2EO offset: no drop for Chroma and 0.2% average gain in Chrom (all in interleaving mode).

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.2% | -0.1% | -0.1% | -0.2% | 0.0% |
| Class C | -0.1% | 0.1% | -0.1% | -0.1% | -0.2% | 0.0% |
| Class D | -0.1% | 0.3% | -0.1% | -0.1% | 0.4% | 0.5% |
| Class E | -0.1% | -0.2% | -0.3% | -0.1% | 0.0% | 0.0% |
| Overall | -0.2% | 0.1% | -0.1% | -0.1% | 0.0% | 0.1% |
|  | -0.2% | 0.1% | -0.1% | -0.1% | 0.0% | 0.1% |
| Class F | -0.1% | -1.3% | -0.4% | -0.4% | -1.7% | -1.5% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 89% |  |  | 85% |  |  |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class F | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.2% | -0.3% | -0.3% | -0.1% | -0.1% |
| Class C | -0.2% | -0.4% | -0.4% | -0.4% | 0.1% | 0.5% |
| Class D | -0.1% | -0.8% | -0.8% | -0.3% | -0.1% | -0.5% |
| Class E | 0.0% | 0.0% | -0.4% | -0.2% | 0.0% | -0.3% |
| Overall | -0.1% | -0.2% | -0.5% | -0.3% | 0.0% | -0.1% |
|  | -0.1% | -0.2% | -0.5% | -0.3% | -0.1% | -0.1% |
| Class F | -0.2% | 1.4% | 0.6% | -0.6% | -0.6% | -0.9% |
| Enc Time[%] | 96% |  |  | 97% |  |  |
| Dec Time[%] | 82% |  |  | 85% |  |  |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 1.1% | 1.2% | -0.1% | -0.3% | 0.0% |
| Class C | 0.0% | -0.3% | -0.2% | -0.4% | -0.6% | -0.5% |
| Class D | -0.1% | -0.5% | -0.1% | -0.2% | -0.8% | -1.0% |
| Class E | 0.2% | 0.4% | 1.1% | -0.2% | -0.2% | -0.2% |
| Overall | 0.0% | 0.2% | 0.5% | -0.2% | -0.5% | -0.4% |
|  | 0.0% | 0.1% | 0.5% | -0.2% | -0.5% | -0.4% |
| Class F | -0.1% | 0.0% | 0.4% | -0.4% | 1.1% | 2.2% |
| Enc Time[%] | 90% |  |  | 89% |  |  |
| Dec Time[%] | 79% |  |  | 82% |  |  |

# References

[1]  K. Minoo and D. Baylon, “CE8.e.4: Selection and coding of SAO parameters,” JCTVC-H0557, Feb. 2012.

[2]  K. Minoo and D. Baylon, “Non-CE8: Modified EO offset operation and offset quantization,” JCTVC-H0552, Feb. 2012.

[3] F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co. Ltd.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).