| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0378 |
| --- | --- |

| Title: | CE1 Test 6.1: Harmonization of LCU-based SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, Yue Yu, David Baylon, and Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA Elena Alshina Alexander Alshin JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +1 (858) 404-2521 kminoo@motorola.com yyu@motorola.com david.baylon@motorola.com limin.wang@motorola.com +82 10 30261305 elena_a.alshina@samsung.com alexander_b.alshin@samsung.com jeonghoon@samsung.com |
| Source: | Motorola Mobility, Samsung Electronics Co. Ltd |  |  |

_____________________________

# Abstract

This document is a Core Experiment report for CE1 Test 6.1 corresponding to [1,2].  In the HM6.0 process, there is only one SAO BO type, and four offsets are transmitted.  In addition, the left edge of the first band is transmitted.  In this experiment, four offsets are transmitted for luma and two offsets are transmitted for chroma.  However, in order to save overhead, information about the BO bands is not transmitted but instead determined based on statistics available at the decoder.

# Introduction

In HM6.0, for SAO BO the left edge of the first band is transmitted in addition to the four offsets.  This additional side information can become a significant overhead especially as the LCU size decreases.  In this experiment, it is proposed to determine the BO bands based on statistics of a subset of pixels inside the LCU (these pixels are not affected by de-blocking operation).

# LCU-based BO types and classes

Since the range of reconstructed values per LCU is known at both the encoder and decoder, it does not need to be transmitted.  In this case, the range of reconstructed values can be determined after the unit is encoded or decoded, and the BO type is derived from this range.  Note that because natural images tend to exhibit high spatial correlation, a smaller unit may tend to have a smaller dynamic range of intensity values, thereby requiring fewer classes or offsets.  In this experiment, the encoder and decoder reconstruct an LCU and determine the range of values [min, max] from a subset of values in the LCU where pixels from that subset would not be affected by the de-blocking operation after reconstruction of the LCUs to the right and/or below the current LCU.  Each BO divides the calculated [min, max] span into segments for which the offset would be calculated, transmitted and applied.  In the first experiment, four BO offsets for luma and two BO offsets for chroma are studied using [min, max] statistics.  In another experiment, the bands are specified based on the average of a subset of values in the LCU where only two bands for luma and chroma are used.

# Simulation Results

Described abode idea was tested as part of CE1 (Test 6.1). Test results vs CE1 anchor are summarized in Table 1. According to CE1 description we testes SAO APS mode for Random Access test case (both main and HE10) [3] with LCU size 64x74. SAO Interleaving mode test was done for Low-Delay-B test case (both main and HE10) and LCU sixes 64x64, 32x32 and 16x16.

# .

Table 1. Summary of CE1 Test6.1 results: in average 0.1% drop for Luma and 0.8% average gain in Chroma.

.

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | 0.3% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.3% | -0.2% | 0.0% | -0.6% | 0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.5% | 0.4% |
| Class E | 0.0% | 0.3% | 0.2% | -0.1% | 0.0% | -0.2% |
| Overall | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class F | 0.4% | -0.5% | -0.1% | 0.2% | -1.2% | -0.7% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -1.5% | -2.5% | 0.0% | -0.2% | -0.6% |
| Class C | 0.0% | -1.1% | -1.1% | 0.0% | -0.6% | -0.5% |
| Class D | 0.1% | -1.2% | -2.2% | 0.1% | -0.5% | -1.0% |
| Class E | 0.1% | -1.1% | -1.2% | 0.0% | -0.5% | -0.2% |
| Overall | 0.1% | -1.2% | -1.8% | 0.0% | -0.4% | -0.6% |
|  | 0.1% | -1.2% | -1.8% | 0.0% | -0.4% | -0.6% |
| Class F | 1.7% | -0.2% | 1.2% | 0.4% | -1.5% | 0.2% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -2.3% | -3.3% | 0.1% | -1.7% | -1.9% |
| Class C | 0.2% | 0.2% | 0.3% | 0.1% | -0.2% | -0.4% |
| Class D | 0.2% | -0.6% | -1.4% | 0.2% | -0.9% | -1.3% |
| Class E | 0.4% | -1.8% | -3.2% | 0.2% | -0.8% | -1.8% |
| Overall | 0.2% | -1.1% | -1.9% | 0.1% | -1.0% | -1.4% |
|  | 0.2% | -1.1% | -1.8% | 0.1% | -1.0% | -1.4% |
| Class F | 2.8% | 0.4% | 3.6% | 0.9% | 0.2% | 2.3% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

# References

[1]  K. Minoo and D. Baylon, “CE8.e.4: Selection and coding of SAO parameters,” JCTVC-H0557, Feb. 2012.

[2]  K. Minoo, Y. Yu, D. Baylon, and L. Wang “Non-CE8: Harmonization of LCU-based SAO,” JCTVC-H0553, Feb. 2012.

[3] F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co. Ltd.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).