| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0379 |
| --- | --- |

| Title: | CE1 Test 6.2: Coding of SAO Type |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo and David Baylon 6450 Sequence Drive San Diego, CA 92121 USA Elena Alshina Alexander Alshin JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +1 (858) 404-2521 kminoo@motorola.com david.baylon@motorola.com +82 10 30261305 elena_a.alshina@samsung.com alexander_b.alshin@samsung.com jeonghoon@samsung.com |
| Source: | Motorola Mobility, Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

This document is a Core Experiment report for CE1 Test 6.2 corresponding to [1].  In the HM6.0 process, the coding of the SAO type makes use of unary coding for the different SAO types.  As such, the bit cost for signaling between BO and EO and the different EO types can be quite different.  In this experiment, it is proposed to first signal between SAO on/off and then signal between BO and EO type.

# Introduction

In the HM6.0 process, the coding of the SAO type makes use of unary coding between the four EO types and the single BO type.  As such, the bit cost for signaling between the types can be quite different.  It is proposed that by first signaling between EO and BO and then signaling the EO type that more efficient coding can be achieved.

# Modification in the entropy coding of the SAO types

It is proposed to modify the codeword or binarization for the representation of SAO type to make the bit cost for signaling the SAO types more even, as shown in Figure 1.  This can be described as follows:

if(SAO off)
{
  	code(1);
} else {
  	code(0);
  	if(EO Type)
  	{
    		code(0);
    		if (EO Type == x)
      			code(x);
  	} else {
    		code(1);
  	}
}

Figure 1.  Example of proposed SAO on/off and type signaling

In the provided code, the VLC entropy coder (in APS signaling mode) uses the same bits as shown in Figure 1.

For the interleaving mode, CABAC is used to code each of the SAO-on/off flag with a dedicated context (per color plane). Also BO/EO type flag uses its own dedicated context (per color plane). Finally the four Edge-Classification-Directions, are coded using Unary binarization with one context (per color plane).

# Simulation Results

Described abode idea was tested as part of CE1 (Test 6.2). Test results vs CE1 anchor are summarized in Table 1. According to CE1 description we testes SAO APS mode for Random Access test case (both main and HE10) [2] with LCU size 64x74. SAO Interleaving mode test was done for Low-Delay-B test case (both main and HE10) and LCU sixes 64x64, 32x32 and 16x16.

# .

Table 1. Summary of CE1 Test6.2 results: in average 0.1% gain for Luma and 0.5% average gain in Chroma.

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.3% | -0.2% |
| Class C | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | 0.2% |
| Class D | 0.0% | 0.1% | -0.4% | 0.0% | 0.2% | 0.3% |
| Class E | -0.2% | -0.2% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class F | -0.2% | -0.6% | -0.9% | -0.1% | 1.0% | 0.6% |
| Overall | -0.1% | -0.1% | -0.2% | 0.0% | 0.2% | 0.2% |
|  | -0.1% | -0.1% | -0.3% | 0.0% | 0.2% | 0.1% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.6% | 0.0% | -0.2% | -0.2% |
| Class C | 0.0% | -0.4% | -0.9% | -0.1% | -0.3% | -0.4% |
| Class D | 0.0% | -0.7% | -0.3% | 0.0% | -0.1% | -0.5% |
| Class E | 0.0% | -0.2% | -0.5% | -0.1% | 0.0% | 0.0% |
| Class F | -0.2% | 0.0% | -1.4% | -0.1% | -0.4% | -0.7% |
| Overall | -0.1% | -0.3% | -0.8% | -0.1% | -0.2% | -0.4% |
|  | -0.1% | -0.3% | -0.8% | -0.1% | -0.2% | -0.4% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.8% | -0.9% | 0.0% | -0.6% | -0.7% |
| Class C | 0.0% | -0.7% | -0.6% | 0.0% | -0.2% | -0.6% |
| Class D | -0.1% | -0.6% | -0.4% | 0.0% | -0.8% | -0.3% |
| Class E | 0.0% | -0.8% | -1.5% | 0.0% | -0.5% | -1.1% |
| Class F | -0.2% | -0.9% | -1.0% | 0.0% | -0.3% | -0.7% |
| Overall | -0.1% | -0.8% | -0.8% | 0.0% | -0.5% | -0.7% |
|  | -0.1% | -0.8% | -0.9% | 0.0% | -0.5% | -0.7% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

# References

[1]  K. Minoo and D. Baylon, “CE8.e.4: Selection and coding of SAO parameters,” JCTVC-H0557, Feb. 2012.

[2] F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co. Ltd.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).