| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0381 |
| --- | --- |

| Title: | ALF filter coefficients distribution and modifications to coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | +1-214-4870-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution shows the distribution of values of different chroma ALF filter coefficients on common test condition video sequences encoded using AHG6 Option 2 reference software. Most of the distributions are reported to be uni-modal with an approximate bell or laplacian shape. The distributions are reported to have peaks at different locations (other than 0 in many locations). The mean ALF filter over these test conditions is reported to be low pass filter with a sinc type of impulse response. Based on these observations, this contribution proposes modifying chroma ALF filter coefficients coding process. Instead of directly coding the coefficients, this contribution proposes coding the coefficients after subtracting a fixed offset (corresponding to the peak location of distribution). The average BD-Rate is reported to be 0.0% for Y and between 0.0% to -0.3% for U and V.

# Introduction

HM-6.0 uses a 10-tap symmetric 2D FIR filter as shown below.

Figure 2 and 3 show the distribution of chroma ALF coefficients for region based and block based ALF respectively.

Figure 2: Chroma ALF coefficient distribution for region based ALF.

Figure 3: Chroma ALF coefficient distribution for block based ALF.

The average filter (averaged over all test bitstreams) along horizontal and vertical axis through center is shown in Figure 4. It can be seen that the average filter impulse response has a sort of sinc shape.

Figure 4: Average chroma ALF filter along horizontal and vertical axis through center coefficients.

Correspondingly, in frequency domain the average chroma ALF filter behaves as a low-pass filter. The frequency response of this filter is show in Figure 5.

Figure 5: Frequency response of average chroma ALF filter.

# Proposal

Since the distributions of chroma ALF coefficient values have peak at other than zero value and also since the average behavior of chroma ALF is low-pass, this contribution proposes that instead of directly coding the chroma ALF coefficients (as in HM-6.0), the coefficients be coded after subtracting a fixed offset (corresponding to the peak of distribution). The different offset values used are:

Int chromaCenter[10] = {1, -11, 0, 34, 0, -3, 6, -13, 34, 0};

## Results

Table below presents the BD-Rate results when proposed technique is used to code chroma ALF coefficients in region based ALF. Anchor is ALF AhG Option2 software with RA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | 0.0% | -0.1% | -0.1% | -0.1% |
| V | 0.0% | -0.1% | -0.2% | -0.3% |

## CD/DIS text modifications

7.3.3.6 Adaptive loop filter info syntax

| alf_picture_info(cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_minus1 | ue(v) |
| if( alf_no_filters_minus1 = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_minus1 > 1 ) |  |
| for( i = 1; i <16; i++ ) |  |
| alf_filter_pattern_flag[ cIdx ] [ i ] | u(1) |
| for( i = 0; i < AlfNumFilters; i++ ) |  |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ] [ i ][ j ] | ge(v) |
| } else |  |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ] [ 0 ][ j ] | se(v) |
| } |  |

alf_filt_coeff[ cIdx ] [ i ][ j ] specifies the j-th filter coefficient of i-th filter used in the adaptive loop filtering process for the colour component cIdx. When cIdx is equal to 1 or 2, alf_filt_coeff[cIdx][0][j] =  alf_filt_coeff[cIdx][0][j] + alfChromaCoeffOffset[j] where alfChromaCoeffOffset[] = {1, -11, 0, 34, 0, -3, 6, -13, 34, 0}. 	(7-1)

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).