| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0382 |
| --- | --- |

| Title: | Crosscheck of Intra transform skipping (JCTVC-I0408) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | +1-214-4870-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Cross-check results

This document cross-checks Intra transform skipping proposal of JCTVC-I0408. The fast version of the proposal provides BD-Rate savings in the range of 2.7% to 7.3% for Class F. BD-Rate savings for other classes is near to 0. See Tables below for a summary of Class F results.

| Class F results – Transform skipping – Fast version |  |  |  |
| --- | --- | --- | --- |
| AI-Main | -7.3% | -5.5% | -5.4% |
| RA-Main | -5.2% | -3.9% | -3.7% |
| LB-Main | -3.0% | -2.9% | -2.9% |
| LP-Main | -2.9% | -2.6% | -2.8% |
| AI-HE10 | -7.3% | -5.7% | -5.5% |
| RA-HE10 | -5.0% | -4.2% | -4.1% |
| LB-HE10 | -2.8% | -1.3% | -2.9% |
| LP-HE10 | -2.7% | -2.1% | -2.1% |

| Class F results – Transform skipping – Slow version |  |  |  |
| --- | --- | --- | --- |
| AI-Main | -7.8% | -7.6% | -8.3% |
| RA-Main | -5.6% | -6.6% | -6.9% |
| LB-Main | -3.4% | -3.9% | -4.9% |
| LP-Main | -3.2% | -3.6% | -5.2% |
| AI-HE10 | -7.7% | -7.1% | -7.8% |
| RA-HE10 | -5.5% | -6.4% | -6.9% |
| LB-HE10 | -3.0% | -2.5% | -3.0% |
| LP-HE10 | -3.1% | 0.1% | -5.3% |

The focus of this cross-check was on decoder modifications. The proponents claim the following (text in blue), cross-checker comments are in read:

- Prediction: No change.

Confirmed.

- Inverse scaling and de-quantization: No change.

Confirmed.

- Inverse transform: Skipped for 4x4 TUs that use transform skipping. Instead, for those TUs, a scaling process is used to align the energy with that of transformed TUs.

Confirmed. Scaling is a shift operation. Transform is not done but instead all input values are shifted.

- Entropy coding. The only change is to add a flag to control if transform should be skipped or not. No change to coefficient coding.

Confirmed. Flag is sent every 4x4 Intra TU.

- Deblocking, SAO and ALF: No change.

Confirmed.

- High level syntax: A flag is added to SPS to indicate if transform skipping is enabled or not.

Confirmed.

- Since 4x4 TUs tend to use a flat quantization matrix, we also propose to change the default quant matrix to a flat one for 4x4 intra TUs.

Common conditions used in simulation did not test this.