| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0387 |
| --- | --- |

| Title: | Cross verification of Picture Raster Scan Decoding in the presence of multiple tiles (JCTVC-I0158) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1 858 658 3937 mcoban@qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of JCTVC-I0158 on picture raster scan decoding in the presence of multiple tiles. It is confirmed that implementation is consistent with the proposal description and the experiment results are matched with the proponent’s data.

# Introduction

In this contribution [1], CABAC is flushed at the end of each LCU line inside a tile. For decoders that decode in picture scan order, the need to store cabac engine state is eliminated, whereas decoders that decode in CU order has to re-initialize the cabac engine state at the beginning of each LCU row.

# Results

Experimental results based on common test conditions [2] with JCTVC-F335 tile configuration, summarized below, are matched with the data provided by the proponent. For the HM 6.1 based proposed algorithm and the HM 6.1 anchor same tile configuration is used. There is negligible complexity impact from mandating CABAC flush at the end of each LCU row withing a tile.

Table 1. HM6.1 + F335 Tiles vs I0158 + F335 Tiles

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.08% | 0.06% | 0.10% | 0.08% | 0.08% | 0.06% |
| Class B | 0.14% | 0.13% | 0.11% | 0.14% | 0.13% | 0.11% |
| Class C | 0.05% | 0.05% | 0.04% | 0.05% | 0.10% | 0.06% |
| Class D | 0.04% | 0.05% | 0.02% | 0.04% | 0.06% | 0.06% |
| Class E | 0.19% | 0.19% | 0.17% | 0.20% | 0.20% | 0.19% |
| Overall | 0.10% | 0.09% | 0.09% | 0.10% | 0.11% | 0.09% |
|  | 0.10% | 0.09% | 0.09% | 0.10% | 0.11% | 0.10% |
| Class F |  |  |  |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.68% | 0.61% | 0.43% | 0.71% | 0.38% | 0.69% |
| Class B | 1.15% | 1.07% | 0.94% | 1.15% | 0.97% | 1.07% |
| Class C | 0.41% | 0.31% | 0.39% | 0.42% | 0.37% | 0.51% |
| Class D | 0.30% | 0.15% | 0.14% | 0.30% | 0.33% | 0.50% |
| Class E |  |  |  |  |  |  |
| Overall | 0.66% | 0.57% | 0.50% | 0.67% | 0.54% | 0.71% |
|  | 0.66% | 0.59% | 0.48% | 0.67% | 0.53% | 0.67% |
| Class F |  |  |  |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.19% | 0.91% | 1.00% | 1.21% | 1.13% | 1.34% |
| Class C | 0.39% | 0.24% | 0.59% | 0.41% | 0.17% | 0.32% |
| Class D | 0.25% | 0.05% | -0.49% | 0.26% | -0.24% | 0.34% |
| Class E | 4.90% | 4.06% | 4.54% | 4.91% | 4.06% | 4.45% |
| Overall | 1.45% | 1.12% | 1.19% | 1.47% | 1.10% | 1.42% |
|  | 1.45% | 1.16% | 1.20% | 1.46% | 1.07% | 1.53% |
| Class F |  |  |  |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# References

- G. Clare, F. Henry, S. Pateux, “AHG4: Picture Raster Scan Decoding in the presence of multiple tiles,” JCTVC-I0158, Geneva, April. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-H1100, San Jose, Feb. 2012.