| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0388 |
| --- | --- |

| Title: | Parameterized RPS Models |  |  |  |  |  |  |  |  | Improved high throughput coding method based on last position information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |  |  |  |  |  | Input Document to JCT-VC |
| Purpose: | Proposal |  |  |  |  |  |  |  |  | Proposal |
| Author(s) or Contact(s): | Jie Zhao, Sachin Deshpande and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: +1-360-817-8487 Email: asegall@sharplabs.com |  |  | Seung-Hwan Kim, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 |  | Tel: Email: | +1-360-817-8487 kimse@sharplabs.com asegall@sharplabs.com |  |  |
| Source: | SHARP |  |  |  |  |  |  |  |  | SHARP |

_____________________________

# Abstract

This contribution proposes a method to signal the RPS of common prediction structures efficiently.  A parameterized model is employed, and the parameters are transmitted from the encoder to the decoder in the PPS.  At the decoder, an RPS set is then generated from the bit-stream information and addressed in the same way as existing, explicit sets.  The result is improved coding efficiency.  For example, for HM RA and LD common test conditions, 18 bits are needed to signal the parameterized model instead of 142 bits for RA and 135 bits for LD to send a RPS set explicitly. Here, it is proposed to support parameterized models for hierarchical B and IPPP configurations.  Arbitrary configurations are still supported through the use of existing syntax.

# Introduction

# The RPS (Reference Picture Set) concept was proposed in JCTVC-G021 and adopted into the WD in the Geneva meeting.   As background, an RPS describes the reference pictures in the decoded picture buffer (DPB) though the use of deltaPOC information. This RPS data can be transmitted in the slice header, or multiple RPS data (an RPS set) can be signaled in the PPS and identified on a slice-by-slice basis.

# The asserted advantage of RPS signaling is error resiliency.  Namely, the state of the DPB buffer is identified explicitly for each picture (or slice) instead of using a state machine approach that is susceptible to corruption when frames are lost.  A disadvantage of the RPS approach is coding efficiency, as the explicit signaling may cost hundreds to thousands of bits depending on the coding structure. For example, Table 1 below shows the RPS set used by the common test condition RA configuration. The RPS set describes a Hierarchical B coding structure with GOP size eight, which results in eight RPS entries.  An additional three RPS entries are present to address IDR or CRA pictures.  For common test condition RA configuration, coding RPS in PPS takes 142 bits when inter RPS is enabled, and 232 bits when inter RPS is disabled. For low delay configuration, coding RPS in PPS takes 135 bits when inter RPS is enabled, and takes 226 bits when inter RPS is disabled. These numbers will increase as the GOP size increases. For example, for random access configuration with GOP 64 provided in G1036 test case 2.5, it takes 796 bits to code the RPS set.

# Many applications use well known prediction structures; for example, hierarchical coding or IPPP configurations.  The purpose of this proposal is to reduce the overhead of signaling RPS information for these prediction structures without sacrificing flexibility.

# Table 1.0    RPS for RA configuration

| - RPS # | - Type | - POC | - Temporal_id | - Ref_buf_size | - ref_pic | - #ref_pics | - Reference Pictures DeltaPOC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - 1: | - B | - 8 | - 0 | - 4 | - 1 | - 4 | - -8 -10 -12 -16 |
| - 2: | - B | - 4 | - 0 | - 2 | - 1 | - 3 | - -4 -6 4 |
| - 3: | - B | - 2 | - 0 | - 2 | - 1 | - 4 | - -2 -4 2 6 |
| - 4: | - B | - 1 | - 0 | - 2 | - 0 | - 4 | - -1 1 3 7 |
| - 5: | - B | - 3 | - 0 | - 2 | - 0 | - 4 | - -1 -3 1 5 |
| - 6: | - B | - 6 | - 0 | - 2 | - 1 | - 4 | - -2 -4 -6 2 |
| - 7: | - B | - 5 | - 0 | - 2 | - 0 | - 4 | - -1 -5 1 3 |
| - 8: | - B | - 7 | - 0 | - 2 | - 0 | - 4 | - -1 -3 -7 1 |

Table 1.1 Partial RPS for RA configuration

| - 9: | - B | - 8 | - 0 | - 4 | - 1 | - 1 | - -8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - 10: | - B | - 4 | - 0 | - 2 | - 1 | - 2 | - -4 4 |
| - 11: | - B | - 2 | - 0 | - 2 | - 1 | - 3 | - -2 2 6 |

The related RPS syntax is:

| ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |

# Signaling Common Prediction Structures

We propose to signal RPS sets for common prediction structures in a parameterized way.  Here, we propose to support parameter models for hierarchical B and IPPP coding.  However, we believe that the approach is general and other models could be incorporated if desired by the group.

The syntax of our proposal is as follows:

Table 2. Syntax Table for RPS model

| Picture_parameter_set( idx ) { | Descriptor |
| --- | --- |
| … |  |
| rps_model_flag |  |
| if (rps_model_flag) { |  |
| coding_structure | u(1) |
| GOP_size_log2 | ue(v) |
| layer_idc | ue(v) |
| num_ref_minus1 | ue(v) |
| temporal_layer_exist_flag | u(1) |
| if (temporal_layer_exist_flag) |  |
| temporal_id_ordering | ue(v) |
| } |  |
| else { |  |
| ….. // send RPS using existing method |  |
| } |  |
| } |  |
| … |  |
| } |  |

coding structure specifies the RPS model used to generate the RPS set.  The value 0 indicates a hierarchical B model; the value equal to 1 denotes an IPPP model.

GOP_size_log2 specifies log2 of the size of the coding structure.  The value is used in the RPS set generation from model process.

layer_idc indicates the number of layers in the coding structure.  The value is used in the RPS set generation from model process.

num_ref_minus1 specifies the number of reference pictures for each RPS.

temporal_layer_exist_flag equal to 1 specifies the presence of temporal layer information for the RPS set generation from model process.  When not present, temporal_layer_exist_flag shall be inferred to be 0.

temporal_id_ordering specifies ordering of pictures with same temporal_id in current GOP. A value of 1 indicates pictures in the current GOP with same temporal_id are continuous in decoding order. A value of 0 indicates pictures in the current GOP with the same temporal_id are interleaved in decoding order

# Defining Common Prediction Structures

With the above signaling, it is possible to derive common hierarchical B and IPPP prediction structures.  For hierarchical B, we propose to use the method described in [4].  It is asserted that this method provides the minimum decoding delay and smallest DPB requirements for a hierarchical B structure.  A description of the technique is provided below:

The coding order inside a GOP is determined as follows (see Figure 3). First all pictures that are directly or indirectly used for motion-compensated prediction of the first picture (in display order) of a GOP and the first picture itself are coded. For the example in Figure 3 that means that first the key picture (16), and then the first pictures of all hierarchy stages (8, 4, 2, and 1) are coded. Then, all pictures that are required for coding the second picture of a GOP and the second picture itself are encoded as long as they haven’t been encoded yet, and so on. In Figure 3, this coding order is illustrated for two groups of 16 pictures. Note, that the restriction to the described coding order does not influence the analysis of the memory requirements; it can be easily seen that it is not possible to find a coding order, which can be realized with a smaller decoded picture buffer. [Ed: Note that the above references a Figure in the reference.]

For example, for a GOP size of eight and with four layers, we would have the following prediction structure:

Fig. 1 Prediction structure for a GOP with size 8 and 4 layer.

For the IPPP coding method, we propose to use the prediction pattern in the HM.  That is that each frame will use the previous frame in display order as well as additional previous frames corresponding to the lowest layer in the hierarchy.  For example, for frame 10 in the prediction structure in Figure 2, the previous frame (9) and additional previous frames corresponding to the lowest temporal layer (8,4,0) would be used to create the RPS.

Fig. 2 Prediction structure for a GOP with size 4 and 3 layer.

# Results

Using HM, we measured the coding benefit of sending the parameterized model compared to explicit signaling.   We simulated sending the information listed in syntax table2 to decoder to derive RPS.  It takes 18 bits for random access of GOP size 8 and 4 layers. It takes 20 bits for random access configuration of GOP 64 and 7 layers. For comparison, random access configuration which has GOP size 8 and 4 layers takes 142 bits, and for random access with GOP size 64 provided in G1036 test case 2.5, it takes 796 bits to code the RPS set. As can been seen in the results, sending parameterized RPS model saves bits significantly comparing to explicit RPS signaling, and the overhead does not vary much with GOP size. This would be useful for commonly used RPS set and adaptive GOP structure.

# Conclusion

A high level concept of using a small number of models to signal common RPS sets is proposed.  Additionally, two models are suggested that correspond to common coding prediction structures – hierarchical B and IPPP.  The parameterized approach provides bit-rate savings for these conditions.  For applications that desire to use alternative prediction structures, an encoder is still free to signal arbitrary RPS sets in the PPS or slice header.

Given its performance and simplicity, we propose the JCT-VC adopt this change into HM.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-G1103, “High Efficiency Video Coding (HEVC) text specification Working Draft 5,” 7th JCT-VC meeting, Geneva, Nov. 2011.

- JCTVC-G021, “JCT-VC AHG report: Reference picture buffering and list construction (AHG21),” 7th JCT-VC Meeting, Geneva, Nov. 2011.

- JCTVC-G198, “AHG21: Inter reference picture set prediction syntax and semantics,” 7th JCT-VC Meeting, Geneva, Nov. 2011.

- JVT-P014, “Hierarchical B Pictures,” MPEG/ITU-T Joint Video Team proposal document, http://ftp3.itu.ch/av-arch/jvt-site/2005_07_Poznan/JVT-P014.zip.

- JCTVC-G643, “AHG21: On reference picture list construction and reference picture marking,” 7th JCT-VC Meeting, Geneva, Nov. 2011.