| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0389 |
| --- | --- |

| Title: | Better interoperability through conformance testing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg Aaron Wells 2975 San Ysidro Way Santa Clara, CA 95051 | Tel: Email: |  |
| Source: | Ambarella |  |  |

_____________________________

# Abstract

Conformance bitstreams that do not push the legal limit permitted by the Profile & Level can lull the implementer into under-designing the performance capabilities of their decoders.  Once decoders are discovered to crash or drop frames under some legal stream conditions, a new, canonical interoperability point is established within industry that must be tracked by encoder vendors for the lifetime of the specification. For HEVC, it is highly recommended that JCT create test definitions and example streams that maximize the performance with the aim of creating one true interoperability point. If the performance is judged to be too high for implementers to meet, then JCT should lower the profile and level limits to match baseline implementation expectations. Otherwise, there are as many potential profiles & levels as there are decoder designs.

# Introduction

There is no true AVC conformance point in practice. Most decoders deployed today will drop frames when passed a legitimate, though improbable, performance intense bitstream.  Some decoders are known to crash when presented low-complexity streams that include obscure corner cases that the designer did not consider. Other still, crash due to bugs that may have been discovered had more thorough test streams been exercised, that for example, tested more syntax combinations.

The basic design philosophy of the proposed test streams is to pack the maximum number of elements of a given type (motion vectors, coefficients, etc.) sustained over the longest buffer-permitted time window. A hypothetical decoder that approximates what most vendors will likely follow a common structure (hardwired, DSP, CPU) that can be used as the basis of designing the particular values of elements. For example, motion vectors that miss the most DRAM page boundaries while referencing random locations in the frame buffer to minimize cache hits.

Part 5 of the MPEG-2 | H.262 served as the conformance test stream set designed towards these patterns designed by adhoc group [1,2] back in the year 1994. A number of commercial test bitstream products followed.  AVC test streams http://wftp3.itu.int/av-arch/jvt-site/draft_conformance/ similarly followed suit [3]

# Proposed Test bitstreams

In addition to the patterns described in section 6.6 (Specification of the test bitstreams) in [3], the following further test bitstreams patterns are proposed. Several may be combined into one bitstream.

- Maximum number of motion vectors permitted per LCU

- Motion vectors are random, maximize page DRAM misses, that also minimize cache hits.

- Short-term reference picture list change as frequently as legally permitted.

- SliceRate is the maximum legally permitted.

- A bitstream that tests all deblocking edge modes.

# References

[1] MPEG 94/008, “Report on the ad-hoc group for video conformance testing”, C. Fogg.

[2] MPEG 94/202, “Report on the ad-hoc group for video conformance testing”, C. Fogg.

[3] H.261.1 Conformance Testing

# Patent rights declaration(s)

Ambarella may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).