| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0392r1 |
| --- | --- |

| Title: | Segment-based weak filtering decision |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Hassene Tmar | Tel: Email: | faouzi@ebriskvideo.com hassene@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a simplification of the HEVC CD deblocking filter (DF) process in which the line-based decision-making in weak luma filtering is changed to segment-based decision-making, in order to reduce complexity. The proposed simplification yields similar coding efficiency to those of the HEVC CD DLF while reportedly improving the subjective quality for most video sequences.

# Segment-based weak filtering decision

In HM 6.0, all decisions within the deblocking filter process [1] are segment-based (each segment comprises 4-sample lines), while those involving weak filtering are line-based. In the latter case, in every line of the 4-sample edge, we apply the (Δ < 10*tc) condition to decide whether to filter or not filter the first samples (q0i and p0i) shown in Figure 1. In addition to the condition mentioned above, the weak filtering is applied to q1i and p1i according to a segment-based condition (dp < ( β + ( β >> 1 ) ) >> 3 for the p side and dq < ( β + ( β >> 1 ) ) >> 3 for the q side).

Figure 1. Candidate samples for weak filtering.

In this contribution, we propose converting the line-based weak filtering decision process to segment-based by computing an average of the Δ values of the first and fourth sample lines, as shown in Figure 2, using the following equations:

0 = (9 * (q00 – p00) − 3 * (q10 – p10) + 8) >> 4

3 = (9 * (q03 – p03) − 3 * (q13 – p13) + 8) >> 4

Δ = (|0| + |3| +1) >> 1

Then we apply, using the computed Δ, the (Δ < 10*tc) condition to decide whether weak filtering should be applied to the four sample lines of the subject 4-sample edge segment.

Figure 2: Samples used in Δ computation for segment-based weak filtering decision

# Proposed CD text changes (highlighted)

8.7.1.4.1	Decision process for luma block edge [2]

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE, dEp, dEq containing decisions,

This process is invoked only when bS is not equal to 0.

Let s’ represent the luma sample array recPictureL of the current picture.

A variable qPL is derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-1)

A variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) ) as input.

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 55, qPL + 2*( bS − 1) + ( tc_offset_div2 << 1) ) as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-2)

pi,k = s’[ xC + xB – i − 1, yC + yB + k ]		(8-3)

- The variables dpq0, dpq3, dp, dq are derived as follows:

dp0 = | p2,0 − 2*p1,0 + p0,0 |

dp3 =| p2,3 − 2*p1,3 + p0,3 | 		(8-4)

dq0 = | q2,0 − 2*q1,0 + q0,0 | 		(8-5)

dq0 = | q2,3 − 2*q1,3 + q0,3 |		(8-6)

dpq0 = dp0 + dq0		(8-7)

dpq3 = dp3 + dq3		(8-8)

dp = dp0 + dp3		(8-9)

dq = dq0 + dq3		(8-10)

- The variable d is derived as follows:

d = dpq0 + dpq3		(8-11)

- The variables dE, dEp and dEq are set equal to 0.

- if d  is less than β, the following ordered steps apply:

- For the sample location ( xC + xB, yC + yB ), the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,0, qi,0, the variables 2*dpq0, β and tC as inputs and a decision dSam0 as output.

- For the sample location ( xC + xB, yC + yB + 3), the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,3, qi,3, the variables 2*dpq3, β and tC as inputs and a decision dSam3 as output.

- The variable dE is set equal to 1

- If dSam0 is equal to 1 and dSam3 is equal to 1, the the variable dE is set equal to 2

- Otherwise

  - Calculate 0 and 3  using the following equation: i = ( 9 * ( qi,0 –  pi,0 ) − 3 * ( qi,1 – pi,1 ) + 8 ) >> 4

  - Calculate   = (|0| + |3| +1) >>1

  - If   is less than tc*10, the variable dE is set equal to 1 otherwise dE is set equal to 0 .

- If dp is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp is set equal to 1.

- If dq is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq is set equal to 1.

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-12)

pi,k = s’[ xC + xB +k, yC + yB – i − 1 ]		(8-13)

- The variables dpq0, dpq3, dp, dq are derived as follows:

dp0 = | p2,0 − 2*p1,0 + p0,0 | 		(8-14)

dp3 =| p2,3 − 2*p1,3 + p0,3 | 		(8-15)

dq0 = | q2,0 − 2*q1,0 + q0,0 | 		(8-16)

dq0 = | q2,3 − 2*q1,3 + q0,3 |		(8-17)

dpq0 = dp0 + dq0		(8-18)

dpq3 = dp3 + dq3		(8-19)

dp = dp0 + dp3		(8-20)

dq = dq0 + dq3		(8-21)

- The variables d is derived as follows:

d = dpq0 + dpq3		(8-22)

- The variables dE, dEp and dEq are set equal to 0.

- if d is less than β, the following ordered steps apply:

- For the sample location ( xC + xB, yC + yB ), the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,0, qi,0 the variables 2*dpq0, β and tC as inputs and a decision dSam0 as output.

- For the sample location ( xC + Xb + 3, yC + yB ), the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,3, qi,3, the variables 2*dpq3, β and tC as inputs and a decision dSam3 as output.

- The variable dE is set equal to 1

- If dSam0 is equal to 1 and dSam3 is equal to 1,  the the variable dE is set equal to 2

- Otherwise

  - Calculate 0 and 3  using the following equation: i = ( 9 * ( qi,0 –  pi,0 ) − 3 * ( qi,1 – pi,1 ) + 8 ) >> 4

  - Calculate   = (|0| + |3| +1) >>1

  - If   is less than tc*10, the variable dE is set equal to 1 otherwise dE is set equal to 0 .

- If dp is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp is set equal to 1.

- If dq is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq is set equal to 1.

8.7.1.4.5	Filtering process for a luma sample [3]

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	variables dEp1 and dEq1 containing decisions to filter pixels p1 and q1 respectively,

–	a variable tC.

Output of this process is:

–	number of filtered samples nDp and nDq,

–	filtered sample values, pi’ and qj’ with i = 0..nDp − 1, j = 0..nDq − 1

Depending on dE, the following applies:

–	When the variable dE is equal to 2, the following strong filtering applies while nDp and nDq are set equal to 3:

p0’ = Clip3( p0−2*tc, p0+2*tc, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-23)

p1’ = Clip3( p1−2*tc, p1+2*tc, ( p2 + p1 + p0 + q0 + 2 ) >> 2 )	(8-24)

p2’ = Clip3( p2−2*tc, p2+2*tc, ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )	(8-25)

q0’ = Clip3( q0−2*tc, q0+2*tc, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-26)

q1’ = Clip3( q1−2*tc, q1+2*tc, ( p0 + q0 + q1 + q2 + 2 ) >> 2 )	(8-27)

q2’ = Clip3( q2−2*tc, q2+2*tc, ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )	(8-28)

–	Otherwise, if the variable dE is equal to 1 the following weak filtering applies while nDp and nDq are set equal to 0:

 = ( 9 * ( q0 –  p0 ) − 3 * ( q1 – p1 ) + 8 ) >> 4		(8-29)

  - When abs() is less than tc*10, the following ordered steps apply:

    - The filtered sample values p0’ and q0’ are specified as follows:

 = Clip3( -tc, tc,  )		(8-30)

p0’ = Clip1Y( p0 +  )		(8-31)

q0’ = Clip1Y( q0 -  )		(8-32)

# Experimental results

Experiments were conducted following the testing conditions described in JCTVC-H1100 [3]. The proposed deblocking filter techniques were compared to those in the HEVC CD. The coding efficiency (BD-rate) numbers, shown in Table 1, were obtained for the Low Delay P, Low Delay B, Random Access and Intra only Main configurations and the Random Access High Efficiency configurations. The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core i7-2600 @3.40 GHz.

- RAM: 8 GB

- OS: Windows 7.

It is clear, from the tables, that the proposed deblocking filter technique achieves similar coding efficiency levels to those of the HEVC CD for the listed configurations. However, such technique reduces the complexity and simplifies the decision process.

Table 1: Simulation Results

|  | Intra Only Main (proposed) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |

|  | Low Delay B Main (proposed) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class B | 0.0% | -0.3% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.5% | 0.2% |
| Class E | 0.1% | 0.1% | 0.4% |
| Class F | -0.1% | 0.1% | -0.4% |
| Overall | 0.0% | -0.1% | 0.0% |

|  | Low delay P Main ( proposed) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class B | 0.0% | -0.2% | -0.2% |
| Class C | 0.0% | -0.4% | 0.0% |
| Class D | 0.0% | -0.3% | -0.5% |
| Class E | -0.1% | 0.2% | 0.5% |
| Class F | 0.0% | -0.4% | -0.3% |
| Overall | 0.0% | -0.2% | -0.1% |

|  | Random Access Main (proposed) |  |  | Random Access HE10 (proposed) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

# Visual quality evaluation

In order to subjectively evaluate, the proposed deblocking filter techniques, we performed a subjective test using several common test sequences (Cactus, Kimono, FourPeople, RaceHorsesC, and BQMall) See Table 2.

Table 2: Test sequences

| Sequences | Notes |
| --- | --- |
| Cactus | From the common test set |
| Kimono |  |
| Four People |  |
| RaceHorsesC |  |
| BQMall |  |

We presented two decoded video sequences side-by-side (always HM_6.0 vs. HM_6.0 + deblocking filter modification) on a1080p Viera-class Panasonic plasma in a darkened room. Our experiments were executed in a single-blind manner (the viewers did not know which sequence was which). The video sequences were displayed in their native size and positioned side-by-side so that the viewable region in each sequence was identical. The video sequences were played twice and the viewers, who sat between 2 and 5 feet from the plasma screen, were asked which video they preferred.

Tables 3 summarizes our subjective viewing results using the segment-based weak luma filtering decision. It is clear that the deblocking filter technique improves the visual quality for the tested video sequences. In fact, only 27% of the viewers preferred video in which the HEVC CD deblocking filter was applied.

Table 3: Subjective viewing results for the segment based luma filtering decision.

| Sequence | Number of viewers | HM-6.0 | Proposed | No Preference | Remarks |
| --- | --- | --- | --- | --- | --- |
| BQMall | 6 | 2 | 3 | 1 | Four viewers noted that the proposed version is slightly better or the same as the anchor. |
|  |  | 33% | 50% | 17% |  |
| Cactus | 6 | 1 | 4 | 1 | Five viewers noted that the proposed version is slightly better or the same as the anchor. |
|  |  | 17% | 66% | 17% |  |
| Four People | 6 | 1 | 2 | 3 | Five viewers noted that the proposed version is slightly better or the same as the anchor. |
|  |  | 17% | 33% | 50% |  |
| Kimono | 6 | 0 | 4 | 2 | Six viewers noted that the proposed version is slightly better or the same as the anchor. |
|  |  | 0% | 66% | 33% |  |
| RaceHorsesC | 6 | 4 | 1 | 1 | Four viewers noted that the anchor is slightly worse than the proposed version |
|  |  | 66% | 17% | 17% |  |
| Total | 30 | 8 | 14 | 8 | Overall, 73% either preferred the proposed version or expressed no clear preference. |
|  | 100% | 27% | 46% | 27% |  |

# Conclusion and Recommendation

The proposed changes reduce the computational complexity of the HEVC weak-filtering decision process and improve the subjective quality for most video sequences. Therefore, we recommend the adoption of the proposed changes.

# References

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-6.0/

- “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003, San José, February 2012.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100, San José, February 2012.

# Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).