| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0393 |
| --- | --- |

| Title: | How to use the new field sequence SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg 4300 N. 1st St, San Jose, CA 95134, USA Aaron Wells 2975 San Ysidro Way Santa Clara, CA 95051, USA | Email: | chadfogg@gmail.com awells@ambarella.com |
| Source: | Harmonic, Ambarella |  |  |

_____________________________

# Abstract

An alternative but functionally identical conditional tree syntax for the field indication SEI is proposed. Examples of how to use the new field sequence SEI are also provided. In particular, the use of systems time stamps such as PTS can identify how to re-interleave progressive fields back into whole progressive frames when progressive content has been passed between field sequence equipment (PFS). It is recommended that JCT provide liaison statements to relevant organizations such as SMPTE, SCTE, DVB, informing them of the field indication SEI proper use.

# Introduction

For the progressive field sequence case signalled in the field indication SEI (field_sequence_flag==1 && progressive_source_flag==1), identifying which fields belong together for re-interleaving for progressive display shall be indicated by adjacent fields in display order with identical PTS values, but opposite field parity and POC that, ideally, differ by 1. A precedent for identical PTS in field pairs with different POC’s for each field has been set already in MPEG-2 systems, particularly the application of SVC.

It may be advisable for the MPEG-2 systems HEVC amendment to include a note indicating that for bitstreams employing field sequence SEI, decoders should anticipate transitions between a field and frame sequences, and therefore operate “glitch-free” at those transitions.  For example, a sequence of 1080i (transported as per SEI as 1920x540 @ 50 or 60 fps) could transition to a sequence of 1080p (1920x1080 @ 24, 25, or 30 fps) at any time, but the receiver system should output video without a visible glitch during such a transition.  Further, field pictures may need to be interpolated into whole frames to fulfill trick mode access unit definitions.

Slides explaining alternative tree syntax for field indication SEI accompanies this document, along with the H0720 slides from San Jose meeting field sequence BoG that illustrate how to apply the SEI.

# Syntax

It is suggested to modify the syntax definition for field indication SEI in section D.1.19 of the HEVC CD or DIS candidate text (JCTVC-I0030)

| field_indication( payloadSize ) { | Descriptor |
| --- | --- |
| field_sequence_flag | u(1) |
| progressive_source_flag | u(1) |
| duplicate_flag | u(1) |
| if( field_sequence_flag==1 ) |  |
| bottom_field_flag | u(1) |
| else if( progressive_source==0) |  |
| top_field_first_flag | u(1) |
| else |  |
| reserved_zero_1bit | u(1) |
| reserved_zero_4bits /* equal to 0 */ | u(4) |
| } |  |

The following additional clarification points are suggested for section D.2.19 (Field indication SEI message semantics) left to the discretion of the HEVC specification editors to wordsmith into the existing semantic description as only such enlightened people know how:

field_sequence_flag:  indicates how pictures are packed and transported through HEVC encoding and decoding. This indicator is independent of whether the content is progressive or interlace. Sequences that have repeated fields should be encoded as field sequences in order to identify which fields are coded duplicate pictures.

progressive_source_flag: indicates whether the content contained in frame or field sequences is progressive or interlace.  This is independent of how a picture is packed into frames or field sequences as indicated by field_sequence_flag.

bottom_field_flag: is the parity of a field picture within a field sequence. It does not convey whether the content is progressive or interlace.

top_field_flag: can only be signalled in frame sequences.

# Patent rights declaration(s)

Harmonic and Ambarella do not have any current or pending patent rights relating to the technology described in this contribution.