| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, Switzerland, 27 April – 7 May, 2012 | Document: JCTVC-I0394 WG11 Number: m24643 |
| --- | --- |

| Title: | Copy Slice Type |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document describes a new slice_type – C (Copy Slice). It is asserted that the proposed Copy slice type allows improved coding efficiency when a slice from one frame is a copy of a slice from other frame.

# Introduction

HEVC CD supports three slice types  - I Slice, P Slice, B Slice [1].

We propose a new slice_type – C (Copy Slice). The proposed Copy slice type allows very low overhead for signaling that a slice from one frame is a copy of a slice from other frame.

The proposed slice_type C is useful for following use cases:

- A slow motion video sequence: One way of coding a slow motion video sequence is to do frame replication. In this case each frame is repeated two or more times. The proposed Copy Slice type can signal the repeated frames efficiently.

- A simple frame rate upsampler with frame repeat: In LCD televisions a frame rate upsampler may be used to generate higher frame rate sequence from original frame rate sequence. As an example a 120 Hz video may be generated and displayed from a 60 Hz original video. A simple frame rate up-converter may double the frame rate of the video by simply repeating each frame. In some cases this higher frame rate video may be already encoded and transmitted as a 120 Hz video bitstream. In this case the proposed Copy slice type can be used for efficient coding of repeated frames.

- 2-UP Videos with static background: In certain news video segments video of news anchor and a reporter may be shown in 2 windows side by side with static background at the top and bottom of the overall video window. In this case if multiple slices are used per frame a Copy slice type can be used to efficiently code entire slice (at the top/ bottom) as a copy of collocated slice from previous frame.

The proposed Copy slice type

- Can reduce required number of bits to code a slice which is a copy of another slice

- Can result in lower computational complexity and lower decoding time for the copy slice types.

# Proposed Change

We propose following syntax and semantics with changes highlighted compared to HEVC CD [1].

## Syntax and Semantics:

### Syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( slice_type = = C ) { |  |
| copyslice_delta_poc_sign | u(1) |
| copyslice_delta_poc_minus1 | ue(v) |
| } else { |  |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt[ i ] | ue(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( slice_type ! = C ) { |  |
| Rest of the elements in slice header |  |
| } |  |
| } |  |

### Semantics

slice_type specifies the coding type of the slice according to Table 7-1.

Table 7-1 – Name association to slice_type

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - P (P slice) |
| - 1 | - B (B slice) |
| - 2 | - I (I slice) |
| - 3 | - C (C slice) |

When nal_unit_type is equal to 5 (IDR picture), slice_type shall be equal to 2.

When max_dec_pic_buffering[ max_temporal_layers_minus1 ] is equal to 0, slice_type shall be equal to 2.

When slice_type is 3 the slice is of type C, i.e. copy slice. In this case following syntax elements are present.

copyslice_delta_poc_sign specifies the sign of copyslice_delta_poc_minus1. A value of 0 indicates that the sign is positive and a value of 1 indicates that the sign is negative.

copyslice_delta_poc_minus1 plus 1 specifies an absolute difference between two picture order count values. The value of copyslice_delta_poc_minus1 shall be in the range of 0 to 215 − 1, inclusive.

### Generation of Decoded Picture for C Slice Type

### For the slice type is C (copy slice) the decoding process is as described below.

Outputs of this process are decoded samples of the current picture (sometimes referred to by the variable CurrPic).

Depending on the value of chroma_format_idc, the number of sample arrays of the current picture is as follows.

–	If chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

Decoding process for Picture Order Count as described in section 8.3.1 [1] is followed to arrive at PicOrderCntVal for the current picture.

Then the Picture Order Count value for the picture to be used for copying SrcPicOrderCntVal is calculated as:

SrcPicOrderCntVal = PicOrderCntVal + copyslice_delta_poc_sign* (copyslice_delta_poc_minus1+1)

Then

- The value of each element in the sample array SL for the current slice of the picture is set to the value of the corresponding element in the sample array SL for the collocated slice of the decoded picture in the decoded picture buffer with the picture order count value SrcPicOrderCntVal.

- The value of each element in the sample arrays SCb and SCr for the current slice of the picture is set is set to the value of the corresponding element in the sample array SCb and SCr respectively for the collocated slice of the decoded picture in the decoded picture buffer with the picture order count value SrcPicOrderCntVal.

This completes the decoding of the current slice of the current picture

# Results

We created video sequence with successive frames repeated (i.e. frame 0 and frame 1 are identical, frame 2 and frames 3 are identical etc.). We encoded this video sequence using HM 6.1 and also using our proposed approach. In the table below we show the required number of bits to code the repeat frames using HM 6.1 and using proposed Copy Slice Type method. (Here, the HM6.1 bits are the minimum number of bits to code a repeat frame for the entire sequence.)  The coding structure used was low delay P main configuration (encoder_lowdelay_P_main.cfg). Results are shown below for various frame resolutions. The bit count depends only on the resolution when frame is a repeat frame. As can be observed our proposed method requires less number of bits compared to the HM. Additionally the decoding time required for decoding the repeat frame coded using proposed method is lower than that required for decoding the repeat frame coded using the HM.

| Frame Resolution | Configuration | CD - HM 6.1 (Bits) | Proposed Method (Bits) | % Reduction |
| --- | --- | --- | --- | --- |
| 3840x2160 | Low Delay P Main | 568 | 96 | 83 % |
| 3840x2160 | Low Delay P Main (With NumTileColumnsMinus1=15) | 1464 | 96 | 93% |
| 3840x2160 | Low Delay P Main (With NumTileColumnsMinus1=15, NumTileRowsMinus1=15) | 8080 | 96 | 98% |
| 1920x1080 | Low Delay P Main | 280 | 96 | 66% |
| 1920x1080 | Low Delay P Main (With NumTileColumnsMinus1=15) | 1056 | 96 | 90% |
| 1920x1080 | Low Delay P Main (With NumTileColumnsMinus1=15, NumTileRowsMinus1=15) | 4520 | 96 | 97% |

# Conclusion

The document proposed a new Copy slice type. The proposed Copy slice type allows very low overhead and computationally efficient coding when a slice from one frame is a copy of a slice from other frame. We propose to adopt the proposed approach in HEVC draft.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” JCTVC-H1003_dK, San Jose, February, 2012.