| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0397 |
| --- | --- |

| Title: | AHG5: Unified coefficient scan for JCTVC-H0228 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer, Mischa Siekmann, Christian Bartnik, Detlev Marpe, Tung Nguyen, Thomas Wiegand Einsteinufer 37, 10587 Berlin, Germany | Email: | Heiner.Kirchhoffer@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This proposal replaces the horizontal and vertical scan in JCTVC-H0228 with the diagonal scan while coefficient positions to be coded are still determined based on the last position in combination with horizontal or vertical scan. This removes most of the cases where the context derivation for of a coefficient depends on the directly previous coefficient while coding efficiency is only slightly decreased. Furthermore, removal of horizontal and vertical scan reduces the complexity of a particular encoder and decoder implementation.

# Introductions

During the last JCTVC-Meeting [1], ad hoc group 5 was established with its mandates targeting unified coefficient scanning while considering throughput/complexity requirements in relation with CE3 [3]. Based on the tests in the CE, this contribution proposes a unification of horizontal, vertical, and diagonal scan for JCTVC-H0228 [2].

# Unified scan

For blocks that are coded in horizontal or vertical scan, the last position in combination with the scan determines which of the coefficient positions are processed and which are not. Based on this distinction, the positions that need to be coded are processed in diagonal scan as exemplified in Figure 1. In this way, derivation of context models using a local template for significant_coeff_flag, coeff_abs_level_greater1_flag, etc. as in JCTVC-H0228 doesn't involve the directly preceding coefficient in scan order. More precisely, the number of scan positions that use the directly preceding coefficient in scan order is limited to 2 for a full sub-block (where all positions need to be processed) and in the worst case (where only one row or column of coefficients is present in a sub-block) it is not greater than 4. For an 8x8 block this gives a worst-case number of 11 directly preceding coefficients involved in context derivation. For a horizontal or vertical scanned block, the worst-case number is 56. This means, the worst case number is reduced by more than 80%.

Figure 1: Example for using the vertical scan for an 8x8 block to determine positions to be scanned (left) and scanning these positions in diagonal scan (right).

Changing the scan from horizontal/vertical to diagonal also involves that coeff-groups also use the 4x4 sub-blocks instead of 8x2 or 2x8 blocks for signaling significant_coeff_group_flag.

# Experimental results

The presented approach has been implemented on top of JCTVC-H0228 software as tested in CE3. Results are presented in Table 1.

Table 1: Results for the proposed scheme compared to HM-6.0 using the common test conditions.

Table 2: Results for the proposed scheme compared to JCTVC-H0228 as tested in CE3.

Note that the proposed scheme only applies to intra blocks. Therefore, the impact on the rate-distortion performance can mainly be seen for INTRA.

# Working draft test

In the following, required syntax changes are listed based on JCTVC-H0228 working draft text. TBD.

| residual_coding_cabac( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if ( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if ( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( max( log2TrafoWidth, log2TrafoHeight ) > 3) { |  |
| xCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 1 ] |  |
| rightCGFlag = (xCG = = (1<< (log2TrafoWidth – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG + 1 ][ yCG ] |  |
| bottomCGFlag = (yCG = = (1 << (log2TrafoHeight – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG ][ yCG + 1 ] |  |
| if( (i = = numLastSubset) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ) |  |
| significant_coeff_group_flag[ xCG ][ yCG ] = 1 |  |
| else |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if ( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] ) { |  |
| numNZInCG = (i = = numLastSubset) ? 1 : 0 |  |
| if ( (n > 0) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ¦ ¦ (numNZInCG > 0) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| numNZInCG += significant_coeff_flag[ xC ][ yC ] |  |
| else |  |
| significant_coeff_flag[ xC ][ yC ] = 1 |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_greater1_flag[ xC ][ yC ] | ae(v) |
| if( coeff_abs_level_greater1_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_greater2_flag[ xC ][ yC ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_minus3[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| coeff_sign_flag[ xC ][ yC ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_minus3[ xC ][ yC ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ xC ][ yC ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |
| } else { |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_greater1_flag[ xC ][ yC ] | ae(v) |
| if( coeff_abs_level_greater1_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_greater2_flag[ xC ][ yC ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_minus3[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| coeff_sign_flag[ xC ][ yC ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_minus3[ xC ][ yC ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ xC ][ yC ] ) |  |
| } else { |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Conclusion

The presented scheme unifies and simplifies coefficient scanning by removing horizontal and vertical scan. Furthermore, it reduces dependencies between context derivation of a particular coefficient and the directly preceding coefficient by 80%.

# References

- Gary Sullivan, Jens-Rainer Ohm, “Meeting report of the eighth meeting of the Joint Collaborative Team on Video Coding (JCT-VC)”, San José, US, 01-10 Feb. 2012.

- Tung Nguyen, Detlev Marpe, Thomas Wiegand, “Non-CE11: Proposed Cleanup for Transform Coefficient Coding”, JCTVC-H0228, San José, US, 01-10 Feb. 2012.

- Vivienne Sze, Tung Nguyen, Yinji Piao, Joel Sole, Krit Panusopone, “Description of Core Experiment (CE3): Coefficient scanning and coding”, JCTVC-H1103, San José, US, 01-10 Feb. 2012.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).