| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0399 |
| --- | --- |

| Title: | CE1: Cross-check by Samsung for Panasonic's Tests 3.1,3.2,3.3,3.4 on removing boundary artifacts for SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results for CE1 tests on block artefacts removing. Original contribution is JCTVC- I0136. The cross-verifications were completed and the results confirm the test results from the proponents. The proponents’ code has also been analyzed and found to match to the context in the proposal. Visual quality improvements are confirmed.

# Introduction

In JCTVC-I0136 [1], the Panasonic tested proposed in [2] method of boundary artifacts removal. This test was included into CE1 (so called Test 3.x series).

Method was tested in 4 different configurations. There is a summary of both 4 experiments description, including comments about complexity, memory usage and possible modifications:

- Test 3.1 – encoder restriction for BO value (magnitude limit is twice lower than in HM6.0)

- Test 3.2 – band offset weakening near LCU boundary.

  - When applied?

    - Only if neighbor blocks BO with the same offsets then no changes are needed; otherwise band offset is modified near boundary

    - Check of these condition on all 4 LCU boundaries is needed and 4 flags per LCU indicating whether BO weakening is applied are needed

  - How BO is modified?

    - In the middle of LCU BO values are is equal to signaled BO

    - 4 lines near LCU boundary use different BO values;

      - BO magnitude is zero at the boundary

      - BO magnitude linearly increases line-by-line up to BO value in the middle

    - Memory to store BO is increase 5 times (compare to HM6.0)

    - For weakening BO calculation multiplication operation is required on decoder side (HM 6.0 SAO in multiplication-free).

- Test 3.3 –  band offset weakening near categories boundary

  - Correcting off-set is calculated for 32 bands in HM6.0 BO.

  - This contribution introduces smoothing for BO across categories (difference between correcting off-sets in neighbor bands is smaller compare to HM6.0).

  - Smoothing of BO across categories needs multiplication operation on decoder side (HM 6.0 SAO in multiplication-free).

- Test 3.4- both Test 3.2 and Test 3.3 modifications are applied together

Offset weakening near LCU boundary simulates de-blocking filter effect.

# Test results

Objective performance. Proponent provided s/w before CE1 dead-line. S/w was up-dated later because of bug fix, but time was enough to cross-check it. The tests were performed under the CE1 common test conditions [4] and the common test condition [3].

The summary of test results is presented in Table.1. Average Luma BD-rate compare to CE1 anchor is reported. All test points are taken into account during averaging. Additionally average BD-rate without class F is reported in the most right column. More details can be found in accompanying excel sheets (run time measurement is not reliable).

Table 1. Summary of Test 3.1, 3.2, 3.3 and 3.4 tests in CE1.

| Test cases | RA main/HE10 | LD main/HE10 | LD main/HE10 | LD main/HE10 | Average ALL (w/o class F) |
| --- | --- | --- | --- | --- | --- |
| SAO mode | APS | Interleaving | Interleaving | Interleaving |  |
| LCU size | LCU = 64x46 | LCU = 64x64 | LCU = 32x32 | LCU = 16x16 |  |
| Test 3.1 | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% (0.0%) |
| Test 3.2 | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% (0.0%) |
| Test 3.3 | 0.2% | 0.2% | 0.2% | 0.0% | 0.1% (0.1%) |
| Test 3.4 | 0.2% | 0.2% | 0.3% | 0.1% | 0.2% (0.1%) |

Since only BO is modified in this contribution class F has big impact into the performance (BO gain is highest for class F). It would be interesting to compare losses of Test 3.x with total BO gain.

Subjective quality check. We have analyzed visual quality according to the list of test cases provided by proponent:

- BasketBallDrill, LD(main), LCU size 64, QP=37, frame 17;

- SlideEditing, LD(main), LCU size 64, QP=37, frame 31;

- Kimono, LD(main), LCU size 64, QP=37, frame 209.

Results of our study are shown on Figures 1-3. There are some non-critical block artifacts on LCU boundary for CE1 anchor. Test 3.1 modification makes them almost invisible. Artifacts totally disappear if Test 3.3 (or Test 3.2 and Test 3.3) modifications are applied.

Figure 1. BasketBallDrill, LD(main), LCU size 64, QP=37, frame 17: CE1 anchor, Test 3.1, Test 3.2, Test 3.3, Test 3.4.

Figure 2. SlideEditing, LD(main), LCU size 64, QP=37, frame 31: CE1 anchor, Test 3.1, Test 3.2, Test 3.3, Test 3.4.

Figure 3. Kimono, LD(main), LCU size 64, QP=37, frame 209: CE1 anchor, Test 3.1, Test 3.2, Test 3.3, Test 3.4.

# Conclusions

The results (BD-rates) provided by Panasonic for Test 3.x series in CE1 are confirmed here. Visual quality improvement for 3 examples suggested by proponent is confirmed as well.

There are some non-critical block artifacts on LCU boundary for CE1 anchor. Test 3.1 modification makes them almost invisible. Artifacts totally disappear if Test 3.3 (or Test 3.2 and Test 3.3) modifications are applied.

.

# References

- T.Matsunobu, T.Sugio, H.Sasai, T.Nishi, “CE1: Method of removing boundary artifacts for SAO”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I0136, April. 2012.

- T.Matsunobu, T.Sugio, H.Sasai, T.Nishi, “Non-CE1: Removing SAO boundary artifacts by weighted offset values”, JCTVC-I0136, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

- F. Bossen, “Common test conditions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1100, Feb. 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, and M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1101, Feb., 2012.