| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0401 |
| --- | --- |

| Title: | Non-CE1: Cross-check by Samsung for Panasonic's removing boundary artifacts by weighted offsets (JCTVC-I0137) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results for Non-CE1 tests on block artefacts removing (proposed by Panasonic in JCTVC-I0137). The cross-verifications were completed and the results confirm the test results from the proponents.

# Introduction

In JCTVC-I0137 [1], the Panasonic tested recently proposed method of boundary artifacts removal. The discontinuity of band offsets in neighbor LCU might create reconstructed signal discontinuity. For SAO APS mode this problem should not be serious since joint optimization of SAO parameters in different LCUs in one picture. So in verified contribution proponent paid attention on SAO Interleaving mode modification only.

Earlier another method of boundary artifact removal was suggested by proponent. It was included into CE 1 as Test 3.2 and Test 3.3 [2]. New contribution can be considered ad simplification of CE1 Tests 3.x. Since only one next to boundary pixel uses modified band offset. So there is only twice memory size increment for BO storage (in Test 3.2 there is 5 times memory size increment for BO storage [3]). As in Test 3.2 and Test 3.3 weakening of BO and BO smoothing across categories requires multiplication on decoder side (HM6.0 SAO is multiplication-free).

There are two more differences compare to Test 3.2. No additional flag is needed to check whether BO weakening is needed on LCU boundary (in SAO Interleaving mode it can be done based on merge up and merge left flags).

Linear smoothing of BO near boundary was done assuming zero BO on a boundary for Test 3.2, but here access to neighbor LCU is needed since boundary pixels values before and after SAO from left (above) neighbor LCU are needed.

Suggested in [1] method was tested in 3 different configurations.

- BO weakening near LCU boundary

- BO smoothing between categories

- Both together

As well as in Test 3.2 offset weakening near LCU boundary helps visual quality since it simulates de-blocking filter effect.

# Test results

Proponent provided s/w in time enough to cross-check it. The tests were performed under the CE1 common test conditions [5] and the common test condition [4].

The summary of test results is presented in Table.1. Average Luma BD-rate compare to CE1 anchor is reported. All test points are taken into account during averaging. Additionally average BD-rate without class F is reported in the most right column.

For comparison Table 2 reports Test 3.x summary.

More details can be found in accompanying excel sheets (run time measurement is not reliable).

Table 1. Summary of verified tests.

| Test cases | I main/HE10 | RA main/HE10 | LD main/HE10 | LDP main/HE10 | Average ALL (w/o class F) |
| --- | --- | --- | --- | --- | --- |
| SAO mode | Interleaving | APS | Interleaving | Interleaving |  |
| LCU size | LCU = 64x64 | LCU = 64x64 | LCU = 64x64 | LCU = 64x64 |  |
| Boundary smoothing | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% (0.0%) |
| Category Smoothing | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% (0.0%) |
| Both | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% (0.0%) |

Table 2. Summary of Test 3.1, 3.2, 3.3 and 3.4 tests in CE1.

| Test cases | RA main/HE10 | LD main/HE10 |
| --- | --- | --- |
| SAO mode | APS | Interleaving |
| LCU size | LCU = 64x46 | LCU = 64x64 |
| Test 3.1 | 0.1% | 0.0% |
| Test 3.2 | 0.1% | 0.1% |
| Test 3.3 | 0.2% | 0.2% |
| Test 3.4 | 0.2% | 0.2% |

Since only BO is modified in this contribution class F has big impact into the performance (BO gain is highest for class F).

Subjective quality check. We have not analyzed visual quality for this contribution because lack of time. But we have analyzed Test 3.x visual quality [3]. Since this contribution is extension and simplification of test 3.3 visual quality results should be similar.

There are no reasons not to trust examples of visual quality improvement provided by proponent [1].

# Conclusions

The BD-rates provided by Panasonic for JCTVC-I0137 are confirmed here. Compare to Test 3.x in CE1 series performance is ~0.1% better for this contribution:

- 0.1% average loss for boundary smoothing (no loss if class F is exluded)

- No loss for category smoothing

# References

- T.Matsunobu, T.Sugio, H.Sasai, T.Nishi, “AHG6: Non-CE1: Removing boundary artifacts by weighted offset values for SAO”, JCTVC-I0137, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

- T.Matsunobu, T.Sugio, H.Sasai, T.Nishi, “Non-CE1: Removing SAO boundary artifacts by weighted offset values”, JCTVC-I0136, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

- E.Alshina, “CE1: Cross-check by Samsung for Panasonic's Tests 3.1,3.2,3.3,3.4 on removing boundary artifacts for SAO”, JCTVC-I0399, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

- F. Bossen, “Common test conditions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1100, Feb. 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, and M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1101, Feb., 2012.