| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0403 |
| --- | --- |

| Title: | Context derivation by using capped coefficients level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen, Wei-Jung Chien, Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-651-8028 cjianle@qualcomm.com wchien@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, the issues of JCTVC-H0228 method are partly addressed and an alternative solution is provided. The coefficients coding process in one coefficient group is separated into 3 scan passes to separate context coded and by-pass bins. The first pass codes the syntax elements including significant_coeff_flag, coeff_abs_greater1_flag, coeff_abs_greater2_flag; the second pass codes coeff_sign_flag and the third pass codes coeff_abs_level_remaining. The context is derived by using the capped coefficients level to reduce the storage size. The simulation results show 0.6% (AI), 0.4% (RA) and 0.2% (LD) BD-Rate gain on average for common test conditions.

# Introduction

In the current design of HEVC, five syntax elements (significant_coeff_flag  coeff_abs_greater1_flag, coeff_abs_greater2_flag, coeff_abs_level_remaining, coeff_sign_flag) are used to represent the coefficients information in one coefficient group, which consists of 16 coefficients. These five syntax elements are coded with five separated passes to facilitate the parallelization parsing capability of each syntax elements. Contrary to the current HEVC design, a scheme that codes all the syntax elements in one single pass is proposed in JCTVC-H0228 [1].

In JCTVC-H0228, in addition to the number of significant neighbours (num_significant_coeff) of the neighboring template used in the current HEVC design, the absolute sum of the neighbours is calculated (sum_absolute_level) for context derivation. The full absolute value is available when a single pass is employed. In JCTVC-H0228, variables sum_absolute_level and sum_absolute_levelMinus1are calculated with following equation. The neighboring template used in JCTVC-H0228 is same to the template in current HEVC test model.

numsignificantcoeff=δixi                                                (1)

with	δixi=1xi≠00xi=0

sumabsolutelevel=xi                                                          (2)

sumabsolutelevelMinus1=φjxi                                         (3)

with	φjx=xi−1xi>00xi=0

sum_absolute_level is used to derive context indices of significant_coeff_flag. sum_absolute_levelMinus1 is used to derive coeff_abs_greater1_flag and coeff_abs_greater2_flag; and it’s also used for derivation of the Rice parameter to code coeff_abs_level_remaining.

Since more information is used for context modeling section, the method of H0228 shows better coding performance comparing to current HM scheme. However, this scheme has several drawbacks.

First, all syntax elements are interleaved coded; therefore, dependency is introduced between consecutively parsed bins, which is alleged to penalize CABAC parsing throughput. Another drawback is the context coded bins and by-pass bins are interleaved which will hinder parallelization parsing capability of by-pass coded syntax elements. The third drawback is that for context derivation, significance as well as entire level information for the transform coefficients in the neighborhood is needed. This significantly increases the amount of storage needed compared to the current HEVC design. This contribution provides an alternative solution to address the second and third drawbacks.

# Technical description

To maintain the similar throughput of by-pass coded syntax elements, this contribution provide an alternative solution of JCTVC-H0228. Instead of coding the syntax elements in the order of interleaved coefficient-by-coefficient, the coefficients coding in each coefficients group is separated into 3 scan passes. The first pass codes all context-coded syntax elements including significant_coeff_flag, coeff_abs_greater1_flag, coeff_abs_greater2_flag; the second pass codes coeff_sign_flag and the third pass codescoeff_abs_level_remaining .

With three separate passes coding, the syntax elements (shall convert to level information) has already been coded (or decoded) is used for context model selection. With this logic, when deriving the context model of a coefficient coding, the neighboring coefficients which belong to the same coefficient group have the maximum level as 3 (reconstructed with already coded context bins) and the neighboring coefficients which belong to the previous coefficient group could have the entire level information. To reduce the buffer size to store the entire level values for each coefficient, the stored coefficients are truncated to 3 before being used in eq.(2) and eq.(3) for context derivation. With the truncation, two bits are used to store each coefficient for context derivation instead 16 bits used in H0228.

HM6.0 Rice parameter update mechanism is used in this contribution to avoid additional calculation and storage of full range of coefficients value if H0228 rice-parameter update scheme is used.

# Experimental results

The method in this contribution is implemented on HM 6.0 and simulated under the common test conditions [2]. The performance is compared to HM6.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate. Table.1 and Table.2 summarize the experimental results for QP={22, 27, 32, 37} and QP={12, 17, 22, 27}, respectively.

## Table.1 Simulation results of common test configuration

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.7% | -0.7% | -0.6% | -0.9% | -0.7% |
| Class B | -0.4% | -0.6% | -0.7% | -0.5% | -0.6% | -0.7% |
| Class C | -0.6% | -0.3% | -0.3% | -0.6% | -0.2% | -0.3% |
| Class D | -0.7% | -0.2% | -0.4% | -0.7% | -0.2% | -0.3% |
| Class E | -0.7% | -0.7% | -0.8% | -0.7% | -0.7% | -0.6% |
| Overall | -0.6% | -0.4% | -0.5% | -0.6% | -0.5% | -0.5% |
|  | -0.6% | -0.4% | -0.5% | -0.6% | -0.5% | -0.5% |
| Class F | -0.7% | -0.5% | -0.6% | -0.7% | -0.8% | -0.7% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | 0.5% | 0.4% | -0.3% | 0.0% | 0.1% |
| Class B | -0.3% | -0.2% | 0.0% | -0.3% | 0.0% | -0.1% |
| Class C | -0.5% | 0.0% | -0.1% | -0.5% | -0.2% | -0.3% |
| Class D | -0.6% | 0.0% | 0.4% | -0.5% | 0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | -0.1% | 0.1% | -0.4% | 0.0% | -0.1% |
|  | -0.4% | 0.0% | 0.1% | -0.3% | 0.0% | -0.1% |
| Class F | -0.6% | -0.5% | -0.3% | -0.7% | -0.7% | -0.6% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.3% | 0.4% | -0.1% | 0.3% | 0.4% |
| Class C | -0.2% | 1.0% | 0.8% | -0.3% | 0.8% | 0.5% |
| Class D | -0.3% | 1.5% | 0.7% | -0.3% | 1.1% | 1.2% |
| Class E | -0.2% | -0.6% | 0.7% | -0.1% | 0.2% | 0.9% |
| Overall | -0.2% | 0.6% | 0.6% | -0.2% | 0.6% | 0.7% |
|  | -0.2% | 0.6% | 0.6% | -0.2% | 0.6% | 0.7% |
| Class F | -0.8% | -0.5% | -0.4% | -0.4% | -2.0% | -0.4% |

## Table.2 Simulation results for QP={12, 17, 22, 27}

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -0.8% | -0.7% | -0.8% | -0.9% | -0.8% |
| Class B | -0.3% | -0.2% | -0.3% | -0.4% | -0.2% | -0.4% |
| Class C | -0.6% | -0.4% | -0.5% | -0.7% | -0.3% | -0.4% |
| Class D | -0.9% | -0.5% | -0.5% | -0.9% | -0.5% | -0.5% |
| Class E | -0.9% | -0.7% | -0.7% | -0.9% | -0.8% | -0.6% |
| Overall | -0.7% | -0.5% | -0.5% | -0.7% | -0.5% | -0.5% |
|  | -0.7% | -0.5% | -0.5% | -0.7% | -0.5% | -0.5% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.2% | -0.3% | -0.5% | -0.2% | -0.3% |
| Class B | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | -0.3% |
| Class C | -0.7% | -0.2% | -0.4% | -0.6% | -0.1% | -0.3% |
| Class D | -0.8% | -0.2% | -0.1% | -0.7% | 0.0% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.5% | -0.1% | -0.2% | -0.5% | -0.1% | -0.3% |
|  | -0.5% | -0.1% | -0.1% | -0.5% | -0.1% | -0.3% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.1% | 0.1% | -0.1% | 0.2% | 0.2% |
| Class C | -0.5% | 0.2% | -0.2% | -0.5% | 0.3% | 0.0% |
| Class D | -0.6% | 0.2% | 0.2% | -0.6% | 0.3% | 0.2% |
| Class E | -0.1% | 0.2% | 0.2% | -0.2% | -0.3% | 0.2% |
| Overall | -0.3% | 0.2% | 0.1% | -0.4% | 0.2% | 0.2% |
|  | -0.3% | 0.2% | 0.0% | -0.4% | 0.2% | 0.2% |

# Conclusion

This proposal provides an alternative solution of based on the method proposed in JCTVC-H0228. Instead of coding the syntax elements in the order of coefficient-by-coefficient, the coefficients coding in each coefficients group is separated into 3 scan passes. This modification facilitates the parallelization parsing capability of by-pass coded syntax elements. The storage of coefficients is also reduced by using capped neighboring coefficient value for context derivation.

# References

[1] T. Nguyen, D. Marpe, T. Wiegand, “Non-CE11: Proposed Cleanup for Transform Coefficient Coding”, JCTVC-H0228, San Jose, Feb, 2012.

[2] F. Bossen, “Common test conditions”, JCTVC-H1100, San Jose, Feb, 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Change to the CD text