| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0404 (rev. 1) |
| --- | --- |

| Title: | CRA pictures with broken links |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 703 5308 garysull@microsoft.com |
| Source: | Microsoft |  |  |

_____________________________

# Abstract

This contribution proposes to go a step further than in the prior JCTVC-H0496 proposal "On bitstreams starting with CRA pictures" – by additionally allowing CRA pictures that are not at the beginning of the bitstream to have non-decodable "leading pictures" like a CRA picture that begins the bitstream can have. In the current draft HEVC standard, a CRA picture that begins a bitstream is allowed to be followed (in bitstream order) by "leading pictures" that cannot be decoded due to missing prior reference pictures. However, a CRA picture that falls in the middle of a bitstream is not currently allowed to have such non-decodable "leading pictures". This contribution proposes to remove this constraint and to add a "broken link" flag that would indicate the potential presence of such non-decodable leading pictures. As proposed, a CRA picture with a broken link indication would be functionally similar to an IDR picture, but would be allowed to have associated "leading pictures" that are not to be decoded or output.

Moreover, we suggest that 1) it may be desirable to use the NAL unit type to identify CRA pictures with and without "broken links", 2) it is not really necessary for the "leading pictures" rule to be used to determine which pictures are not to be decoded under some circumstances and we suggest the use of an explicit indicator such as the NAL unit type to be used for this purpose as well, 3) the concept of an IDR picture becomes unnecessary if the pictures that are to be discarded under some circumstances are indicated explicitly, and finally 4) that a constraint should be specified to prohibit pictures that use any picture in the bitstream as a reference for inter-picture prediction that precedes more than two CRA pictures in decoding order.

# Introduction / Problem Statement

This contribution builds on the concept of a "clean random access" (CRA) picture and the recently-adopted contribution JCTVC-H0496 "On bitstreams starting with CRA pictures" (by Y.-K. Wang and Y. Chen), which proposed to allow a bitstream to begin with a CRA picture rather than an IDR picture (also previously proposed in JCTVC-G319). Such a CRA picture is allowed to have non-decodable "leading pictures" (i.e., pictures that follow the CRA picture in decoding order but precede it in output/display order that may contain references to reference pictures that are not actually present in the bitstream).

This contribution proposes to go one step further than in the prior JCTVC-H0496 proposal – by additionally allowing CRA pictures that are not at the beginning of the bitstream to act in the same way as those that begin the bitstream – i.e. by creating a type of CRA picture that indicates the potential presence of a "broken link".

Allowing a bitstream to begin with a CRA picture rather than an IDR picture is intended to increase editing flexibility. However, the draft HEVC standard currently still requires a CRA splice point that lies in the middle of the bitstream to be completely "well-behaved" – i.e., it requires all of the "leading pictures" of the CRA picture to be fully decodable. The DPB is required to contain all of the pictures that are referenced by the syntax of any such "leading pictures" (in the DPB reference picture set syntax and referencing for inter prediction). We suggest that this requirement is unnecessary and undesirable.

We suggest to add a "broken link" flag to the picture-level information of CRA pictures (specifically, in the slice header or some other appropriate place for picture-level information – such as the APS). When this flag is equal to 1, the bitstream would be allowed to contain "leading pictures" of the CRA picture that are not decodable due to missing preceding reference pictures (as is currently already the case with leading pictures of a CRA picture that starts a bitstream).

We note that this proposed "broken link" flag differs from that found in the MPEG-2 group of pictures header and in the AVC recovery point SEI message. In those cases, the pictures that followed a random access point in the bitstream were still required to be fully decodable if the bitstream decoding process had begun at the beginning of the entire bitstream, and the flag was merely an advisory indication that a subset of the decoded pictures may contain serious visual artefacts that resulted from a splicing operation. This imposed a serious constraint on the allowed DPB state at which a random access splice point could be established. By removing the requirement that the pictures used as references for the "leading pictures" are present in the bitstream, we can substantially simplify the ability of systems to create splice points in bitstreams.

A CRA picture with broken_link_flag equal to 1 would act essentially the same way as an IDR picture, except as follows:

- It would be allowed to be followed (in bitstream order) by "leading pictures" that might refer to pictures that are not present in the bitstream. Leading pictures of the CRA picture would be discarded by the decoder. The standard would specify for the decoder to skip the decoding process for such pictures and would specify for the decoder to not output them – as is already specified for CRA pictures at the beginning of the bitstream. As currently stated in the HEVC draft for leading pictures of a CRA picture that starts the bitstream, "In actual decoders, any leading pictures of a CRA picture at the beginning of the bitstream may simply be ignored (removed from the bitstream and discarded), as they are not specified for output and have no effect on the decoding process of any other pictures that are specified for output." Leading pictures of a CRA picture that is indicated as a broken link would have those same characteristics as those at the beginning of the bitstream.

- Its picture order count would not be required to be equal to 0. Instead, the picture order count MSBs would be set to 0 and the LSBs would be set to the LSB value sent in the CRA picture (as is already specified for CRA pictures at the beginning of the bitstream). Hypothetically, the picture order count of an IDR picture might also not need to be required to be equal to 0 either.

A CRA picture with broken_link_flag equal to 1 would also contain a no_output_of_prior_pics_flag that acts in the same way as for an IDR picture, and a random_access_pic_id that acts in the same way as the idr_pic_id of IDR pictures. The current idr_pic_id should also be renamed to random_access_pic_id and its constraints should be made to apply to both CRA pictures and IDR pictures rather than just to IDR pictures only. Like an IDR picture, a CRA picture with broken_link_flag equal to 1 could activate a different SPS, change the picture size, etc.

Alternatively, we suggest that rather than using a flag, a different NAL unit type could be used to indicate whether a CRA picture has "broken link" status.

# Explicit identification of non-decodable pictures

We suggest that it is not really necessary for the "leading pictures" rule (i.e. the output order relative to that of a CRA picture) to be used to determine which pictures are not to be decoded when a random access is performed or a "broken link" indication is encountered. As an alternative, we suggest the use of an explicit indicator such as the NAL unit type to be used for this purpose as well.

Moreover, we suggest that the concept of an IDR picture becomes unnecessary if the pictures that are to be discarded under some circumstances are indicated explicitly. The only thing that an IDR picture can really currently accomplish that is different than what can be done with a CRA picture is that an IDR picture can be followed in bitstream order by "leading pictures" in output order that are to be decoded and output when performing random access by a decoder. If the classification of whether a picture is to be decoded or not is determined by an explicit syntax indicator (such as the NAL unit type) rather than determined implicitly by the picture order count, then a CRA picture can have all the functionality of an IDR picture (and we therefore would no longer need the IDR picture type).

We suggest that, in such a usage, a constraint should be specified to prohibit pictures that use any picture in the bitstream as a reference for inter-picture prediction that precedes more than two CRA pictures in decoding order. Without such a constraint, recovery may not be assured when performing random access by a decoder – even after multiple CRA pictures have been encountered.

# Proposed picture type / NAL unit type structure

Altogether, we propose to enable identification of four basic types of pictures (e.g. by NAL unit type value):

- One type that identifies the picture to be a CRA picture without a "broken link",

- One type that identifies the picture to be a CRA picture with a "broken link",

- One type that identifies the picture to be a picture that is always to be decoded when the decoding process began at the location of any preceding CRA picture in bitstream order, and

- One type that identifies the picture to be a picture that is not to be decoded when random access has been performed at the random access point of the preceding CRA picture in bitstream order or when the preceding CRA picture in bitstream order is indicated as a "broken link".

Although it is possible to additionally specify a type of picture that will not be followed (in bitstream order) by a picture of the third type listed above (that precedes any subsequent CRA picture in bitstream order), which would be a picture type equivalent to an IDR picture, we suggest that this is not necessary.

# Conclusion

We suggest that the proposed "broken link" CRA picture functionality would be simple to specify and would substantially improve the ability for systems to use the standard for flexible adaptive video delivery, production editing, commercial insertion, etc.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).