| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0405 |
| --- | --- |

| Title: | Non-CE1: Cross-check of TI and Qualcomm proposal (JCTVC-I10193) for LCU SAO Enable Flag Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Junghye Min Elena Alshina JeongHoon Park | Email: | jh643.min@samsung.com elena_a.alshina@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of LCU SAO Enable Flag Coding in the JCTVC- I0193. The cross-verifications were completed and the results confirm the test results from the proponents. The proponents’ code has also been analyzed and found to match to the context in the proposal.

# Introduction

In JCTVC-I0193 [1], the TI and Qualcomm have proposed “SAO Enable Flag Coding” which was designed to separate SAO enabling flags from SAO types.

The provided s/w was analyzed and following design was observed which match with the descriptions in the JCTVC-I0193. In the interleaving mode, SAO enabling flags for three color components are combined and coded as one index. When SAO enabling flags are not true, merge flags are not coded either. In APS mode, SAO enabling flag is assigned to each color component independently. Unnecessary merge flags removal is also applied in APS mode. Due to the separation of enabling flag which was assigned as the first index for the SAO type previously; all SAO type indexes are reduced by 1.

# Test results

The S/W provided by TI and Qualcomm was compiled on 64-bit Linux and executed on Linux cluster. The tests were performed under the CE1 common test conditions [3] and the common test condition [2].

The summary result is presented in the following table, and more details can be found in accompanying excel sheet.

TABLE 1. BD-rate results of the interleaving mode compared to CE1 anchor

|  | Low delay B Main CU64x64 |  |  | Low delay B HE10 CU64x64 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.7% | -0.8% | -0.2% | -0.3% | 0.0% |
| Class C | -0.2% | -0.6% | -0.4% | -0.3% | -0.3% | -0.3% |
| Class D | -0.4% | -0.5% | -0.9% | -0.4% | -0.6% | -0.4% |
| Class E | -1.6% | -2.2% | -1.5% | -1.5% | -1.1% | -1.4% |
| Overall | -0.5% | -0.9% | -0.9% | -0.5% | -0.5% | -0.5% |
|  | -0.5% | -0.9% | -0.9% | -0.5% | -0.6% | -0.6% |
| Class F | -0.5% | -0.4% | -0.7% | -0.4% | 0.2% | -0.6% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B Main CU32x32 |  |  | Low delay B HE10 CU32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.5% | -0.8% | -0.3% | -0.3% | 0.0% |
| Class C | -0.4% | -0.5% | -0.6% | -0.4% | -0.4% | -0.3% |
| Class D | -0.7% | -0.7% | -0.7% | -0.8% | -0.6% | -0.6% |
| Class E | -2.4% | -2.4% | -2.5% | -2.3% | -1.9% | -2.3% |
| Overall | -0.8% | -0.9% | -1.0% | -0.8% | -0.7% | -0.7% |
|  | -0.8% | -0.9% | -1.0% | -0.8% | -0.7% | -0.7% |
| Class F | -0.9% | -0.5% | -1.3% | -0.8% | -0.7% | -0.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay B Main CU16x16 |  |  | Low delay B HE10 CU16x16 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -0.9% | -0.5% | -0.8% | -0.9% | -0.5% |
| Class C | -0.6% | -0.5% | -0.4% | -0.7% | -0.7% | -0.3% |
| Class D | -0.9% | -0.8% | -0.8% | -0.9% | -0.1% | 0.1% |
| Class E | -4.2% | -3.4% | -3.6% | -4.1% | -3.5% | -3.6% |
| Overall | -1.5% | -1.3% | -1.2% | -1.4% | -1.2% | -0.9% |
|  | -1.5% | -1.3% | -1.2% | -1.4% | -1.2% | -0.9% |
| Class F | -1.7% | -2.0% | -1.6% | -1.8% | -1.0% | -1.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

TABLE 2. BD-rate results of the APS mode compared to CE1 anchor

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

# Conclusions

The results (BD-rates) provided by TI and Qualcomm were confirmed. Proposed method is applied for both interleaving and APS mode. From test results, it can be observer that The SAO Enable Flag Coding provides performance gains. It also removes unnecessary coding of merge flags for SAO if neighbor LCU doesn’t use SAO.

# References

- W-S.Kim, D-K. Kwon, I.S.Chong, M. Karczewicz, “Non-CE1: LCU SAO Enable Flag Coding”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I0193, April. 2012.

- F. Bossen, “Common test conditions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1100, Feb. 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, and M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1101, Feb., 2012.