| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0406 |
| --- | --- |

| Title: | CE3: Report for CE3 Subtest 3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Nguyen H. Kirchhoffer C. Bartnik D. Marpe Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This input document reports simulation results for conducted experiments in CE3 Subtest 3. Seven configurations addressing different aspects were implemented and tested. All experiments were simulated using the common test condition, i.e. QP ranged from 22 to 37 with a step size equal to 5. In addition, results for the disabled rate-distortion optimized quantization (RDOQ) and higher bitrate, i.e. QP ranged from 12 to 17, were generated.

# Experiments and Results

## H0228 in HM6

In this experiment, the impact of H0228 using HM6 is tested. Only context initialization related to transform coefficient level coding were exchanged. Furthermore, tools and constraints newly intergrated into HM6 are considered by the implementation of H0228.

| QP range | RDOQ | Intra Main | RA Main | LB Main | Intra HE 10 | RA HE 10 | LB HE 10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 – 27 | yes | -0.8 | -0.5 | -0.2 | -0.8 | -0.4 | -0.2 |
| 22 – 27 | no | -1.0 | -0.7 | -0.5 | -1.0 | -0.6 | -0.3 |
| 12 – 27 | yes | -1.0 | -0.7 | -0.4 | -1.1 | -0.6 | -0.4 |
| 12 – 27 | no | -1.2 | -0.8 | -0.5 | -1.2 | -0.7 | -0.5 |

TABLE 1: Results for transform coefficient level coding scheme as presented in H0228 in HM6

## Encapsulate Results

As the H0228 scheme uses a generic way to select parameters for the transform coefficient level coding with CABAC, i.e. context modeling and Rice parameter derivation, partial results are requested to understand the impact of each modification. Hence, the next Tables present encapsulate results for different aspects of the parameter selection.

| QP range | RDOQ | Intra Main | RA Main | LB Main | Intra HE 10 | RA HE 10 | LB HE 10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 – 27 | yes | -0.2 | -0.1 | 0.0 | -0.2 | -0.1 | -0.1 |
| 22 – 27 | no | -0.2 | -0.1 | -0.1 | -0.2 | -0.1 | 0.0 |
| 12 – 27 | yes | -0.3 | -0.2 | -0.1 | -0.3 | -0.2 | -0.1 |
| 12 – 27 | no | -0.2 | -0.1 | -0.1 | -0.2 | -0.1 | -0.1 |

TABLE 2: Context modeling for coeff_abs_level_greaterX as in H0228 only

| QP range | RDOQ | Intra Main | RA Main | LB Main | Intra HE 10 | RA HE 10 | LB HE 10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 – 27 | yes | -0.4 | -0.3 | -0.2 | -0.4 | -0.3 | -0.2 |
| 22 – 27 | no | -0.7 | -0.5 | -0.4 | -0.7 | -0.4 | -0.3 |
| 12 – 27 | yes | -0.4 | -0.4 | -0.2 | -0.4 | -0.4 | -0.3 |
| 12 – 27 | no | -0.6 | -0.5 | -0.4 | -0.6 | -0.5 | -0.4 |

TABLE 3: Context modeling for coeff_significant_flag as in H0228 only

| QP range | RDOQ | Intra Main | RA Main | LB Main | Intra HE 10 | RA HE 10 | LB HE 10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 – 27 | yes | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| 22 – 27 | no | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| 12 – 27 | yes | -0.3 | -0.1 | 0.0 | -0.3 | -0.1 | 0.0 |
| 12 – 27 | no | -0.3 | -0.1 | 0.0 | -0.3 | -0.1 | 0.0 |

TABLE 4: Rice parameter selection as in H0228 only

## 3-Scan Passes

In this experiment, 3 scan passes is done instead of 1 and affects the parameter selection as follows. In the first scan pass, bins coded with context models, i.e. bins resulting from the truncated unary binarization, are coded. The second scan pass codes bypass bins from the Golomb-Rice binarization scheme. And finally, sign bins are coded in the last scan pass. As the result, bins coded with context models and bypass bins are grouped together into the bitstream.

| QP range | RDOQ | Intra Main | RA Main | LB Main | Intra HE 10 | RA HE 10 | LB HE 10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 – 27 | yes | -0.7 | -0.5 | -0.2 | -0.7 | -0.4 | -0.2 |
| 22 – 27 | no | -1.0 | -0.6 | -0.5 | -1.0 | -0.6 | -0.4 |
| 12 – 27 | yes | -1.0 | -0.6 | -0.4 | -1.0 | -0.6 | -0.4 |
| 12 – 27 | no | -1.1 | -0.7 | -0.5 | -1.1 | -0.7 | -0.5 |

TABLE 5: Using 3 scan passes to separate bins coded with context models from bypass bins

## Separate Context Models for 4×4 Luma Transform Blocks

In this experiment, a seperate set of context models for 4×4 transform blocks is used.

| QP range | RDOQ | Intra Main | RA Main | LB Main | Intra HE 10 | RA HE 10 | LB HE 10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 – 27 | yes | -0.8 | -0.5 | -0.3 | -0.8 | -0.5 | -0.3 |
| 22 – 27 | no | -1.1 | -0.7 | -0.5 | -1.1 | -0.6 | -0.4 |
| 12 – 27 | yes | -1.1 | -0.7 | -0.5 | -1.1 | -0.7 | -0.5 |
| 12 – 27 | no | -1.2 | -0.8 | -0.6 | -1.2 | -0.8 | -0.5 |

TABLE 6: Separate context models for 4×4 luma transform blocks

## Skip Evaluation from Previous Coded Scan Position

In order to remove the dependency from previous scan position, this experiment modifies the local template.

| QP range | RDOQ | Intra Main | RA Main | LB Main | Intra HE 10 | RA HE 10 | LB HE 10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 – 27 | yes | -0.3 | -0.2 | -0.1 | -0.4 | -0.2 | -0.1 |
| 22 – 27 | no | -0.6 | -0.4 | -0.4 | -0.6 | -0.4 | -0.2 |
| 12 – 27 | yes | -0.5 | -0.4 | -0.3 | -0.6 | -0.4 | -0.3 |
| 12 – 27 | no | -0.6 | -0.5 | -0.4 | -0.6 | -0.5 | -0.4 |

TABLE 7: Avoid the direct dependency from the previous coded scan position

# Conclusion

As can be seen from the results, the proposed scheme of H0228 behaves similar in HM6 compared to the results in HM5. Furthermore, from the encapsulated experiment, most gains comes with the modified context modeling for the coeff_significant_flag, i.e. the first bin index of the bin string. However, with increased bitrate, the impact for the coeff_abs_greaterX flag and the Rice parameter selection gets higher.

In the 3-scan pass experiment, the effect of having limited information on the neighbors is shown. The experiment uses separate 4×4 context models for luma improves the coding efficiency for some cases, which is maybe beneficial for sequences outside of the test set. Finally, skipping the evaluation from previous coded scan positions leads to a drop in coding performance.

Furthermore, the performance seems to be higher with increased bitrate and without RDOQ. And finally, it should be noted that the presented scheme with its modification allows a single concept for parameter selection.