| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0407 |
| --- | --- |

| Title: | Compression Performance of HEVC WD6 based on ITU-R Rec. BT.1210 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kei Kawamura Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories) |  |  |

_____________________________

# Abstract

This contribution provides the experimental results to evaluate the coding performance of HEVC under the sufficient coverage of test materials. Those employed materials are recommended in ITU-R Recommendation BT.1210-4. Experimental results show that HM6.0 can achieve BD-Bitrate of 21%, 36% and 43% for the all intra, the random access and the low delay main-configurations, respectively, compared to JM18.3. It is additionally clarified that the gains are decreased to 15%, 24% and 29% especially for sequences comprised by many fine objects with complex motions.

# Introduction

This contribution presents the experimental results to evaluate the coding performance of HEVC under the sufficient coverage of test materials. The employed materials are conforming to 1080/60p HDTV format as recommended in ITU-R Recommendation BT.1210-4.

As for the input video characteristics to evaluate the coding performance, some of importantelements such as the existence of high resolution textures, spatially complicated motion distribution and various camera operations are not covered by the current test materials in JCT-VC. Therefore, further coding performance evaluation under the sufficient coverage of video characteristics is strongly required.

# Test materials

Representative features of recommended test materials are summarized in Table 1.

This contribution employed sequences corresponding to the general type of materials B specified in ITU-R Report BT.2245 [2]. For each sequence, only the first 600 frames were extracted for experiments. The materials of 24 fps were excluded from general type sequences. Color sampling is changed from 4:2:2 into 4:2:0.

As the newly introduced features of test sequences, three materials consist of high resolution textures corresponding to small flowers and fine leaves. One material includes dolly shot at the fountain where accurate motion estimation is difficult due to the complicated motion behavior. Another three materials contains monotonous motion accompanied by representative camera works such as pan-up, pan-down and zoom-out. Those features introduced above are not contained in the class B materials of common test condition.

# Experimental results

The coding experiments based on HEVC Working Draft 6 were conducted by using the latest available reference software HM6.0 and common test conditions. JM18.3 was also employed as the anchor coding scheme based on AVC High Profile. The JM and HM used the same QP settings. The average YUV PSNR (shown in the right-most column of the tables below) was calculated as

PSNRavg=(6×PSNRY+PSNRU+PSNRV)8.

The summary results are shown in Table 2, 3, and 4 corresponding to three conditions with main profile. For the results of other condition such as high-efficiency 10bit condition and/or additional QP=42, Excel sheets are attached.

TABLE 1

Image characteristics [2]

| Series | Materials A |  | Materials B |  | Materials C |  |
| --- | --- | --- | --- | --- | --- | --- |
| Video standard | Part 2 of Rec. ITU-R BT.709 |  |  |  |  |  |
| Number of pixels | 1 920×1 080 |  |  |  |  |  |
| Bit depth | 10-bit |  |  |  |  |  |
| Signal format | RGB |  | YCBCR |  | YCBCR |  |
| Colour sampling | 4:4:4 |  | 4:2:23 |  | 4:2:2 |  |
| Scanning | Interlace |  | Progressive |  | Interlace |  |
| Frame rate | 29.97 Hz (59.94/I) |  | 59.94 Hz (59.94/P) |  | 29.97 Hz (59.94/I) |  |
| File format | Consecutively numbered TIFF files |  | Consecutively numbered YUV files |  | Consecutively numbered TIFF files |  |
| Duration | Title (1 s) + Test Sequence (15 s) |  | Title (1 s) + Test Sequence (15 s) |  | Title (1 s) + Test Sequence (15 s) |  |
| Number of files | 480 (400 for 50/I) |  | 960 (384 for 24/P) |  | 480 (400 for 50/I) |  |
| Number of sequences | 45 |  | 33 |  | 45 |  |
|  | General: 26 | Specific: 19 | General: 18 | Specific: 15 | General: 26 | Specific: 19 |

TABLE 2

All intra case

|  | All Intra Main |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | YUV |
| s201 | -11.14% | -12.44% | -10.72% | -11.25% |
| s202 | -21.36% | -22.53% | -22.84% | -21.72% |
| s203 | -15.40% | -15.16% | -14.35% | -15.31% |
| s204 | -12.31% | -13.71% | -13.98% | -12.62% |
| s205 | -17.59% | -17.05% | -17.67% | -17.64% |
| s206 | -17.14% | -16.59% | -18.63% | -17.32% |
| s207 | -24.78% | -21.94% | -23.67% | -24.52% |
| s208 | -20.37% | -10.37% | -14.28% | -19.13% |
| s209 | -20.28% | -2.74% | -18.57% | -19.65% |
| s210 | -21.40% | -20.82% | -21.93% | -21.53% |
| s211 | -24.37% | -23.33% | -25.20% | -24.49% |
| s212 | -26.99% | -25.15% | -24.19% | -26.58% |
| s213 | -27.18% | -21.81% | -26.42% | -26.89% |
| s214 | -28.54% | -27.26% | -26.33% | -28.23% |
| s215 | -22.71% | -21.03% | -22.26% | -22.60% |
| s216 | -20.48% | -15.78% | -15.06% | -19.78% |
| s217 | -20.22% | -14.21% | -15.59% | -19.48% |
| s218 | -26.75% | -26.58% | -24.62% | -26.55% |
| Overall | -21.06% | -18.25% | -19.79% | -20.85% |
| Enc Time[%] | 69% |  |  |  |
| Dec Time[%] | 206% |  |  |  |

TABLE 3

Random access case

|  | Random Access Main |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | YUV |
| s201 | -21.21% | -21.85% | -24.03% | -21.46% |
| s202 | -44.27% | -48.92% | -47.81% | -45.02% |
| s203 | -24.35% | -15.82% | -23.36% | -23.80% |
| s204 | -21.84% | -16.58% | -19.72% | -21.31% |
| s205 | -24.35% | -26.72% | -32.16% | -25.36% |
| s206 | -30.49% | -25.10% | -41.00% | -31.26% |
| s207 | -43.27% | -45.56% | -41.47% | -43.27% |
| s208 | -41.40% | -28.00% | -36.52% | -40.16% |
| s209 | -28.02% | -37.57% | -28.61% | -28.65% |
| s210 | -32.95% | -34.84% | -39.16% | -33.94% |
| s211 | -45.79% | -46.09% | -50.05% | -46.41% |
| s212 | -48.63% | -53.71% | -46.08% | -48.75% |
| s213 | -53.15% | -57.72% | -48.63% | -53.06% |
| s214 | -37.36% | -37.89% | -36.09% | -37.36% |
| s215 | -42.13% | -49.91% | -48.87% | -44.01% |
| s216 | -25.59% | -30.46% | -29.66% | -26.46% |
| s217 | -31.31% | -42.19% | -31.56% | -32.44% |
| s218 | -46.99% | -45.04% | -42.35% | -46.41% |
| Overall | -35.73% | -36.89% | -37.06% | -36.06% |
| Enc Time[%] | 97% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

TABLE 4

Low delay case

|  | Low delay B Main |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | YUV |
| s201 | -28.71% | -33.15% | -34.24% | -29.51% |
| s202 | -58.37% | -54.88% | -56.76% | -58.00% |
| s203 | -25.36% | -23.28% | -30.02% | -25.45% |
| s204 | -43.33% | -46.23% | -49.23% | -44.12% |
| s205 | -23.08% | -25.47% | -26.15% | -23.60% |
| s206 | -42.94% | -45.87% | -48.96% | -43.83% |
| s207 | -51.24% | -48.53% | -49.14% | -50.87% |
| s208 | -50.92% | -39.89% | -45.24% | -49.74% |
| s209 | -23.13% | -30.09% | -22.43% | -23.65% |
| s210 | -35.79% | -35.71% | -37.55% | -36.10% |
| s211 | -46.16% | -44.27% | -47.38% | -46.25% |
| s212 | -56.41% | -57.03% | -54.19% | -56.26% |
| s213 | -71.41% | -71.38% | -69.96% | -71.29% |
| s214 | -44.80% | -44.46% | -44.02% | -44.74% |
| s215 | -49.72% | -50.12% | -51.51% | -50.15% |
| s216 | -38.98% | -45.41% | -40.13% | -39.76% |
| s217 | -39.79% | -46.74% | -44.75% | -40.90% |
| s218 | -52.82% | -51.06% | -50.28% | -52.44% |
| Overall | -43.50% | -44.09% | -44.55% | -43.70% |
|  | -43.51% | -44.25% | -44.93% | -43.73% |
| Enc Time[%] | 144% |  |  |  |
| Dec Time[%] | 122% |  |  |  |

# Consideration

For an additional consideration, the comparison between the encoding difficulty and BD-bitrate is shown in Figure 1. The difficulty is defined as the interpolated bit-rate of JM18.3 corresponding to the picture quality of 35dB.

From experimental results, it was confirmed that HM6.0 can save about 15%/24%/29% for AI/RA/LD conditions at five materials with higher compression difficulty compared to the common test condition. Those five materials can be regarded as sequences with video characteristics that were not included in common test materials. We further analyze the result for those difficult materials. The blue circle corresponds to the result for sequences (S201, S203, and S205) with fine objects accompanied by complicated motions, and the gains of AI/RA/LD were 15%/23%/26%. The purple circle indicates the result for the sequence S209 with flowing water, and the gains were 20%/28%/23%. Furthermore, we clarify the impact of camera operation on the efficiency of HEVC. From the result for sequences (S204, S206 and S215) with camera operation, it is confirmed that the roughly similar performance improvement can be achieved independent of the camera operation type such as panning and zooming.

# Conclusion

This contribution provided the experimental results to evaluate the coding performance of HEVC under the sufficient coverage of test materials. The employed materials are conforming to 1080/60p HDTV format as recommended in ITU-R Recommendation BT.1210-4. The features of these materials can compensate the restriction under the materials in the common test condition.

For all materials, experimental results show that HM6.0 can save about 21%, 35% and 43% in bitrate for the all intra, the random access, and the low delay main-configurations, respectively, compared to JM18.3. The overall BD-Bitrate was decreased compared to that for common test materials, due to the inefficiency especially for sequences that require comparatively large bit amount to maintain the sufficient picture quality.

Figure 1

Bitrate of JM v.s. BD-bitrate on Random Access condition

# References

- ITU-R Recommendation BT.1210-4, "Test materials to be used in assessment of picture quality," 01/2012. http://www.itu.int/rec/R-REC-BT.1210

- ITU-R Report BT.2245, "HDTV test materials for assessment of picture quality," 10/2011. http://www.itu.int/pub/R-REP-BT.2245

# Patent rights declaration(s)

KDDI Corporation does not have any current or pending patent rights relating to the technology described in this contribution.

Down-sampled from Materials A.

Including four sequences at 50/I, 4:4:4

Including four sequences at 24/P, 4:4:4

Including four sequences at 50/I, 4:2:2