| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0408 |
| --- | --- |

| Title: | Intra transform skipping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cuiling Lan Jizheng Xu Gary J. Sullivan Feng Wu | Email: | jzxu@microsoft.com garysull@microsoft.com |
| Source: | Xidian Univerisity Microsoft Corp. |  |  |

_____________________________

# Abstract

In this document, a transform skipping modification for 4x4 intra TUs is presented. Except for adding one flag to indicate if a 4x4 intra TU uses transform skipping or not, there is no change to the prediction, de-quantization and scaling, in-loop filters and entropy coding modules. The modification has little influence on the coding efficiency for class A, B, C, D and E. But it brings the average coding gain for class F by 7.3%, 5.2% and 3.0% for AI_Main, RA_Main and LD_Main. The coding gains for HE10 configurations are similar. Visual comparison shows that the transform skipping modification can lead to less ringing artefacts, shaper edges and more details for decoded videos.

# Introduction

In [1], we have presented a transform skipping design for intra TUs to improve the coding efficiency. It has been shown that transform skipping can significantly improve the coding efficiency for class F, while the improvement can also be observed for sequences of other classes. There were some concerns about the extra complexity of the transform skipping scheme on the decoder. Thus, in this document, we present a simplified and cleaned design of intra transform skipping. The modification has been tested on top of HM6.0 software and clear improvement on the coding efficiency for class F sequences is obtained.

# Intra transform skipping design

For a 4x4 intra TU (both luma and chroma), we enable it to skip transform. The detail modifications for each module by TS mode are list below.

- Prediction: No change.

- Transform: Skipped. Instead, for transform skipping TUs, a simple scaling process is used. Since a 4x4 inverse transform in the current design scale down the coefficients by 32, to let transform skipping TUs have similar magnitudes as other TUs, a scaling-down process by 32 is performed on transform skipping TUs.

- De-quantization and scaling. No change.

- Entropy coding: A flag for each 4x4 intra TU is sent to indicate if transform is bypassed or not. Two contexts are added to code the flag for Y, U and V TUs.

- Deblocking, SAO and ALF: No change.

- A flag in the SPS to indicate whether transform skipping is enabled or not.

- We do not change quantization process for TUs with transform skipping. That is also the case when quantization matrices are used. Because it may not be reasonable to have different quantization parameters according to spatial locations for those TUs with transform skipping, we also suggest changing the default quantization matrix to a flat matrix for 4x4 intra TUs, when transform skipping is enabled. The other reason is that a small transform tends to use a flat quantization matrix. An alternative to this is to leave how to better use quantization matrix and transform skipping simultaneously to the encoder.

Please note that transform skipping can also be implemented by adjusting the number of right shift in the scaling and quantization process, as what we proposed in [1]. They have a similar performance but not identical. We prefer to keep scaling and quantization process unchanged to keep the modification as simple as possible.

# Objective quality comparison

Table 1 shows the coding performance improvement of the proposed transform skipping over HM6. Although the proposed scheme increase the encoding time by about 30% for all intra cases, it does not influence the encoding time for inter coding cases, while the bit saving is still significant. The encoding time increase is because we double the full RDO procedures for each 4x4 TU to check if the TU should skip transform or not. For the decoder, the proposed scheme makes little impact on the running time, as expected.

We can disable RDOQ and apply simple check for chroma TUs’s transform skipping decision to reduce the encoding time. The results are shown in table 2. By using the fast encoding, only 15% encoding time for all intra coding is observed and the loss compared with the above scheme is about 0.4%~0.5%.

Table 1. Comparison between the proposed intra transform skipping and the HM6.0 anchor. Details can be found in JCTVC-I0408_TS.xlsm.

Table 2. Comparison between the proposed intra transform skipping with a fast encoding and the HM6.0 anchor. Details can be found in JCTVC-I0408_TS_fast.xlsm.

We also tested transform skipping for TUs with larger sizes. The summary results are shown in Table 3. From the table, we can see that transform skipping for 4x4 TUs can obtain most gain of applying it to all TUs. While only applying transform skipping for 4x4 TUs can make the modification at the decoder simple and also reduce the encoder complexity on the decision for transform skipping.

Table 3. Coding performance when transform skipping (TS) is applied to TUs with different sizes. Details can be found in JCTVC-I0408_TS.xlsm, JCTVC-I0408_TS_4_8.xlsm, JCTVC-I0408_TS_4_8_16.xlsm and JCTVC-I0408_TS_4_8_16_32.xlsm.

# Subjective quality comparison

Please refer to the attached slide deck for some visual comparisons. Basically with the proposed transform skipping method, sharper edges, less ringing artefact and more details can be observed in the reconstructed videos compared to those reconstructed by HM6.

# Encoder description

The following modifications are based on JCTVC-G1102_d1, to describe how an encoder can be if intra transform skipping is used. The modifications are marked in Italic.

## 5.5 Transform and Quantization

If a TU of 4x4 uses transform skipping, the residual samples rij (i, j=0..15) is scaled to dij (i, j=0..15) as specified in the following.

dij =  rij << (13 - BitDepth), with i,j = 0,..,15

– Otherwise,

Transforms of sizes 4x4 to 32x32 are supported. The transform coefficients dij (i, j=0..nS-1) are derived from the transform matrix cij (i, j=0..nS-1) of subclause Error: Reference source not foundand the residual samples rij (i, j=0..nS-1) as specified in the following ordered steps.

- The intermediate sample values eij (i, j=0..nS-1) are derived as

eij 	=	ci0*rj0 + ci1*rj1 + … + cinS-1*rjnS-1 with i,j = 0,..,nS-1

### 6.9.1 Intra prediction mode and parameters

The unified intra coding tool provides up to 34 angular and planar prediction modes for luma component of different PUs. With the PU size of 4x4, 8x8, 16x16, 32x32 and 64x64, there are 17, 34, 34, 34 and 34 angular prediction modes available respectively for luma. The best intra prediction mode for luma component of each PU is derived as follows. Firstly, a rough mode decision process is performed. Prediction cost Jpred,SATD specified in the section Error: Reference source not found is computed for all possible prediction modes and pre-determined number of intermediate candidates are found per each PU size (8 for 4x4-8x8 PU, 3 for other PU sizes) resulting in least prediction costs. In this rough decision process, number of coded bits for intra prediction mode is set to Bpred. Then, RD optimization using the coding cost Jmode specified in the section Error: Reference source not found, is applied to the candidate modes selected by the rough mode decision and MostProbableMode. During this RD decision, prediction parameters and coefficients for luma component of the PU are accumulated into Bmode. Concerning chroma mode decision, all possible intra chroma prediction modes are evaluated through RD decision process, where coded bits for intra chroma prediction mode and chroma coefficient are used as Bmode.

When transform skipping is enabled, for each candidate mode selected by the rough mode decision and MostProbableMode, RD optimization using the coding cost Jmode specified in the section Error: Reference source not found is used to determine if the current intra 4x4 TU should be transformed or not. The procedure is also applied to all chroma 4x4 TUs as long as they are evaluated by the coding cost Jmode.

When the fast algorithm for transform skipping is enabled, whether the current intra 4x4 chroma TU is transformed or not is directly derived from the corresponding intra luma TUs. Only when all its corresponding luma TUs use transform skipping, the current 4x4 chroma TU skips the transform. In addition, RDOQ is not utilized for 4x4 TUs that skip transform.

# Proposed text modifications

The following proposed modifications are based on JCTVC-H1003_dK.

7.4.2.4 Scaling list semantics

scaling_list_delta_coef specifies the difference of the matrix coefficient from the previous matrix coefficient, when pred_mode is equal to 1.

When useDefaultScalingMatrixFlag is derived to be equal to 1, the scaling list is inferred from the default scaling list as specified as follows.

- If transform_skip_enabled_flag is equal to 0, it is specified from Error: Reference source not found to Error: Reference source not found.

- Otherwise, it is specified from Error: Reference source not found and Error: Reference source not found.

Table 7-6 – Specification of default values of ScalingList[ 0 ][ MatrixID ][ i ] with i=0..15

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[0][0..2][i] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 |
| - ScalingList[0][3..5][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 21 | - 21 | - 21 | - 24 | - 24 | - 24 | - 36 | - 36 | - 57 |

### 8.6.4 Transformation process for scaled transform coefficients

……

The constructed residual samples are derived as specified in the following ordered steps.

- If ts_flag[x0][y0][cIdx] is equal to 1 and both nW and nH are equal to 4, residual sample value rij with i=0..(nW)−1, j=0..(nH)−1 is derived as follows,

rij = ( dij + (1 << ( shift − 1) ) ) >> shift

The variable shift is derived as follows:

–	If cIdx is equal to 0,

shift = 13 - BitDepthY

–	Otherwise,

shift = 13 – BitDepthC

The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture.

–  Otherwise,

- Each (horizontal) row of scaled transform coefficients dij (i=0..nW−1, j=0..nH−1) is transformed to eij (i=0..nW−1, j=0..nH−1) by invoking the one-dimensional transformation process as specified in subclause Error: Reference source not found according to the width of the transform unit nW, with the (nW)x(nH) array d and the transform type variable horizTrType as the inputs and the output is the (nW)x(nH) array e.

- The intermediate sample values gij (i=0..nW−1, j=0..nH−1) are derived by

gij = Clip3( −32768, 32767, ( eij + 64 ) >> 7 )		(8-1)

- Each (vertical) column of the resulting matrix gij (i=0..nW−1, j=0..nH−1) is transformed to fij (i=0..nW−1, j=0..nH−1) by invoking the one-dimensional transformation process as specified in subclause Error: Reference source not found according to the height of the transform unit nH, with the (nW)x(nH) array e and the transform type variable vertTrType as the inputs and the output is the (nW)x(nH) array f.

- The variable shift is derived as follows:

–	If cIdx is equal to 0,

shift = 20 – BitDepthY		(8-2)

–	Otherwise,

shift = 20 – BitDepthC		(8-3)

- The residual sample value rij with i=0..(nW)−1, j=0..(nH)−1 is derived as follows.

rij = ( fij + (1 << ( shift − 1) ) ) >> shift		(8-4)

#### 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| deblocking_filter_in_aps_enabled_flag | u(1) |
| …… |  |
| } |  |

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| if(transform_skip_enabled_flag && (PredMode == MODE_INTRA) && ( log2TrafoWidth == 2) &&(log2TrafoHeight == 2)) { |  |
| ts_flag [ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| …… |  |
| } |  |

ts_flag[ x0 ][ y0 ][ cIdx ] specifies whether the associated transform block takes TS mode: If it is equal to 1, the transform coding unit is coded using TS mode. When the flag is not present, it shall be infered to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr.

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialization process

|  | Syntax element | ctxIdxTable | initialisationType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| …… |  |  |  |  |  |
| residual_coding() | last_significant_coeff_x_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..54 |
|  | last_significant_coeff_y_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..54 |
|  | significant_coeff_group_flag | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Error: Reference source not found | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found | 0..23 | 24..47 | 48..71 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..5 | 6..11 | 12..17 |
|  | ts_flag[0] | Error: Reference source not found | 0 | 1 | 2 |
|  | ts_flag[1], ts_flag[2] | Error: Reference source not found | 0 | 1 | 2 |

Table 9-2 – Values of variable initValue for ts_flag[0], ts_flag[1] and ts_flag[2] ctxIdx

| Initialization variable | ts_flag[0] ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 139 | 139 | 139 |

# An alternative

Instead of changing 8.6.4, transform skipping can also be implemented by changing the scaling process when transform skipping is used for a TU. In such a case, 8.6.4 is skipped. And the change to the scaling process is as following.

### 8.6.2 Scaling and transformation process

The (nW)x(nH) array of residual samples r are derived as specified as follows:

- If qpprime_y_zero_transquant_bypass_flag is equal to 1 and QP’Y is equal to 0, (nW)x(nH) array r is set equal to (nW)x(nH) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause  is invoked with the width of the transform unit nW, the height of the transform unit nH, the (nW)x(nH) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ], the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a scaled transform coefficient (nW)x(nH) array d.

- If ts_flag[x0][y0][cIdx] is equal to 0, the transformation process for scaled transform coefficients as specified in subclause  is invoked with the width of the transform unit nW, the height of the transform unit nH, the scaled transform coefficient (nW)x(nH) array d and the chroma component variable cIdx as the inputs and the output is a residual samples (nW)x(nH) array r.

### 8.6.3 Scaling process for transform coefficients

……

The variable shift is derived as follows:

–	If cIdx is equal to 0,

  - If ts_flag[x0][y0][cIdx] is equal to 1,

shift = 6

  - Otherwise

shift = BitDepthY + log2TrSize − 9		(8-5)

levelLimit = 1 << Min( 15, 12 + BitDepthY + log2TrSize – QP / 6 ) 	(8-6)

–	Otherwise,

  - If ts_flag[x0][y0][cIdx] is equal to 1,

shift = 6

  - Otherwise

shift = BitDepthC + log2TrSize − 9		(8-7)

levelLimit = 1 << Min( 15, 12 + BitDepthC + log2TrSize – QP / 6 ) 	(8-8)

……

The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture.

# References

[1] C. Lan, J. Xu and F. Wu, “CE5.f: Residual Scalar Quantization for HEVC,” JCTVC-H0361. Feb. 2012. San Jose, USA.

# Patent rights declaration(s)

Xidian University and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Derivation of the equivalent condition about the two modification methods

Here we denote that the modification in section 6 as method 1 and the modification in section 7 as method 2. It can be proved that both methods are identical when QP>=6.

For simplicity, we only analysis the case when scaling_list_present_flag is equal to 0. The analysis procedure with scaling_list_present_flag being 1 will be similar.

For the modification method 2 where the right shift bit number in the inverse quantization is modified, the scaled coefficients dij is derived by

dij = (cij * levelScale[ qP%6 ] << per + ( 1 << ( shift0 − 1 ) ) ) >> shift0                      (1)

where shift0 = 6, per = qP/6.

With the inverse transform skipped, the residual rij is derived as

rij =  dij                                                                         (2)

For the modification method 1, the inverse quantization is not changed as in (3), and a right shift over the scaled coefficients dij will be conducted to get the residual rij.

dij  =  (cij * levelScale[ qP%6 ] << per + ( 1 << ( shift − 1 ) ) ) >> shift                     (3)

where shift = BitDepthY + log2TrSize – 9.

Let’s represent shift using transformshift as

shift = 6 – transformshift  =  shift0 - transformshift

where transformshift  = 15 - BitDepthY  -  log2TrSize

Correspondingly, we can represent dij as

dij  = (cij * levelScale[ qP%6 ] << per + ( 1 << ( shift0 - transformshift − 1 ) ) )

>> (shift0 - transformshift)                                                 (4)

We can see that the residual rij being consistent with the result in (2) cannot be directly obtained by a right shift of transformshift bits over dij in (4) since the offset of ( 1 << ( shift0 − 1 ) ) in (1) is not equal to the offset of ( 1 << ( shift0 - transformshift − 1 ) ) in (4). To approach the result in (2), we derive the residual rij as

rij = ( dij + (1 << ( transformshift − 1) ) ) >> transformshift                             (5)

Since the proposed transform skipping mode is performed only for 4x4 block size, to be simple, we make the following analysis under this concrete case, where transformshift = 15 – 8 – 2 = 5.

For the modification method 2, residual rij is represented as

rij  =  (cij * levelScale[ qP%6 ] << per + ( 1 << 5 ) ) >> 6                              (6)

For the modification method 1, residual rij is represented as

rij =  ( ((cij * levelScale[ qP%6 ] << per+ ( 1 << 0 ) ) >> 1) + (1 << 4 ) ) >> 5              (7)

Let x = cij * levelScale[ qP%6 ] << per, thus

For the modification method 2, residual rij is represented as

rij  =  (x + ( 1 << 5 ) ) >> 6                                                     (8)

For the modification method 1, residual rij is represented as

rij = ( ( (x+ 1) >> 1) + (1 << 4 ) ) >> 5                                             (9)

We know x can be represented in the form of binary complement. From the complement, we can derive that (8) and (9) are not equivalent when and only when x can be represented with a binary pattern of a011111, where a is a symbol to represent any integer and 011111 are binary bits.

Equivalently, we can say that

1). When x is a positive number, (8) and (9) is not equal when and only when x can be represented by (64a + 31).

2). When x is a negative number, the original value pattern from the complement form can be represented as the 1b100001, where b is a symbol to represent any non-negative integer, 100001 are binary bits, the 1 ahead b indicates this is a negative number. (8) and (9) is not equal when and only when x can be represented by - (64b + 33).

We can see that the cases when x can be represented by (64m + 31) or - (64b + 33) are rare over the integer set. Thus, in most cases, (8) and (9) are equivalent, which means the two methods are equivalent in most cases.

When qP>= 6, per >= 1. Thus x will be even. (8) and (9) will be always equivalent. For the 10 bits configuration, the qP are always not less than 12 and then the two methods will be always equivalent.