| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0414r2 |
| --- | --- |

| Title: | Removal of the restriction on the number of combined merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Oguz Bici Kemal Ugur Jani Lainema | Email: | ext-oguz.bici@nokia.com kemal.ugur@nokia.com jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

In HM6, the number of combined merge candidates is restricted to 5. When this limit on the maximum number of combined candidates was introduced, the design was such that each additional candidate would have a redundancy check with the previous candidates. Therefore, the motivation of the limiting was to limit the worst case redundancy checks. However, in the current design, there is no redundancy check for the additional candidates after the adoption of JCTVC-G397. Therefore, as this limit is obsolete and the number of combined candidates used can never reach 5, it is proposed to remove the aforementioned restriction as a clean-up for the design. Experimental results with common test conditions indicate no coding efficiency difference.

# Introduction

In HM6, the number of combined merge candidates is restricted to 5. Therefore, among the combined candidates whose List0 and List1 motion information are different, maximum of 5 of them are considered. When this limit on the maximum number of combined candidates was introduced, the HEVC design was such that each additional candidate would undergo a redundancy check with the previous candidates. Therefore, the motivation of the restriction was to limit the worst case redundancy checks.

However, the redundancy checks for the additional candidates were removed with the adoption of JCTVC-G397[1]. Moreover, the number of combined candidates used can never reach 5 since the maximum number of merge candidates is 5 and at least 2 initial candidates are required to generate a combined candidate. Therefore, this limit is obsolete and redundant.

# Proposal

It is proposed to remove the restriction on the number of maximum combined candidates. With the proposed change, counting the number of combined candidates that are inserted into the merge list and checking whether this number has exceeded 5 or not is eliminated.

# Results

Simulations are run following common test conditions JCTVC-H1100 [2]. The proposal is compared against HM6.0 software [3] as the anchor. The results are summarized in Table 1. As it is observed from the table, there is no impact on the coding efficiency.

Table 1

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |

# Conclusion

In this document, it is proposed to remove the restriction on the maximum number of combined merge candidates, which is obsolete since the additional candidates do not undergo redundancy checks. Simulation results indicate no coding efficiency change. Therefore, it is recommended to adopt this proposal which would serve as a clean-up for the design.

# Text Change

##### 8.5.2.1.3 Derivation process for combined bi-predictive merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0combCandk and refIdxL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

When numOrigMergeCand is greater than 1 and less than MaxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables combIdx and combCnt are is set to 0, the variable combStop is set to FALSE and the following steps are repeated until combStop is equal to TRUE.

- The variables l0CandIdx and l1CandIdx are derived using combIdx as specified in Error: Reference source not found.

- The following assignments are made with l0Cand being the candidate at position l0CandIdx and l1Cand being the candidate at position l1CandIdx in the merging candidate list mergeCandList ( l0Cand = mergeCandList[ l0CandIdx ] , l1Cand = mergeCandList[ l1CandIdx ] ).

- When all of the following conditions are true,

    - predFlagL0l0Cand = = 1

    - predFlagL1l1Cand = = 1

    - PicOrderCnt( RefPicListL0( refIdxL0l0Cand ) ) != PicOrderCnt( RefPicListL1( refIdxL1l1Cand ) ) | | mvL0l0Cand != mvL1l1Cand

the following applies.

    - The candidate combCandk with k equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = combCandk ) and the reference indices, the prediction list utilization flags and the motion vectors of combCandk are dervied as follows and numMergeCand is incremented by 1.

refIdxL0combCandk = refIdxL0l0Cand	(8-1)

refIdxL1combCandk = refIdxL1l1Cand	(8-2)

predFlagL0combCandk = 1	(8-3)

predFlagL1combCandk = 1	(8-4)

mvL0combCandk[ 0 ] = mvL0l0Cand[ 0 ]	(8-5)

mvL0combCandk[ 1 ] = mvL0l0Cand[ 1 ]	(8-6)

mvL1combCandk[ 0 ] = mvL1l1Cand[ 0 ]	(8-7)

mvL1combCandk[ 1 ] = mvL1l1Cand[ 1 ]	(8-8)

numMergeCand = numMergeCand + 1	(8-9)

    - The variable combCnt is incremented by 1.

- The variable combIdx is incremented by 1.

When combIdx is equal to ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) or numMergeCand is equal to MaxNumMergeCand or combCnt is equal to 5, combStop is set to TRUE.

# References

[1] B. Li, J. Xu, H. Li, "Non-CE9/Non-CE13: Simplification of adding new merge candidates," JCTVC-G397, Nov 2011

[2]	Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100,  San Jose, USA, Feb 2012.

[3]	HM 6.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-6.0.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).