| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0415 |
| --- | --- |

| Title: | Mode-dependent DCT/DST for chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hui Yong Kim, *Kyung Yong Kim, *Gwang Hoon Park, Sung-Chang Lim, Jinho Lee, and Jin Soo Choi ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {hykim5, sclim, jinosoul, jschoi}@etri.re.kr *+82-31-201-3680 *{kimky, ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

In this contribution, mode-dependent DCT/DST selection schemes for 4x4 chroma transform blocks are presented. In the first method, the mode-dependent DCT/DST concept for luma is applied for the explicitly signaled chroma prediction modes (i.e., intra_chroma_pred_mode = 0, ..., 3). In the second method, the same concept is applied for chroma LM mode. In the last method, the first and the second method are combined. It is reported that the three variations of the proposal shows average BD-rate reduction in chroma in range of 1.3~ 1.5% for All-Intra Main and of 0.5~0.9% for All-Intra HE10 configurations. Combined experiment results with JCTVC-I0103 are also provided in this contribution, where some additive gain is observed reportedly. Allegedly, the best result comes from combination of the first proposal of this contribution with JCTVC-I0103.

# Introduction

In the current HM, prediction mode-dependent transform type selection scheme is used for intra luma residuals, as specified in the sub-clause 8.6.4 of the WD6_dK text [1]. Figure 1 summarizes the transform type derivation process of the current HM. Note in the current HM, the mode-dependent DCT/DST selection is only applied to luma signals.

Figure 1. Mode dependent DCT/DST of HM 6

# Proposal

In this proposal, we propose three methods for extending intra prediction mode dependent DCT/DST in 4x4 luma transform blocks to 4x4 chroma transform blocks.

## Proposal 1 (DCT/DST for the explicitely signaled chroma prediction modes)

The first proposal extends the HM’s mode-dependent DCT/DST selection method to chroma EM  modes (the explicitly signaled chroma prediction modes: i.e., intra_chroma_pred_mode = 0, ..., 3). The following figure shows the required change to the current HM, where the modified procedure is highlighted with yellow color. As shown in the figure, the chroma intra prediction mode IntraPredModeC is used as an input for determining the transform types for chroma 4x4 transform. Please refer to Appendix A.1 for corresponding WD modifications in detail.

Figure 2. Proposal 1: Mode dependent DCT/DST for chroma EM modes

## Proposal 2 (DCT/DST for chroma LM mode)

The second proposal extends the HM’s mode-dependent DCT/DST selection method to chroma LM modes. The following figure shows the required change to the current HM, where the modified procedure is highlighted with yellow color. As shown in the figure, the luma intra prediction mode IntraPredMode is used as an input for determining the transform types for chroma 4x4 transform. Please refer to Appendix A.2 for corresponding WD modifications in detail.

Figure 3. Proposal 2: Mode dependent DCT/DST for chroma LM mode

## Proposal 3 (Combination of Proposal 1 and Proposal 2)

The third proposal is a combination of the Proposal 1 and 2, where the HM’s mode-dependent DCT/DST selection method is extended to chroma EM and LM modes. The following figure shows the required change to the current HM, where the modified procedure is highlighted with yellow color. Please refer to Appendix A.3 for corresponding WD modifications in detail.

Figure 4. Proposal 3: Mode dependent DCT/DST for chroma EM and LM modes

# Experimental results

The proposed method was implemented on HM6.0 software. Table 1 shows the summary results of the proposed method against HM 6.0 anchor under common test conditions [2]. Note that slight but consistent chroma gain is observed in the table.

Table 1. Summary result of the Proposal 1

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.7% | -0.7% | 0.0% | -0.3% | -0.5% |
| Class B | 0.1% | -1.1% | -1.2% | 0.0% | -0.6% | -0.6% |
| Class C | 0.0% | -1.6% | -1.9% | 0.0% | -0.9% | -1.0% |
| Class D | 0.0% | -1.9% | -2.2% | 0.0% | -1.2% | -1.6% |
| Class E | 0.0% | -1.3% | -1.7% | -0.1% | -0.2% | -0.9% |
| Overall | 0.0% | -1.3% | -1.5% | 0.0% | -0.7% | -0.9% |
|  | 0.0% | -1.3% | -1.5% | 0.0% | -0.7% | -0.9% |
| Class F | 0.0% | -1.0% | -1.2% | 0.0% | -0.6% | -0.8% |
| Enc Time[%] | 96% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2. Summary result of the Proposal 2

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | Identical to the Anchor |  |  | 0.0% | -0.1% | -0.2% |
| Class B |  |  |  | 0.0% | -0.2% | -0.1% |
| Class C |  |  |  | -0.1% | 0.0% | 0.1% |
| Class D |  |  |  | -0.1% | -0.4% | -0.3% |
| Class E |  |  |  | 0.0% | -0.2% | -0.3% |
| Overall |  |  |  | 0.0% | -0.2% | -0.2% |
|  |  |  |  | 0.0% | -0.2% | -0.2% |
| Class F |  |  |  | 0.0% | 0.2% | 0.3% |
| Enc Time[%] |  |  |  | 99% |  |  |
| Dec Time[%] |  |  |  | 101% |  |  |

Table 3. Summary result of the Proposal 3 (= Proposal 1 + Proposal 2)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | Identical to Proposal 1 |  |  | 0.0% | -0.3% | -0.5% |
| Class B |  |  |  | 0.0% | -0.5% | -0.5% |
| Class C |  |  |  | -0.1% | -0.5% | -0.6% |
| Class D |  |  |  | -0.1% | -1.1% | -1.3% |
| Class E |  |  |  | -0.1% | -0.2% | -0.9% |
| Overall |  |  |  | 0.0% | -0.5% | -0.7% |
|  |  |  |  | 0.0% | -0.5% | -0.7% |
| Class F |  |  |  | -0.1% | -0.2% | -0.4% |
| Enc Time[%] |  |  |  | 100% |  |  |
| Dec Time[%] |  |  |  | 100% |  |  |

# Combined experiments with JCTVC-I0103 [3]

Table 4 and Table 5 shows experiment results of combined experiments of this contribution with JCTVC-I0103 [3], where a mode-dependent scanIdxC derivation for chroma LM mode is proposed. Note that 0.1% additive chroma gain is consistently obtained by combining JCTVC-I0103. By comparing Table 4 and Table 5, we can notice that “Proposal 1 + I0103” provides 0.2% more chroma gain at HE10 while “Proposal 3 + I0103” provides 0.1% more luma gain at HE10. Since luma gains are more important to chroma gain, we can conclude that “Proposal 3 + I0103” provides the best results.

Table 4. Summary result of the Proposal 1 + I0103

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.7% | -0.7% | 0.0% | -0.5% | -0.5% |
| Class B | 0.1% | -1.1% | -1.2% | 0.0% | -0.7% | -0.8% |
| Class C | 0.0% | -1.6% | -1.9% | 0.0% | -1.0% | -1.2% |
| Class D | 0.0% | -1.9% | -2.2% | 0.0% | -1.4% | -1.7% |
| Class E | 0.0% | -1.3% | -1.7% | 0.0% | -0.2% | -0.9% |
| Overall | 0.0% | -1.3% | -1.5% | 0.0% | -0.8% | -1.0% |
|  | 0.0% | -1.3% | -1.5% | 0.0% | -0.8% | -1.0% |
| Class F | 0.0% | -1.0% | -1.2% | -0.1% | -0.9% | -0.9% |
| Enc Time[%] | 96% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | #NUM! |  |  |

Table 5. Summary result of the Proposal 3 + I0103

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | Identical to Proposal 1 + I0103 |  |  | 0.0% | -0.3% | -0.5% |
| Class B |  |  |  | 0.0% | -0.6% | -0.6% |
| Class C |  |  |  | -0.1% | -0.6% | -0.6% |
| Class D |  |  |  | -0.1% | -1.2% | -1.4% |
| Class E |  |  |  | -0.1% | -0.2% | -0.9% |
| Overall |  |  |  | -0.1% | -0.6% | -0.8% |
|  |  |  |  | -0.1% | -0.6% | -0.8% |
| Class F |  |  |  | -0.1% | -0.3% | -0.4% |
| Enc Time[%] |  |  |  | 100% |  |  |
| Dec Time[%] |  |  |  | #NUM! |  |  |

# Conclusions

In this contribution, mode-dependent DCT/DST selection schemes for 4x4 chroma transform blocks were proposed. Experimental results of the three variations of the proposal showed average BD-rate reduction in chroma of 1.3~ 1.5% for All-Intra Main and 0.5~0.9% for All-Intra HE10 configurations. Combined experiment results with JCTVC-I0103 were also provided in this contribution, where some additive gain is observed. The best results were from “Proposal 3 + I0103”, where 1.3~1.5% chroma gain in AI-Main and 0.1% luma gain and 0.7% chroma gain were observed without noticeable complexity increase of the design. We recommend adopting “Proposal 3 + I0103” into the next version of HM.

# Reference

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand (Editors), “High Efficiency Video Coding (HEVC) text specification draft 6,” JCTVC-H1003, Feb. 2012.

- F. Bossen, "Common test conditions and software reference configurations," JCTVC-H1100, Feb. 2012.

- H. Y. Kim, et al., “Non-CE3: On scanIdxC derivation for chroma LM mode,” JCTVC-I0103, April, 2012.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Required WD modifications (based on WD6_dK)

## A.1 Proposal 1

    - Transformation process for scaled transform coefficients

......

Depending on PredMode, cIdx, and IntraPredMode, IntraPredModeC, and intra_chroma_pred_mode the following applies:

–	If PredMode is equal to MODE_INTRA and Log2(nW*nH) is equal to 4, and the following applies.

–	If cIdx is equal to 0, the variables horizTrType and vertTrType are specified as Table 8-1 with IntraPredMode as input.

–	Otherwise, if intra_chroma_pred_mode is less than 4, the variables horizTrType and vertTrType are specified as Table 8-1 with IntraPredModeC as input.

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-1 – Specification of horizTrType and vertTrType

| IintraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| IintraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| horizTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

## A.2 Proposal 2

    - Transformation process for scaled transform coefficients

......

Depending on PredMode, cIdx, and IntraPredMode, and IntraPredModeC, the following applies:

–	If PredMode is equal to MODE_INTRA, Log2(nW*nH) is equal to 4, and one of the following conditions is true, the variables horizTrType and vertTrType are specified as Table 8-1 with IntraPredMode as input.

–	cIdx is equal to 0

–	cIdx is equal to 1 or 2, and IntraPredModeC is equal to Intra_FromLuma

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-2 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

## A.3 Proposal 3

    - Transformation process for scaled transform coefficients

......

Depending on PredMode, cIdx, and IntraPredMode, IntraPredModeC, and intra_chroma_pred_mode the following applies:

–	If PredMode is equal to MODE_INTRA and Log2(nW*nH) is equal to 4, and the following applies.

–	If cIdx is equal to 0, the variables horizTrType and vertTrType are specified as Table 8-1 with IntraPredMode as input.

–	Otherwise if IntraPredModeC is equal to Intra_FromLuma, the variables horizTrType and vertTrType are specified as Table 8-1 with IntraPredMode as input.

–	Otherwise, if intra_chroma_pred_mode is less than 4, the variables horizTrType and vertTrType are specified as Table 8-1 with IntraPredModeC as input.

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-3 – Specification of horizTrType and vertTrType

| IintraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| IintraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |