| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0416 |
| --- | --- |

| Title: | On definition of ref_pic_list_combination_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution relaxes the condition to signal identical reference picture lists using the ref_pic_list_combination_flag syntax.

# Introduction

In current WD6, ref_pic_list_combination_flag equal 0, indicates that reference list 0 and reference list 1 are identical, i.e., same number of reference pictures in each list and their elements are the same. Another alternative to the definition of identical lists is to define two lists to be identical if the first N = min(num_ref_idx_l0_active_minus1, num_ref_idx_l1_active1_minus1)+1 of the shorter list, (reflist0 or reflist1), is equal to the first (top) N elements of the longer reference list. An example of identical lists is shown below with unequal number of elements in each reference list.

| Reference list 0, num_ref_idx_l0_active_minus1 = 1 | Reference list 1, num_ref_idx_l1_active_minus1 = 3 |
| --- | --- |
| Idx = 0, POC = 4 | Idx = 0, POC = 4 |
| Idx = 1, POC = 3 | Idx = 1, POC = 3 |
|  | Idx = 2, POC = 2 |
|  | Idx = 3, POC = 1 |

In this method, the longer of the two lists is used to signal syntax elements, and the corresponding elements on the shorter can be derived from the longer list by truncating the longer list after N elements from the top.

The new definition of ref_pic_list_combination_flag would be:

ref_pic_list_combination_flag equal to 1indicates that the reference picture list 0 and the reference picture list 1 are combined to be an additional reference picture lists combination used for the prediction units being uni-directional predicted. This flag equal to 0 indicates that the reference picture list 0 and reference picture list 1 are identical thus the list that has larger number of num_ref_active_lX  is used as the reference picture lists combination. The reference picture lists combination is set to be empty at the start of the loop defined in this table.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).