| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0417 WG11 Number: m24666 |
| --- | --- |

| Title: | Cross-verification of Intra Transform Skipping (JCTVC-I0408) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of certain cases from JCTVC-I0408, “Intra Transform Skipping.” The cases verified were transform skip mode for TUs of size 4x4-8x8, 4x4-16x16, and 4x4-32x32. The software was examined, and three groups of simulations were performed. The software appears to be consistent with the explanations given in JCTVC-I0408. The simulation results performed for the cross-verification match those provided by the proponents.

# Introduction and examination of software

The proposed method enables a transform skip mode for luma and chroma Intra TUs. When a transform is skipped, the data is scaled. A flag and associated entropy coding contexts are added to each TU to indicate whether skip mode is enabled. A flag to enable this tool is also added to the SPS. The proposal also modifies the 4x4 default quantization matrix so it becomes flat.

The changes to the software that affect the decoder include the following: Modification of the transform functions to use a skip-mode flag; additional quantization-related functions to scale the skipped data; modifications to the entropy coder to include the skip flag; addition of a m_bUseRSQ flag to the Sequence Parameter Set. Modifications to the encoder include changing the RDO loop to also handle the skip modes; modification to the chroma prediction functions.

The overall number of sections of code that were changed for both the encoder and decoder is approximately 113. Changes to the encoder description and text specifications were also provided. Because the proposal recommends only having this skip mode for 4x4 TUs, the proposed modifications to the text specification are for transform blocks with nW and nH both equal to 4. The modifications to the encoder description, however, supports transform sizes from 4x4 to 32x32. If this proposal is adopted, a decision will need to be made as to whether the encoder description changes should be only for 4x4 blocks, to match the changes to the decoder text specification.

The proposal also refers to a slide deck for visual comparisons. Those slides, however, were not available at the time of this cross-verification.

# Performance

The performance of the proposed tool is compared to a reference using HM-6.0. For this cross-verification, three experiments were performed to handle cases where the skip mode was supported for 4x4-8x8, 4x4-16x16, and 4x4-32x32 transform block sizes. The changes to the compile-time flags for these experiments are:

Skip mode for 4x4 and 8x8:

#define INTRA_RSQ                 1

#if INTRA_RSQ

#define ONLY_4x4_RSQ              0

#define ONLY_4x4_8x8_RSQ          1

#define ONLY_4x4_8x8_16x16_RSQ    0

#define INTRA_RSQ_FAST            0

#endif

Skip mode for 4x4, 8x8, and 16x16:

#define INTRA_RSQ                 1

#if INTRA_RSQ

#define ONLY_4x4_RSQ              0

#define ONLY_4x4_8x8_RSQ          0

#define ONLY_4x4_8x8_16x16_RSQ    1

#define INTRA_RSQ_FAST            0

#endif

Skip mode for 4x4, 8x8, 16x16, and 32x32:

#define INTRA_RSQ                 1

#if INTRA_RSQ

#define ONLY_4x4_RSQ              0

#define ONLY_4x4_8x8_RSQ          0

#define ONLY_4x4_8x8_16x16_RSQ    0

#define INTRA_RSQ_FAST            0

#endif

## Skip mode enabled for 4x4 and 8x8 TUs

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | -0.6% | 0.1% | -0.1% | -0.5% |
| Class B | 0.1% | 0.0% | -0.6% | 0.1% | -0.1% | -0.6% |
| Class C | 0.0% | -0.1% | -0.4% | 0.0% | 0.0% | -0.3% |
| Class D | 0.0% | 0.0% | -0.4% | 0.0% | 0.1% | -0.2% |
| Class E | 0.2% | -0.1% | -0.9% | 0.1% | -0.1% | -0.8% |
| Overall | 0.1% | 0.0% | -0.5% | 0.0% | 0.0% | -0.5% |
|  | 0.1% | -0.1% | -0.6% | 0.0% | 0.0% | -0.5% |
| Class F | -8.2% | -7.9% | -8.6% | -8.1% | -7.2% | -7.8% |
| Enc Time[%] | 147% |  |  | 144% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

## Skip mode enabled for 4x4, 8x8, and 16x16 TUs

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | -1.1% | 0.1% | -0.1% | -0.9% |
| Class B | 0.1% | -0.1% | -0.8% | 0.1% | -0.1% | -0.8% |
| Class C | 0.0% | -0.1% | -0.4% | 0.0% | 0.0% | -0.4% |
| Class D | 0.1% | 0.0% | -0.4% | 0.0% | 0.1% | -0.3% |
| Class E | 0.2% | -0.1% | -1.2% | 0.2% | -0.1% | -1.0% |
| Overall | 0.1% | -0.1% | -0.8% | 0.1% | 0.0% | -0.7% |
|  | 0.1% | -0.1% | -0.8% | 0.1% | 0.0% | -0.7% |
| Class F | -8.3% | -8.0% | -9.0% | -8.2% | -7.6% | -8.3% |
| Enc Time[%] | 158% |  |  | 154% |  |  |
| Dec Time[%] | 102% |  |  | 98% |  |  |

## Skip mode enabled for 4x4, 8x8, 16x16, and 32x32 TUs

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.2% | -1.1% | 0.1% | -0.1% | -1.0% |
| Class B | 0.1% | -0.1% | -0.8% | 0.1% | -0.1% | -0.8% |
| Class C | 0.0% | -0.1% | -0.4% | 0.0% | 0.0% | -0.4% |
| Class D | 0.1% | 0.0% | -0.4% | 0.0% | 0.1% | -0.3% |
| Class E | 0.2% | -0.1% | -1.2% | 0.2% | -0.1% | -1.0% |
| Overall | 0.1% | -0.1% | -0.8% | 0.1% | 0.0% | -0.7% |
|  | 0.1% | -0.1% | -0.8% | 0.1% | 0.0% | -0.7% |
| Class F | -8.3% | -8.1% | -9.0% | -8.3% | -7.6% | -8.4% |
| Enc Time[%] | 173% |  |  | 167% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# Conclusions

The simulation results for the cross-check match those of the proponent. Encode and decode times are consistent as well. The decoder-related modifications to the software appear to be consistent with what is described in JCTVC-I0224.

# References

- C. Lan, J. Xu, G. J. Sullivan, F. Wu, “Intra transform skipping,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I0408, Geneva, CH, April 2012.