| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0418_r1 WG11 Number: m24667 |
| --- | --- |

| Title: | Cross-verification of Results for Non-Square Intra Prediction with NSQT (JCTVC-I0224) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of JCTVC-I0224, “Results for Non-Square Intra Prediction with NSQT.” The software was examined, and two groups of simulations were performed. The software appears to be consistent with the explanations given in JCTVC-I0224. The simulation results performed for the cross-verification match those provided by the proponents.

# Introduction and examination of software

The proposed method allows rectangular partitions, 2NxN and Nx2N, to be used for Intra prediction in addition to the existing 2nx2N and NxN partitions. Because there are more than two partition types, a binarization table for the total of four intra partition modes is provided. The transforms that already are being used for non-square transforms (NSQT) are also used for these new Intra PUs. To reduce encoder complexity, an early-skip optimization is used. A second experiment includes additional encoder optimizations that reduce the number of candidate modes.

The changes to the software that affect the decoder include the following: Changes to get the address (location) of the new non-square partitions; addition of an Intra prediction mode parameter for the MxN transforms; addition of a parameter to the DST transform to support non-square blocks; additional scan orders for these non-square partitions; modifications of the Intra prediction functions to support blocks where the width and height are not equal; modification of the entropy decoder to decode up to an additional two bits when parsing the partition size, and other modifications. Many of these changes are consistent with what one would expect, in that existing NSQT-related functions are modified to include the Intra prediction mode, and many of the addressing and pattern-related functions were changed to allow for non-square Intra blocks. The encoder optimizations are non-normative, so encoder-specific optimizations such as the early skip modes were not examined for this cross-verification. The overall number of sections of code that were changed, including both the encoder and decoder, is approximately 263. The only syntax provided at this time is a table of proposed partition modes and corresponding binarizations.

# Performance

The performance of the proposed tool is compared to a reference using HM-6.0. For this cross-verification, two experiments were performed. The proposed method was first tested with the following parameters:

#define NS_INTRA                      1

#if NS_INTRA

#define NS_INTRA_EARLYSKIP            1        // NSIP earlyskip, Turin meeting based encoder method

#define NS_INTRA_fast                 0

#define NS_INTRA_SCAN                 1

#endif

The second experiment used the following parameters, which enabled a second set of encoder optimizations:

#define NS_INTRA                      1

#if NS_INTRA

#define NS_INTRA_EARLYSKIP            1        // NSIP earlyskip, Turin meeting based encoder method

#define NS_INTRA_fast                 1

#define NS_INTRA_SCAN                 1

#endif

## Proposed method

Nebu BQTersequestill bei

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.9% | -2.5% | -2.7% |
| Class B | -1.3% | -3.0% | -3.3% |
| Class C | -1.3% | -2.5% | -2.8% |
| Class D | -1.2% | -1.9% | -1.8% |
| Class E | -2.0% | -3.5% | -4.4% |
| Overall | -1.3% | -2.7% | -3.0% |
|  | -1.3% | -2.7% | -3.0% |
| Class F | -1.3% | -1.6% | -1.4% |
| Enc Time[%] | 129% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | -0.6% | -1.1% | -2.0% |
| Class B | -0.7% | -1.5% | -1.6% |
| Class C | -0.6% | -1.4% | -1.6% |
| Class D | -0.5% | -0.8% | -0.8% |
| Class E |  |  |  |
| Overall | -0.6% | -1.2% | -1.5% |
|  | -0.6% | -1.2% | -1.5% |
| Class F | -0.9% | -1.2% | -1.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2% | -1.0% | -0.4% |
| Class C | -0.3% | -0.5% | -0.7% |
| Class D | -0.2% | -1.1% | -0.2% |
| Class E | -0.4% | -1.1% | -1.7% |
| Overall | -0.3% | -0.9% | -0.6% |
|  | -0.3% | -0.8% | -0.6% |
| Class F | -0.5% | -1.0% | 0.8% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

  - Proposed method with additional optimization

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.7% | -2.4% | -2.5% |
| Class B | -1.1% | -3.0% | -3.3% |
| Class C | -1.0% | -2.4% | -2.6% |
| Class D | -0.9% | -1.8% | -1.8% |
| Class E | -1.8% | -3.4% | -4.4% |
| Overall | -1.1% | -2.6% | -2.9% |
|  | -1.1% | -2.6% | -2.9% |
| Class F | -1.1% | -1.5% | -1.5% |
| Enc Time[%] | 123% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusions

The simulation results for the cross-check match those of the proponent. Encode and decode times so far are consistent as well. The decoder-related modifications to the software appear to be consistent with what is described in JCTVC-I0224.

# References

- X. Zhang, S. Liu, and S. Lei, “Results for Non-Square Intra Prediction with NSQT,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I0224, Geneva, CH, April 2012.