| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0419_r1 WG11 Number: m24668 |
| --- | --- |

| Title: | Syntax for supporting 4x4 DST quantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Cohen Anthony Vetro Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com avetro@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

The current text specification draft includes two different transforms for use with 4x4 Intra Luma Transform Blocks. The same scaling list (quantization matrix), however, is associated with both of these transforms. The purpose of this document is to propose modifications to the draft text so that different scaling lists can be associated with these transforms. Two versions are provided, to support one additional and three additional scaling lists respectively.

# Introduction

In the High Efficiency Video Coding (HEVC) text specification draft 6 [1], if scaling_list_enable_flag is equal to 1, then a quantization matrix (scaling list) is used for scaling transform coefficients. For 4x4 Intra Luma Transform Blocks, two transforms are available for use. These are commonly, although non-normatively, referred to as the HM core transform and the DST. The current text specification, however, uses the same quantization matrix for both the HM transform and the DST. The purpose of this document is to propose modifications to the text specification to include support for an additional 4x4 scaling list, which can be used on Transform Blocks that use the DST. The syntax proposed here makes no changes to the use of the scaling_list_pred_matrix_id_delta flag, and is thus is consistent with the existing syntax.

# Changes to the syntax

## Support for one additional scaling list

Additions are indicated with yellow highlighting.

Deletions are indicated with blue highlighting.

#### 7.3.2.3   Scaling list data syntax

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| scaling_list_present_flag | u(1) |
| if( scaling_list_present_flag ) |  |
| for( sizeID = 0; sizeID < 4; sizeID++ ) |  |
| for( matrixID = 0; matrixID < (sizeID = = 3) ? 2 : 67; matrixID++ ) { |  |
| scaling_list_pred_mode_flag | u(1) |
| if( !scaling_list_pred_mode_flag ) |  |
| scaling_list_pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( ScalingList[ sizeID ][ matrixID ], sizeID , matrixID ) |  |
| } |  |
| } |  |

Table 7-3 – Specification of MatrixID according to SizeID, prediction type and colour component

| - SizeID | - Prediction type | - Colour component | - MatrixID |
| --- | --- | --- | --- |
| - 0, 1, 2 | - Intra | - Y | - 0 |
| - 0, 1, 2 | - Intra | - Cb | - 1 |
| - 0, 1, 2 | - Intra | - Cr | - 2 |
| - 0, 1, 2 | - Inter | - Y | - 3 |
| - 0, 1, 2 | - Inter | - Cb | - 4 |
| - 0, 1, 2 | - Inter | - Cr | - 5 |
| - 0 | - Intra | - Y (DST) | - 6 |
| - 3 | - Intra | - Y | - 0 |
| - 3 | - Inter | - Y | - 1 |

Table 7-4 – Specification of default values of ScalingList[ 0 ][ MatrixID ][ i ] with i=0..15

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[0][0..2][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 20 | - 20 | - 21 | - 25 | - 30 | - 25 | - 41 | - 41 | - 70 |
| - ScalingList[0][3..5][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 21 | - 21 | - 21 | - 24 | - 24 | - 24 | - 36 | - 36 | - 57 |
| - ScalingList[0][6][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 20 | - 20 | - 21 | - 25 | - 30 | - 25 | - 41 | - 41 | - 70 |

## Support for three additional scaling lists

In JCTVC-I0268 [2], additional scaling lists are proposed for when the DST is used. Their example of specifying values for MatrixID, however, includes distinct entries for chroma components. The current HEVC text specification, however, applies the DST only to luma Intra 4x4 blocks. The syntax of Section 2.1 above can be extended to support the quantization matrices of JCTVC-I0268 while minimizing syntax changes by only including matrices for luma, as follows:

Additions are indicated with yellow highlighting.

Deletions are indicated with blue highlighting.

#### 7.3.2.3   Scaling list data syntax

| scaling_list_param( ) { | Descriptor |
| --- | --- |
| scaling_list_present_flag | u(1) |
| if( scaling_list_present_flag ) |  |
| for( sizeID = 0; sizeID < 4; sizeID++ ) |  |
| for( matrixID = 0; matrixID < (sizeID = = 3) ? 2 : 69; matrixID++ ) { |  |
| scaling_list_pred_mode_flag | u(1) |
| if( !scaling_list_pred_mode_flag ) |  |
| scaling_list_pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( ScalingList[ sizeID ][ matrixID ], sizeID , matrixID ) |  |
| } |  |
| } |  |

Table 7-3 – Specification of MatrixID according to SizeID, prediction type and colour component

| - SizeID | - Prediction type | - Colour component | - MatrixID |
| --- | --- | --- | --- |
| - 0, 1, 2 | - Intra | - Y | - 0 |
| - 0, 1, 2 | - Intra | - Cb | - 1 |
| - 0, 1, 2 | - Intra | - Cr | - 2 |
| - 0, 1, 2 | - Inter | - Y | - 3 |
| - 0, 1, 2 | - Inter | - Cb | - 4 |
| - 0, 1, 2 | - Inter | - Cr | - 5 |
| - 0 | - Intra | - Y (ver dst) | - 6 |
| - 0 | - Intra | - Y (hor DST) | - 7 |
| - 0 | - Intra | - Y (ver & hor DST) | - 8 |
| - 3 | - Intra | - Y | - 0 |
| - 3 | - Inter | - Y | - 1 |

Table 7-4 – Specification of default values of ScalingList[ 0 ][ MatrixID ][ i ] with i=0..15, using scaling matrix coefficients from JCTVC-I0268

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[0][0..2][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 20 | - 20 | - 21 | - 25 | - 30 | - 25 | - 41 | - 41 | - 70 |
| - ScalingList[0][3..5][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 21 | - 21 | - 21 | - 24 | - 24 | - 24 | - 36 | - 36 | - 57 |
| - ScalingList[0][6][i] | - 8 | - 8 | - 7 | - 8 | - 13 | - 13 | - 12 | - 14 | - 19 | - 21 | - 29 | - 36 | - 25 | - 30 | - 47 | - 78 |
| - ScalingList[0][7][i] | - 8 | - 13 | - 19 | - 25 | - 8 | - 13 | - 21 | - 30 | - 7 | - 12 | - 29 | - 47 | - 8 | - 14 | - 36 | - 78 |
| - ScalingList[0][8][i] | - 5 | - 7 | - 8 | - 10 | - 7 | - 12 | - 15 | - 17 | - 8 | - 15 | - 29 | - 43 | - 10 | - 17 | - 43 | - 87 |

# References

- B. Bross et al., “High Efficiency Video Coding (HEVC) text specification draft 6,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H1003_dK (JCTVC-H1003-v22.zip), San José, CA, February 2012.

- J. Lou and L. Wang, “Quantization matrices for 4x4 DSTs in HEVC,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I0268, Geneva, CH, April 2012

# Patent rights declaration(s)

Mitsubishi Electric Research Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).