| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0420 |
| --- | --- |

| Title: | High-level Syntax: Proposed fix on signaling of TMVP disabling flag |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |  |
| Purpose: | Proposal |  |  |  |  |
| Author(s) or Contact(s): | Chong Soon Lim Sue Mon Thet Naing Jizheng Xu Bin Li | Tel: Email: | (65) 6550 5368 chongsoon.lim@sg.panasonic.com sue.naing@sg.panasonic.com |  |  |
| Source: | Panasonic Corporation Microsoft Corporation University of Science and Technology of China |  |  |  |  |

_____________________________

Abstract

This contribution proposes to explicitly signal the enable_temporal_mvp_flag at every P and B slice header. It is purported that the benefit includes cleaner design for TMVP disabling process. It is suggested that the JCT-VC consider this proposal in refining the design of TMVP disabling process.

# Introduction

JCTVC-G398 is adopted in 7th JCT-VC meeting (Geneva, CH), which provides a complete design to the TMVP disabling flag. The adopted scheme introduces an enable_temporal_mvp_flag syntax element to the PPS. When a slice (or picture) refers to a PPS with enable_temporal_mvp_flag value equal to 0 and temporal_id equals to 0, the marking process of reference pictures in the DPB is invoked. Furthermore, all pictures in the DPB are marked as “unused for temporal motion vector prediction”.

In 8th JCT-VC meeting (San José, USA), it was agreed to signal temporal_id syntax in all NAL unit headers as proposed in JCTVC-H0388. In addition, temporal_id is signalled in PPS, as well as in slices.

# Problem Statement

Due to the adoption of JCTVC-H0388, signaling of temporal_id in all NAL unit headers, the description for the marking process specified in Error: Reference source not found becomes ambiguous while the process is triggered by enable_temporal_mvp_flag and temporal_id syntax elements.

8.3.5 Marking of reference pictures before decoding

This process is invoked once per picture, after decoding of a slice header and the decoding process for reference picture set as specified in Error: Reference source not found but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause Error: Reference source not found.

When enable_temporal_mvp_flag of the current picture is equal to 0 and temporal_id of the current picture is equal to 0, all the reference pictures in the DPB are marked as "unused for temporal motion vector prediction".

When enable_temporal_mvp_flag is signaled in PPS, the classification of stated temporal_id in the marking process is vague and can be assumed that it is a PPS’s temporal_id. Therefore, the process may result in inter prediction errors where a group of pictures share the same PPS parameters and temporal_id value.

Even if the above marking process is triggered correctly, a slice/picture that triggers the marking process can be lost or contains errors. Then, a video decoder may not know that the marking operation should be done and the last non-TMVP picture should be updated. This leads to a mismatch between the encoder and decoder.

# Proposal

It is proposed to send the enable_temporal_mvp_flag at the slice headers of all P and B slices. This explicit method is conceptually similar to DPB management scheme using reference picture set, where DPB configuration is explicitly signalled in the bitstream instead of relying on frame-by-frame update of a buffer state.

TMVP is derived from the collocated picture, which is either RefPicList0[0] or RefPicList1[0], depending on the value of collocated_from_l0_flag. Therefore, a video encoder should keep track of the intended non-TMVP frame and should set the enable_temporal_mvp_flag accordingly to ensure that there is no TMVP dependency on the pictures preceding the non-TMVP frame in coding and display order.

- Diagram showing the current TMVP disabling with marking process (enable_temporal_mvp_flag signalling at PPS)

- Diagram showing proposed TMVP disabling without marking process (enable_temporal_mvp_flag signalling at a slice header of P or B slice)

The proposed method removes the need for a marking process and determines the TMVP pictures independently instead of relying on frame-by-frame update of a buffer state. The processing burden is placed on the encoder for correctly setting the enable_temporal_mvp_flag, while the decoder only needs to retrieve enable_temporal_mvp_flag to determine whether to perform temporal motion vector prediction.

# Simulation Results

The data below shows the results of HM-6.1 compared to signaling of “enable_temporal_mvp_flag” in slice headers and simulated results of TMVP disabling mechanism for “refresh point” B-pictures (temporal_id equal to 0).

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

- Signaling of “enable_temporal_mvp_flag” flag at slice headers for all P & B slices

Table 1 shows the results for signaling of “enable_temporal_mvp_flag” flag at slice headers for all P & B slices. HM-6.1 revision 2183 with common test condition setting is used as the reference for the comparison.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.6% | 0.5% | 0.4% | 0.5% | 0.1% | 0.4% |
| Class B | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% |
| Class C | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% |
| Class D | 0.4% | 0.4% | 0.3% | 0.3% | 0.1% | 0.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.2% |
|  | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.2% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

- Disabling TMVP at every “refresh point” B-picture

Table 2 shows the results for disabling TMVP at every “refresh point” B-picture when temporal_id is equal to 0. I0420, signaling of “enable_temporal_mvp_flag” flag at slice headers with TMVP enabled to all slices is used as the reference and compared to TMVP disabled at “refresh point” B-pictures.

# Conclusion

This contribution proposes to explicitly signal the enable_temporal_mvp_flag at every P and B slice header. It is purported that the benefit includes cleaner design for TMVP disabling process. It is suggested that the JCT-VC consider this proposal in refining the design of TMVP disabling process.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “CD: High efficiency video coding (HEVC) text specification draft 6,” JCTVC-H1103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting, San José, CA, USA, 1–10 February, 2012.

- B. Li, J. Xu, H. Li, “High-level Syntax: Marking process for non-TMVP pictures,” JCTVC-G398, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, CH, November 2011.

- J. Boyce, S. Wenger, W. Jang, D. Hong, Y.K. Wang, Y. Chen, “High level syntax hooks for future extensions,” JCTVC-H0388, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting, San José, CA, USA, 1–10 February, 2012.

- V.Wahadaniah, C.S.Lim, J. Xu, B. Li “AHG14/AHG15/non-CE9: Loss robustness issue in TMVP disabling”, JCTVC-H0570, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting, San José, CA, USA, 1–10 February, 2012.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Proposed changes for HEVC WD6 (dk)

Proposed CD text is included below using the following text conventions:

- Unchanged text that is not shown here is indicated with "…"

- Removed text is shown in blue with strikethrough.

- Newly added text is shown in yellow highlight.

7.3.2.2	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| enable_temporal_mvp_flag | u(1) |
| … |  |
| } |  |

7.3.3	Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| enable_temporal_mvp_flag | u(1) |
| num_ref_idx_active_override_flag | ue(v) |
| … |  |
| } |  |
| … |  |
| if ( enable_temporal_mvp_flag) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ (weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| … |  |

7.4.2.2	Picture parameter set RBSP semantics

…

enable_temporal_mvp_flag specifies whether temporal motion vector predictor can be used or not. If enable_temporal_mvp_flag is equal to 1, temporal motion vector predictors are used; otherwise (enable_temporal_mvp_flag is equal to 0), the temporal motion vector predictors are not used. enable_temporal_mvp_flag equal to 0 together with temporal_id equal to 0 also indicates that the marking process will occur before decoding the first picture with an output order greater than the current non-TMVP picture.

…

7.4.3	Slice header semantics

…

enable_temporal_mvp_flag specifies whether temporal motion vector predictor can be used or not. If enable_temporal_mvp_flag is equal to 1, temporal motion vector predictors are used; otherwise (enable_temporal_mvp_flag is equal to 0), the temporal motion vector predictors are not used. enable_temporal_mvp_flag equal to 0 together with temporal_id equal to 0 also indicate that any pictures that follow the current picture both in decoding order and output order shall not use temporal motion vector prediction from any picture that precedes the current picture either in decoding order or output order.

NOTE – When a slice of a coded picture has temporal_id equal to 0, the value of enable_temporal_mvp_flag shall be the same in all slice headers of the picture. When a slice of a coded picture has the value of enable_temporal_mvp_flag equal to 0 and temporal_id equal to 0, that picture represents a refresh point where the propagation of temporal motion vector prediction is terminated.

…

    - Marking of reference pictures before decoding

This process is invoked once per picture, after decoding of a slice header and the decoding process for reference picture set as specified in 8.3.2 but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause 8.3.3.

When enable_temporal_mvp_flag of the current picture is equal to 0 and temporal_id of the current picture is equal to 0, all the reference pictures in the DPB are marked as "unused for temporal motion vector prediction".

8.4.2.1.7	Derivation process for temporal luma motion vector prediction

…

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If one of the following conditions is true, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

  - colPu is coded in an intra prediction mode.

  - colPu is unavailable.

  - colPic is marked as “unused for temporal motion vector prediction”.

  - enable_temporal_mvp_flag is equal to 0.

…