| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0422 |
| --- | --- |

| Title: | High-level Syntax: MVP scaling issue for LTRPs |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Chong Soon Lim Sue Mon Thet Naing | Tel: Email: | (65) 6550 5368 chongsoon.lim@sg.panasonic.com sue.naing@sg.panasonic.com |  |
| Source: | Panasonic Corporation |  |  |  |

_____________________________

# Abstract

This contribution proposes to disable the motion vector scaling process and implicit weighted prediction process when a picture refers to a long-term reference picture as a reference for inter prediction. The proposed AVC-like method support motion vector scaling and implicit weighted prediction with consideration of different characteristics of short-term and long-term reference pictures. It is suggested that the JCT-VC considers this proposal in refining the specification for MVP scaling process and implicit weighted prediction.

# Introduction

In HEVC committee draft (CD), supports motion vector (MV) predictions and MV scaling for spatial and temporal predictions. A motion vector of a current picture is predicted from a collocated/neighbouring prediction unit (PU) and the predicted motion vector (PVM) is scaled according to POC (picture order count) distance between the current picture and the selected reference picture.

A reference picture is classified as a short-term reference picture (STRP) or a long-term reference picture (LTRP). However, current CD does not distinguish between motion vector prediction from short-term and long-term reference pictures and MV scaling is applied regardless of the reference picture type. Similarly, current CD does not distinguish implicit weighted prediction from short-term and long-term reference pictures.

# Problem Statement

POC-based MV scaling scheme in CD does not consider long term reference pictures where the delta POC difference of a target picture and a LTRP may be extremely large. In that case, performing POC-based scaling on an MV which points to a long term reference picture may result in the scaled MV having an extremely large or small magnitude. In such a case, the accuracy and efficiency of MV prediction process becomes suboptimal.

# Proposal

Similar to AVC, the proposed method disable the POC-based MV scaling and implicit weighted prediction when a picture refers to a long-term reference picture as a reference.

For spatial MV prediction, POC-based MV scaling is disabled when any of the following conditions is true (i.e. taking a neighbouring PU’s MV directly as a spatial PMV):

- target PU refers to a long term reference picture;

- neighbouring PU refers to a long term reference picture.

For temporal MV prediction, POC-based MV scaling is disabled when any of the following conditions is true (i.e. taking a collocated PU’s MV directly as a temporal PMV):

- target PU refers to a long term reference picture;

- collocated PU refers to a long term reference picture.

For implicit weighted prediction, default weights are selected when any of the following conditions is true:

- List 0 refIdx refers to a long term reference picture;

- List 1 refIdx refers to a long term reference picture.

# Conclusion

This contribution proposes to disable the motion vector scaling process and implicit weighted prediction process when a picture refers to a long-term reference picture as a reference for inter prediction. The proposed AVC-like method support motion vector scaling and implicit weighted prediction with consideration of different characteristics of short-term and long-term reference pictures. It is suggested that the JCT-VC considers this proposal in refining the specification for MVP scaling process.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “CD: High efficiency video coding (HEVC) text specification draft 6,” JCTVC-H1103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting, San José, CA, USA, 1–10 February, 2012.

- Y.-K. Wang, M. M. Hannuksela, T. K. Tan, R. Sjöberg, “Common conditions for reference picture marking and list construction proposals”, JCTVC-G1036, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, CH, November 2011.

- G. J. Sullivan, Y. Wu, J. Xu, B. Li “AHG21: Proposal on Decoded Picture Buffer Description Syntax Relating to JCTVC-F493”, JCTVC-G0788, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, CH, November 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)