| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0423 |
| --- | --- |

| Title: | Cross-verification of JCTVC-I0177: Table size reduction and CABAC context reduction for last significant position coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye 9710 Scranton Rd, Ste 250, San Diego, CA92121 | Tel: Email: | +1-858-210-4809 jie.dong@interdigital.com yan.ye@interdigital.com |
| Source: | InterDigital Communications |  |  |

_____________________________

# Abstract

This document is a cross-check report on HKUST's proposal JCTVC-I0177 on table size and context reduction for the last significant position coding in CABAC. It is verified that the encoding/decoding results completely match those reported in I0177, and the description in JCTVC- I0177 matches the implementation in the software.

# Introduction

HKUST's contribution, JCTVC-I0177, proposes two modifications of the last significant coefficient position coding: reducing the size of the LUT for ctxIdxInc and reducing the contexts for coding bins. The first modification provides bit-exact results of HM-6.0, because the eliminated entries in the LUT are actually never used; the second modification trades off a marginal performance loss for the context reduction. In this cross-check, the source code and BD-rate results of the proposal are examined and verified.

# Experimental Results

The implementation of the proposal is based on HM-6.0, and the modified places in the software are indicated by the macros “HKUST_LAST_Table_Reduction” and “HKUST_CtxSim” for table size reduction and context simplification, respectively. The modified source code matches the description in the proposal. The cross-check experiments were conducted on 64-bit Linux cluster under the common test condition JCTVC-H1100. Note that we didn’t test the optional test settings: low delay P Main and low delay P HE10. Tables 1 and 2 show the performances of the two parts of the proposal, respectively, which are identical to what the proponents report.

Table 1 BD-rate of LUT size reduction

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 177% |  |  | 131% |  |  |
| Dec Time[%] | 153% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 148% |  |  | 112% |  |  |
| Dec Time[%] | 144% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  | 108% |  |  |
| Dec Time[%] | 102% |  |  | 98% |  |  |

Table 2 BD-rate of context simplification

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 122% |  |  | 137% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | -0.1% | 0.1% | -0.1% | 0.0% |
| Enc Time[%] | 110% |  |  | 116% |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.4% | -0.1% | 0.1% | -0.2% | 0.0% |
| Class C | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.1% | -0.3% | 0.2% | 0.0% | -0.2% | 0.0% |
| Class E | 0.0% | -0.1% | 0.5% | 0.0% | 0.1% | 0.3% |
| Overall | 0.1% | -0.2% | 0.1% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.2% | 0.0% | -0.1% | -0.1% |
| Class F | -0.1% | 0.1% | 0.3% | 0.1% | -0.9% | 2.2% |
| Enc Time[%] | 114% |  |  | 101% |  |  |
| Dec Time[%] | 110% |  |  | 91% |  |  |

# Conclusion

It was verified that the simulation results matched exactly those reported by the proponent, and the description in the proposed scheme in JCTVC-I0177 matches the implementation in the software.