| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC- I0424 |
| --- | --- |

| Title: | Non-CE2: Cross check of eBrisk’s ALF filter modification (JCTVC-I0287, JCTVC-I0289) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | I. S. Chong and M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-845-5477 Email: ichong@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports verification results of JCTVC- I0287 and JCTVC- I0289 for eBrisk’s Non-CE2 proposal.  Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time have some degree of deviation.  Considering the difference of operating the environments, results are confirmed.

# Introduction

This contribution is a report of verification results of JCTVC- I0287 and JCTVC- I0289, which is a contribution to Non-CE2. Verification is performed under the CE2 test conditions.  In addition, source code is reviewed. JCTVC-I0287 proposes reduced coefficient precision for ALF and JCTVC-I0289 propose cross 7x5 shape for ALF.

# Experimental Results

Our encoding environment is using 64bit linux cluster and decoding was done on sinlge 64 bit windows PC. There was no mismatch between encoder and decoder. There is no mismatch of overall BD-rate between the verification results and the proponent’s results.

| All Intra HE10 |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.2% | -0.6% | -0.5% |
| 0.3% | -0.6% | -0.6% |
| 0.3% | -0.6% | -0.9% |
| 0.1% | -0.6% | -0.7% |
| 0.9% | -1.7% | -1.4% |
| 0.3% | -0.7% | -0.8% |
| 0.3% | -0.7% | -0.8% |
| 0.1% | -0.6% | -0.7% |
| 98% |  |  |
| 98% |  |  |
| Random Access HE10 |  |  |
| Y | U | V |
| 0.5% | 0.6% | 0.7% |
| 0.8% | 0.7% | 0.5% |
| 0.3% | 0.2% | 0.1% |
| 0.3% | -0.1% | 0.0% |
| 0.5% | 0.4% | 0.3% |
| 0.5% | 0.4% | 0.3% |
| 0.0% | 0.1% | 0.1% |
| 98% |  |  |
| 98% |  |  |
| Low delay B HE10 |  |  |
| Y | U | V |
| 0.5% | -0.9% | -1.0% |
| 0.2% | -0.4% | -0.6% |
| 0.1% | -0.5% | -0.5% |
| 0.8% | -1.6% | -1.5% |
| 0.4% | -0.8% | -0.9% |
| 0.4% | -0.7% | -0.9% |
| 0.0% | -0.8% | -0.2% |
| 97% |  |  |
| 97% |  |  |
| Low delay P HE10 |  |  |
| Y | U | V |
| 1.0% | -1.0% | -1.4% |
| 0.3% | -0.3% | -0.5% |
| 0.0% | -0.3% | -1.0% |
| 1.7% | -1.2% | -1.3% |
| 0.7% | -0.7% | -1.1% |
| 0.7% | -0.7% | -1.0% |
| 0.1% | 0.8% | -0.3% |
| 97% |  |  |
| 97% |  |  |

Table 1. Cross 7x5 ALF shape (JCTVC-I0289)

| All Intra HE10 |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.0% | 0.3% | 0.1% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.1% | 0.2% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.5% | 0.2% |
| 0.0% | 0.2% | 0.1% |
| 0.0% | 0.2% | 0.1% |
| 0.0% | 0.1% | 0.1% |
| 98% |  |  |
| 98% |  |  |
| Random Access HE10 |  |  |
| Y | U | V |
| 0.0% | 0.2% | 0.1% |
| 0.0% | 0.2% | 0.1% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.1% | 0.1% |
| 0.0% | 0.1% | 0.1% |
| 0.0% | 0.1% | 0.1% |
| 98% |  |  |
| 98% |  |  |
| Low delay B HE10 |  |  |
| Y | U | V |
| 0.0% | 0.2% | 0.2% |
| 0.0% | 0.3% | 0.0% |
| 0.0% | 0.0% | 0.0% |
| 0.0% | 0.4% | 0.1% |
| 0.0% | 0.2% | 0.1% |
| 0.0% | 0.2% | 0.1% |
| 0.0% | -0.2% | 0.8% |
| 97% |  |  |
| 97% |  |  |
| Low delay P HE10 |  |  |
| Y | U | V |
| 0.0% | 0.2% | 0.1% |
| 0.0% | 0.1% | 0.1% |
| 0.0% | -0.1% | -0.2% |
| 0.2% | 1.2% | 0.6% |
| 0.0% | 0.3% | 0.1% |
| 0.0% | 0.3% | 0.1% |
| 0.0% | 0.8% | 0.3% |
| 97% |  |  |
| 97% |  |  |

Table 2. Reduced coefficient precision of ALF (JCTVC-I0287)

# Source code review

The code follows the description of Non-CE2 proposal