| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0426_r2 |
| --- | --- |

| Title: | QP determination by lambda value |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Dong Zhang Houqiang Li Jizheng Xu |  |  |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution presents the QP determination algorithm in HEVC. Usually, to improve the coding efficiency multiple-QP optimization (including slice level, LCU level, and sub-LCU level) can be applied in HEVC. In the multiple-QP optimization, lambda value is fixed and the QP value, which provides the smallest RD cost, is selected as the best QP. But this multiple-QP optimization increases the encoding complexity significantly. If 3 QPs are used in the RDO process, roughly 300% encoding time is required. In the document, QP is determined by the lambda value, according to the pre-trained QP-lambda relationship. When applying the proposed method to HM-6.0, about 1.8%, 1.7%, 1.4%, and 1.4% luma bits saving is obtained for RA-Main, RA-HE10, LD-Main, and LD-HE10 respectively. The bits saving on U and V are reported somewhat larger than that on Y. The method in this contribution only includes non-normative modifications.

# Introduction

We retrain the QP-lambda relationship in HEVC, similar as what was done in [1], and then apply the re-trained QP-lambda relationship to the HEVC encoding as an alternative, complexity increasing free method to determine the optimal QP value. When applying the proposed method to HM-6.0, about 1.8%, 1.7%, 1.4%, and 1.4% luma bits saving is obtained for RA-Main, RA-HE10, LD-Main, and LD-HE10 respectively. The retrained QP-lambda relationship can also be applied together with the adaptive QP determination method proposed in [2]. If both methods are enabled, about 2.2%, 2.0%, 2.3%, and 2.2% luma bits saving is obtained for RA-Main, RA-HE10, LD-Main, and LD-HE10 respectively.

We do not use the new QP-lambda relationship to determine lambda according to QP, as the lambda value is not only determined by the QP value but also determined by the coding structure. Instead, we recalculate the QP value after determining the lambda value according to the normal lambda calculation process.

We will illustrate the experiments of re-training the QP-lambda relationship in Section 2, and provide the experimental results when the QP-lambda relationship to HEVC in Section 3.

# Experiments to determine QP-lambda relationship

As an important part of encoding, RDO process has great impact on the coding efficiency. And Lagrange multiplier, λ, also plays an important role on the performance. For practical use, λis related to QP and the coding structure. The optimization target of video coding is usually

minJ=D+λR	(1)

where D is the distortion, R is the bits cost, and λ is the Lagrange multiplier, J is usually called RD cost.

When multiple-QP optimization is applied, the optimization target is

QPopt=argminD(QPi)+λR(QPi),QPi∈QPs	(2)

where the optimal QP is the QP which provides the smallest RD cost. The multiple-QP optimization can improve the coding efficiency as well as increase the encoding time significantly.

We retrain the QP-lambda relationship similar as what was done in [1]. We use fix lambda encoding, and use multiple-QP optimization to determine the best QP for each LCU. The optimization target of determining the optimal QP for each LCU is the same as (2). To obtaining the QP-lambda relationship in HEVC, we perform experiments on RA and LD coding structures (All-Intra case is not tested, as All-Intra is not so widely applied in common use compared with the other two.) To know whether QP-lambda relationship is independent with coding structure, we also apply another simple coding structure, which is IBBB low delay coding without hierarchical QP setting and only one reference picture (this coding structure will be referred as IBBB (Flat QP) in the rest of this document). All sequences in Class C and Class D are used in the experiments. The initial QPs (QPi, QP value for the intra picture) we used for this investigation are 22, 25, 28, 31, 34, 37, and 40. Then the QP for each picture (QP0) is determined according to the coding structure and the λ is calculated by the original HM-6.0 algorithm. For RA, total 5 λs will be used for each input QP values, i.e. λ for I picture plus 4 hierarchical levels. For LD, total 4 λs will be used, i.e. I pictures plus 3 hierarchical levels. For IBBB (Flat QP), total 2 λs will be used, i.e. I picture and B picture. LCU based multiple-QP optimization is performed to obtain the QP-lambda relationship. To guarantee every QP has the same opportunity to be selected as the best QP, the bits to represent delta QP information are not considered in the RDO process. We fully search 13 QPs around (QP0); usually the search range is [QP0− 6, QP0+6]. As the λ value for the highest hierarchical level, i.e. level 3, of RA coding structure is relatively large, the search range for those pictures are [QP0−4, QP0+8]. For instance, the λs used in the tests are shown in Table 1.

Table 1 λ used for each QPi and hierarchical level

|  | Lambda values |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |  | Low Delay |  |  |  | IBBB (Flat QP) |  |
| QPi | I | L0 | L1 | L2 | L3 | I | L0 | L1 | L2 | I | B |
| 22 | 3.7 | 5.6 | 11.3 | 15.4 | 40.3 | 4.9 | 7.3 | 14.8 | 20.2 | 5.7 | 5.9 |
| 25 | 7.5 | 11.2 | 28.3 | 38.0 | 97.9 | 9.8 | 14.7 | 37.0 | 49.7 | 11.5 | 11.7 |
| 28 | 14.9 | 22.5 | 67.9 | 90.3 | 230.3 | 19.5 | 29.4 | 88.8 | 118.1 | 23.0 | 23.5 |
| 31 | 29.9 | 44.9 | 158.4 | 209.1 | 529.6 | 39.1 | 58.7 | 207.2 | 273.4 | 46.0 | 47.0 |
| 34 | 59.8 | 89.8 | 362.1 | 456.2 | 1105.3 | 78.1 | 117.4 | 473.5 | 596.6 | 91.9 | 94.0 |
| 37 | 119.5 | 179.6 | 724.2 | 912.4 | 2210.7 | 156.3 | 234.9 | 947.0 | 1193.1 | 183.8 | 187.9 |
| 40 | 239.0 | 359.2 | 1448.4 | 1824.8 | 4421.4 | 312.5 | 469.8 | 1894.0 | 2386.3 | 367.7 | 375.8 |

We observe that

- For a given lambda value, only a small range of QPs are used. Some of the relative QP selection occurrence is shown in Fig. 1. As the QP selected is in a narrow range, we use the average QP value for each given λ in the following discussion.

Fig. 1 Relative QP selection occurrence for a given λ

- For each sequence, the QP and ln(λ) are in linear relationship, no matter what the hierarchical coding level is and what the coding structure is. The QP-lambda relationship of BasketballDrill, BQMall, and PartyScene are shown in Fig. 2. There are 77 points in each figure, 7x5 points for RA, 7x4 points for LD, and 7x2 points for IBBB (Flat QP).

Fig. 2 QP-lambda relationship

These points in each figure are roughly in linear relationship. And the QP-lambda relationship could be formulated as

QP=a×ln(λ)+b	(3)

where a is the slope and b is the intercept.

- If we consider all the sequences together, the QP and ln(λ) are also in good linear relationship, as shown in Fig. 3.

Fig. 3 QP-lambda relationship of all sequences

If we take all the (QP, λ) pair into consideration, the QP-lambda relationship in (3) can be written as

QP=4.2005×ln(λ)+13.7122	(4)

# Experimental results when applying QP-lambda relationship

We show the experimental results of the QP-lambda relationship in (4) in this section. We use the default HM-6.0 as anchor [3], including RA-Main, RA-HE10, LD-Main, LD-HE10.

When the new trained QP-lambda relationship is applied, the original QP is determined by the QP of Intra picture and coding structure (the original QP is identical to the QP of HM-6.0 default coding structure), and the lambda is determined by the original lambda calculation process as in HM-6.0, which takes the original QP and the coding structure into consideration. We do not modify the lambda calculation process, but recalculate the QP value according to (4) when the lambda is determined, as shown in Fig. 4.

Fig. 4 Encoding procedure when retrained QP-lambda relationship is applied

When implementing it into HM-6.0, if we do not take the lines of reading parameters from configuration file or command line and the lines of setting parameters into account, the affected lines are about 10.

We perform the following tests

- Retrained QP-lambda relationship.

- LCU level 3 QPs optimization around original QP, as the retrained QP-lambda relationship is used as an alternative and complexity increasing free method of multiple-QP optimization.

- LCU level 5 QPs optimization around original QP.

- LCU level 3 QPs optimization around the QP determined by (4), as from the average result, the QP determined by (4) can be treated as a good start point of multiple-QP optimization.

- Adaptive QP determination proposed in [2], which is an adaptive QP determination method which takes different characteristics of different sequences into consideration.

- Retrained QP-lambda relationship plus adaptive QP determination.

The summary results of Y BD-Rate are shown in the tables below. The bits saving on U and V are reported somewhat larger than that on Y and the detailed results can be found in the spreadsheets attached.

Table 2 Y BD-Rate of retrained QP-lambda relationship

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | −1.6% | −1.2% |  |  |
| Class B | −1.6% | −1.3% | −1.1% | −1.0% |
| Class C | −2.2% | −2.3% | −1.2% | −1.3% |
| Class D | −1.8% | −1.9% | −1.2% | −1.2% |
| Class E |  |  | −2.3% | −2.4% |
| Overall | −1.8% | −1.7% | −1.4% | −1.4% |
| Enc. Time | 100% | 100% | 100% | 100% |
| Dec. Time | 101% | 99% | 101% | 100% |

Table 3 Y BD-Rate of LCU level 3 QPs optimization around original QP

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | −1.1% | −0.8% |  |  |
| Class B | −1.1% | −0.9% | −0.5% | −0.4% |
| Class C | −1.3% | −1.3% | −0.7% | −0.7% |
| Class D | −1.1% | −1.1% | −0.7% | −0.7% |
| Class E |  |  | −1.0% | −1.1% |
| Overall | −1.2% | −1.1% | −0.7% | −0.7% |
| Enc. Time | 299% | 294% | 299% | 297% |
| Dec. Time | 102% | 101% | 102% | 101% |

Table 4 Y BD-Rate of LCU level 5 QPs optimization around original QP

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | −1.6% | −1.1% |  |  |
| Class B | −1.7% | −1.4% | −1.1% | −0.9% |
| Class C | −2.1% | −2.2% | −1.3% | −1.2% |
| Class D | −1.8% | −1.9% | −1.2% | −1.2% |
| Class E |  |  | −1.8% | −1.9% |
| Overall | −1.8% | −1.6% | −1.3% | −1.3% |
| Enc. Time | 497% | 486% | 498% | 494% |
| Dec. Time | 101% | 101% | 101% | 101% |

Table 5 Y BD-Rate of LCU level 3 QPs optimization around QP determined by (4)

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | −2.0% | −1.4% |  |  |
| Class B | −2.2% | −1.8% | −1.7% | −1.4% |
| Class C | −2.8% | −2.8% | −1.8% | −1.8% |
| Class D | −2.3% | −2.3% | −1.6% | −1.7% |
| Class E |  |  | −2.6% | −2.4% |
| Overall | −2.3% | −2.1% | −1.9% | −1.8% |
| Enc. Time | 298% | 292% | 298% | 296% |
| Dec. Time | 101% | 100% | 102% | 101% |

Table 6 Y BD-Rate of adaptive QP determination in [2]

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | −1.4% | −1.3% |  |  |
| Class B | −1.0% | −0.7% | −2.0% | −1.7% |
| Class C | −1.4% | −0.9% | −2.0% | −1.6% |
| Class D | −0.9% | −0.5% | −1.4% | −1.1% |
| Class E |  |  | −1.4% | −1.1% |
| Overall | −1.1% | −0.8% | −1.8% | −1.4% |
| Enc. Time | 99% | 99% | 99% | 99% |
| Dec. Time | 101% | 99% | 101% | 100% |

Table 7 Y BD-Rate of retrained QP-lambda relationship and adaptive QP determination in [2]

|  | RA-Main | RA-HE10 | LD-Main | LD-HE10 |
| --- | --- | --- | --- | --- |
| Class A | −1.6% | −1.3% |  |  |
| Class B | −1.9% | −1.6% | −2.0% | −1.8% |
| Class C | −3.0% | −2.9% | −2.3% | −2.2% |
| Class D | −2.4% | −2.2% | −1.9% | −1.6% |
| Class E |  |  | −3.4% | −3.5% |
| Overall | −2.2% | −2.0% | −2.3% | −2.2% |
| Enc. Time | 99% | 99% | 99% | 99% |
| Dec. Time | 100% | 99% | 101% | 100% |

We could see that, if the QP is recalculated according to the lambda value, bits saving could be achieved without significant impact on the complexity.

The recalculated QP value can also be used as a good start point if multiple-QP optimization is applied. 3 QPs optimization (around the QP determined by (4)) works better than 5 QPs optimization (around original QP).

Recalculating QP and the adaptive QP determination can be used together, and more gain can be achieved than single method is applied.

# More discussion on the results in Table 2

After obtaining the new QP-lambda relationship as in (4), we at least have two ways to apply (4) in the encoding. One method is we recalculate the lambda value according to original QP setting. The other method is we recalculate the QP value according to the original lambda setting.

According to our understanding, lambda is used to express how “expensive” a bit is. Larger lambda means we prefer to use fewer bits even the distortion is larger; and vice versa.

In our practical encoding, lambda is not only determined by the QP value, but also determined by the coding structure. For example, in both HM and JM, when a picture is not used as reference picture, the lambda value will be set relative larger than that of a picture is used as reference picture. This kind of setting makes sense as the reconstruction quality of non-reference picture will not affect the other pictures. Another example is that GOP size also has impact on the lambda calculation. Fig.5 shows an example of hierarchical coding. Picture A and B are the pictures with the lowest “level”. There are some other pictures between picture A and B. And they will use picture A and B as reference pictures. When GOP size is larger, which means there are more pictures between A and B, the lambda value of A and B will be smaller. In this case, smaller lambda leads to picture A and B have relative higher reconstruction quality. And all the pictures between picture A and B can benefit from predicted by high quality reference pictures.

Fig. 5 Hierarchical Coding Structure

Generally speaking, lambda value is mainly determined by the QP value and further refined by the coding structure. And the refinement of lambda value according to the coding structure can also bring some benefit on the coding efficiency as the refinement process takes the referencing structure into consideration.

We think the refinement of lambda value makes sense. Thus, we do not want to change the lambda value, as it is determined by lots of experiences. We want to keep the original lambda setting for each picture but recalculate the QP value.

Another reason that we do want to change the lambda setting is that the bitrate is more sensitive to the lambda value. As we know, both QP value and lambda value have impact on the bitrate. Smaller QP and lambda value lead to high bitrate; and vice versa. But lambda value has much more impact on the bitrate. Both HM and JM encoder can choose to skip the entire residue, if they think costing too many bits is too “expensive” no matter how large the original residue is. When the residue is skipped, the quantization step size could be recognized as infinite. To verify that lambda value has larger impact on the bitrate, we performed the following experiments. The first test is keeping the original QP setting but applying the lambda of the first picture, which is equal to or smaller than the original lambda value of each picture, to all the pictures (referred as original-QP encoding). The second test is keeping the original lambda setting but applying the QP of the first picture, which is also equal to or smaller than the original QP value of each picture, to all the pictures (referred as original-lambda encoding). The bit rate of the anchor and the two alternative methods are provides in the attached spreadsheet. It could be seen from the attached spreadsheet clearly that when changing the QP setting but keeping the original lambda, the bit rate does not change much (about only 3% variation for RA-Main and 3% variation for LB-Main on average). And keeping the original QP setting but changing the lambda setting will lead to significant bitrate changing (about 114% variation for RA-Main and 77% variation for LB-Main on average). From the results we can conclude that

- Lambda value is a more important factor to determine the bitrate, compared to the QP value.

- When keeping the lambda setting unchanged but changing the QP value a little, the bitrate variation is not too much.

According to the discussion above, when applying (4), we do not change the lambda setting but recalculate the QP value according to the original lambda setting, as shown in Fig. 4.

Although the QP value is recalculated, we do not apply adaptive QP encoding, but fix QP encoding, as the lambda value is fixed and (4) and Fig. 4 are also fixed. The QP value is also fixed. The QP value used in the RA and LB cases are shown in Table 8 (original HM-6.0 QP setting), 9 (recalculated QP setting).

Table 8 QP used for each QPi and hierarchical level (original HM-6.0 QP)

|  | QP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |
| QPi | I | L0 | L1 | L2 | L3 | I | L0 | L1 | L2 |
| 22 | 22 | 23 | 24 | 25 | 26 | 22 | 23 | 24 | 25 |
| 27 | 27 | 28 | 29 | 30 | 31 | 27 | 28 | 29 | 30 |
| 32 | 32 | 33 | 34 | 35 | 36 | 32 | 33 | 34 | 35 |
| 37 | 37 | 38 | 39 | 40 | 41 | 37 | 38 | 39 | 40 |

Table 9 QP used for each QPi and hierarchical level (recalculated QP)

|  | QP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |
| QPi | I | L0 | L1 | L2 | L3 | I | L0 | L1 | L2 |
| 22 | 19 | 21 | 24 | 25 | 29 | 20 | 22 | 25 | 26 |
| 27 | 24 | 26 | 30 | 31 | 35 | 25 | 27 | 31 | 33 |
| 32 | 29 | 31 | 36 | 37 | 41 | 30 | 32 | 37 | 38 |
| 37 | 34 | 36 | 41 | 42 | 46 | 33 | 37 | 42 | 43 |

To verify the efficiency of the retrained QP-lambda relationship, we performed the following tests. We compared the RD Cost of encoding one picture with original QP and the RD Cost of encoding one picture with the QP determined by (4). The RD cost is calculated by (1), where lambda is the Lagrange multiplier when encoding the current picture, D is the SSE between the reconstructed picture and the original picture, R is the bits cost. Class C sequences with RA-Main, RA-HE10, LB-Main, and LB-HE10 are tested. We encode one picture twice, the first time with QP determined by (4), and the second time with original QP. The reconstruction of the second time, which is identical to the original HM-6.0 reconstruction, is kept and used as reference picture to the following picture. We calculate the RD Cost ratio of the two different methods by

ratio=RDCostQPby(4)RDCostOriginalQP

As can be seen from the attached spreadsheet, for most of the pictures, the ratio is smaller than 100%, which means the QP determined by (4) is more efficient. As (4) is not sequence or even picture specified, the RD Cost of some pictures may also increase. But there are only about 8.8% pictures with increased RD cost. From the aspect of average RD Cost Ratio ((4) is also derived from average results), the QP determined by (4) is efficient.

Analysis on all intra case results

It can be seen in the attached spreadsheet that for all intra cases, the BD-Rate results are loss. The summary results are shown in the table below.

Table 10 Y BD-Rate of retrained QP-lambda relationship (All Intra cases)

|  | AI-Main | AI-HE10 |
| --- | --- | --- |
| Class A | 0.1% | 0.2% |
| Class B | 0.0% | 0.1% |
| Class C | 0.3% | 0.2% |
| Class D | 0.2% | 0.1% |
| Class E | 0.4% | 0.4% |
| Overall | 0.2% | 0.2% |
| Enc. Time | 102% | 102% |
| Dec. Time | 101% | 100% |

The possible reasons that we do not get performance gain for all intra cases may be that the training data set of new QP-lambda relationship does not include all intra cases.

Similar, we also calculate the RD cost ration for all intra cases. It is very interesting that, for most of the intra pictures, the RD cost ratio is smaller than 100%. And the average RD cost ration is also smaller than 100%. But the BD-Rate results are loss.

The per-level PSNR results are provided in JCTVC-I0426_QPDetermination_RetrainedQP-lambda_r2.xlsm. The PSNR difference is calculated by

PSNR difference = average PSNR of highest level – average PSNR of lowest level.

From that file we can see that for low delay cases, the per-level PSNR results of the proposed QP setting are not so different with original HM-6.0 QP setting. The average PSNR difference of the proposed QP setting is only about 0.1 dB larger than that of original HM-6.0 QP setting. For random access cases, as there are more intra pictures than low delay cases, and both the lambda and QP of intra pictures are relative smaller, the PSNR difference is a little larger when comparing the proposed QP setting with the original HM-6.0 QP setting. But the difference is not too large, only about 0.43~0.46 dB.

For random access cases, it can also be seen from those results that even we use much larger QP for the highest level pictures (which are non-reference pictures), the average PSNR results are not bad for those pictures, actually a little better than the original HM-6.0 QP setting, because of the quality of reference pictures is better.

# References

- G. Sullivan and T. Wiegand, “Rate-distortion optimization for video compression,” Signal Processing Magazine, IEEE, vol. 15, pp. 74–90, Nov. 1998.

- JCTVC-G382, “CE4 Subtest-2 Adaptive Reconstruction Levels,” Geneva, CH, Nov. 2011.

- JCTVC-H1100, “Common test conditions,” San José, CA, USA, Feb. 2012.