| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0427 |
| --- | --- |

| Title: | Category-prefixed data batching for tiles and wavefronts |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sandeep Kanumuri Gary Sullivan Yongjun Wu Ji-Zheng Xu One Microsoft Way Redmond WA 98052-6399 USA | Tel: Email: | +1-425-707-3140 skanumu@microsoft.com garysull@microsoft.com yongjunw@microsoft.com jzxu@microsoft.com |
| Source: | Microsoft Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a modification to the formatting of entropy-coded bitstream data in HEVC for use with the tile and wavefront coding features, as originally proposed in JCTVC-G815. The same concept could also apply to PIPE/V2V/V2F entropy coding or other such schemes that include the need to convey different categories of data. In the current HEVC draft design that uses a single method of entry point signalling for tiles and wavefronts (JCTVC-H0556), an index table is used in the slice header to identify the location of the starting point of the data for each entry point. The use of these indices increases the delay and memory capacity requirements at the encoder (to batch up all of the data before output of the index table and the subsequent sub-streams) and at the decoder (to batch up all of the input data in every prior sub-stream category while waiting for the data to arrive in some other category).

This contribution proposes, rather than using the current index table approach, for the different categories of data to be chopped up into batches, and for each batch to be prefixed with a batch type identifier and a batch size indicator. The different categories of data can then be interleaved with each other in relatively-small batches instead of being buffered up for serialized storage into the bitstream data. Since the encoder can emit these batches of data as they are generated, and parallelized decoders can potentially consume them as they arrive, the delay and buffering requirements are asserted to be reduced. It is also asserted that the decoder can skip scanning for start codes within the batch which reduces complexity. Furthermore, if the decoder is not interested in consuming a particular category of data, it is asserted that the decoder can skip the removal of emulation prevention bytes in data corresponding to that category. The average BD bit rate impact, comparing the proposal to HM 6.1 as the reference, is asserted to be 0.0% for a representative All-Intra configuration, 0.1% for a representative Random Access configuration and 0.2% for a representative Low Delay configuration.

The contribution also reports a bug in HM 6.1.

# Category-prefixed data batching

The concept of category-prefixed data batching was originally proposed in JCTVC-G815 [1]. Rather than using the current index table approach to signal entry points for different categories of data, we propose for the data in each category to be chopped up into batches, and for each batch to be prefixed with a batch type identifier and a batch size indicator. The different categories of data can then be interleaved with each other instead of being placed sequentially into the bitstream data.

We begin by introducing the concept (previously discussed in JCTVC-G815) of a category-prefixed data batch (CPDB) that consists of a batch prefix (BP) followed by the payload data for the batch.

An example of a data stream that uses category-prefixed data batching is illustrated below:

| BP | payload data | BP | payload data | BP | payload data | … |
| --- | --- | --- | --- | --- | --- | --- |

The batch prefix contains a category identifier (CI) and a data quantity indicator (DQI).

The CI identifies the category of data and the DQI indicates the amount of data in the current batch. (The next batch of data can then be found by adding the indicated data quantity to the current location in the bitstream.)

# Special batch sizes (SBS) and Custom batch sizes (CBS)

We propose for the CPDB scheme to make use of particular "special batch sizes" (SBS), coupled with "custom batch sizes" (CBS). The batch prefix (BP) would be only one byte long when using an SBS and would be two bytes long when using a CBS.

The first byte of the BP is composed of an "abbreviated DQI" (3 bits) and the CI (5 bits). The value of the abbreviated DQI indicates whether the CPDB uses an SBS or CBS, and when the CPDB uses an SBS, the abbreviated DQI also specifies the value of the SBS as shown in the table below. The table indicates the information derived from an abbreviated DQI:

| Abbreviated DQI (bit pattern) | Abbreviated DQI (value) | SBS or CBS? | Batch size in bytes |
| --- | --- | --- | --- |
| '001' | 1 | SBS | 8192 |
| '010' | 2 | SBS | 4096 |
| '011' | 3 | SBS | 2048 |
| '100' | 4 | SBS | 1024 |
| '101' | 5 | SBS | 512 |
| '110' | 6 | SBS | 256 |
| '111' | 7 | CBS | DQI extension value |

The second byte of the BP is present only when the abbreviated DQI indicates the use of a CBS, and is composed of a DQI extension value (8 bits). The value of the DQI extension then directly indicates the batch size (in bytes). The DQI extension, when present, can have a value ranging from 1 to 255, inclusive.

The abbreviated DQI and the DQI extension (when present) together form the complete DQI.

An encoder can emit a CPDB whenever it has data ready to send in a particular category. CPDBs are emitted until the data from all categories has been exhausted for the current slice.

# Mapping from tiles and wavefronts to categories

The CI of length 5 bits can represent up to 32 distinct categories, indexed from 0 to 31. The number of categories used determines the degree of parallelism at which at the decoder can operate.

Tiles can be used not just for enabling decoder parallelism, but also for enabling encoder parallelism. The desired level of parallelism with the use of tiles can sometimes be higher at the encoder (especially for real-time encoding scenarios since encoders have a lot more computational complexity) than at the decoder. Given this, it is not necessary to use as many categories as the number of tiles. This is already reflected in the current text of the standard, as the encoder is not required to provide an entry point in the slice header for every tile in a slice.

When the total number of tiles in the picture is greater than 32, the tiles are initially grouped into categories to reduce the number of necessary category identifiers to no more than 32, as follows:

- A mapping factor M is calculated as M = Ceil( (number of tiles)  32 ), where Ceil(x) is an operator that rounds the fraction x to the nearest integer greater than or equal to x.

- Tiles from index 0 to (M – 1) are placed into category 0, tiles from index M to 2*M – 1 are placed into category 1, tiles from index 2*M to 3*M – 1 are placed into category 2, and so on.

Furthermore, when tiles are used, consecutive categories can be combined for use as a single category when desired for BP overhead reduction. This usage is directly analogous to the allowance in the current text for the encoder to send entry points for some but not all tiles.

In such a case, the smallest category among the categories that are combined together is used to identify the combined category, and will be used in the determination of CI. An example of this approach is as follows: categories 0, 1, 2 can be combined such that the data in these categories is sequentially concatenated and packaged into CPDBs with CI equal to 0; similarly, categories 3, 4, 5, 6, 7, 8 can be combined, and data from these categories can be packaged into CPDBs with CI equal to 3; and all remaining categories (categories 9 to 31) can be combined and data from these categories can be packaged into CPDBs with CI equal to 9.

When more than one tile is present in a category (or in a combined category), the CPDBs for that category are arranged in the bitstream for the current slice such that all payload data for a tile with index Ti appears before any data for a tile with index Tj where Tj > Ti. This is the same concept as the current concatenation of such tile data sub-streams when fewer entry points are coded in the header than the total number of tiles.

For wavefronts, since each wavefront substream needs to be identified separately, and a one-to-one mapping from wavefront substream index to category index is used. This implies that the maximum number of wavefronts supported within a slice is 32.

# Start code emulation prevention

The BP is formatted such that it always avoids the need for start code emulation prevention byte insertion (e.g., as currently specified for AVC). The abbreviated DQI is not allowed to use the bit pattern '000' which means that the first byte of BP is always greater than or equal to 0x20. (Note that any byte value greater than 0x03 will avoid insertion of a start code emulation prevention byte at the current position in a NAL unit.) Moreover, the second byte of the BP (when present) representing the DQI extension is never allowed to be equal to 0. The DQI extension is used for payload sizes ranging from 1 to 255.

Furthermore, we propose for the size represented using an SBS or CBS to actually inform the decoder of the size of the payload data after the payload data has gone through start code emulation prevention. It is useful to represent the size in this manner, as this helps the decoder to skip through the data in a batch (if it chooses to do so) by simply moving forward by the number of bytes indicated by size of the payload without needing to scan through the payload data for emulation prevention bytes. In addition, the decoder can avoid scanning for start codes in the payload data since it is guaranteed that start codes do not occur in the payload data. This reduces the amount of processing needed when performing such tasks as storing the video data into a box-format file, or passing the data directly to a hardware accelerator, as it allows direct copying of data chunks without the need for scanning through individual bytes of the data.

In a rare event, it is possible that when constructing a CPDB of a certain size CUR_SIZE, the payload data after emulation prevention is such that the bytes at location CUR_SIZE-2 and CUR_SIZE-1 are both equal to 0. In such a case, there is only one byte of payload left to be filled in the payload data and if the next byte from that category happens to trigger an emulation prevention condition, we will end up wanting to add two bytes – including the emulation prevention byte. If only one of these bytes is sent in the payload, the emulation prevention byte would actually be followed in the bitstream by a different byte value than the one that triggered the insertion of the emulation prevention byte. To avoid this situation, whenever we encounter a situation where the bytes at location CUR_SIZE-2 and CUR_SIZE-1 are both equal to 0, we add a filler byte equal to 0x04 (any value above 0x03 should work fine as well). The same condition is detected at the decoder and the filler byte is discarded. Any remaining bytes of actual payload data would then be sent in a subsequent batch (with the same category identifier).

# Bug in HM-6.1

There appears to be a bug in HM-6.1 software whereby an additional unnecessary byte (RBSP stop bit and trailing zero bits) is written to the bitstream after all tiles or wavefront substreams are added to the bitstream. Since both tiles and wavefront substreams are already byte-aligned after introducing RBSP stop bits and trailing zero bits between them as necessary, this is unnecessary and causes the decoder to append this extra byte at the end of the last tile or wavefront substream. It will then eventually be discarded by the decoder. This bug has been reported to the HEVC issue tracker and has been assigned the ticket number 490.

# Removal of tile markers

In the current HEVC draft design, there are two methods to enable decoder-parallelism for tiles. One is through the use of entry point offsets in the slice header. The other method is through the use of special tile markers that are not repeatable anywhere else in the bitstream. According to JCTVC-F594, the purpose of using such tile markers is to allow low latency operation at the encoder since the signalling of entry point offsets requires the bitstream to be buffered until the offsets are determined.

The design of CPDB eliminates the need for having two methods, as it offers low latency operation at both the encoder and decoder.

The encoder latency offered by CPDB is even lower than that of tile markers; when using CPDB, as all parallel encoders can start emitting data (in the form of CPDBs) of any size at any time; however, with the use of tile markers, data from an earlier tile needs to be completely emitted before data from a later tile can be emitted. CPDB also offers low latency at the decoder which is not possible with both methods available in the current HEVC draft design.

When using CPDB, all parallel decoders can start operating at roughly the same time (since data from different categories can be interleaved in the bitstream) as opposed to the current situation where some parallel decoders must start later (since data from different categories is sequentially ordered), depending on the speed at which the data arrives.

# Experimental Results

We implemented our proposal on top of the HM 6.1 software. We present results comparing our proposal to HM 6.1 software using tiles and wavefronts.

For tiles, the following encoder options are used when enabling 4 tiles (for both the anchor and the proposal) in addition to common conditions:

- TileMarkerFlag = 0

- UniformSpacingIdc = 1

- NumTileColumnsMinus1 = 1

- NumTileRowsMinus1 = 1

- TileLocationInSliceHeaderFlag = 1

The table below shows the averaged BD bit rate impact over all classes (including class F).

|  | Y | U | V |
| --- | --- | --- | --- |
| Intra-Main | 0.0% | 0.0% | 0.0% |
| Intra-HE10 | 0.0% | 0.0% | 0.0% |
| RA-Main | 0.1% | 0.1% | 0.1% |
| RA-HE10 | 0.1% | 0.1% | 0.1% |
| LB-Main | 0.3% | 0.2% | 0.2% |
| LB-HE10 | 0.2% | 0.2% | 0.2% |
| LP-Main | 0.2% | 0.2% | 0.2% |
| LP-HE10 | 0.2% | 0.2% | 0.2% |

Comparison between anchor and proposal for 4 tiles

For wavefronts, the following encoder options are used when enabling 4 wavefronts (for both the anchor and the proposal) in addition to common conditions:

- WaveFrontSynchro=1

- WaveFrontFlush=1

- WaveFrontSubstreams=4

The table below shows the averaged BD bit rate impact over all classes (including class F).

|  | Y | U | V |
| --- | --- | --- | --- |
| Intra-Main | 0.0% | 0.0% | 0.0% |
| Intra-HE10 | 0.0% | 0.0% | 0.0% |
| RA-Main | 0.2% | 0.1% | 0.1% |
| RA-HE10 | 0.1% | 0.1% | 0.1% |
| LB-Main | 0.2% | 0.2% | 0.2% |
| LB-HE10 | 0.2% | 0.2% | 0.2% |
| LP-Main | 0.2% | 0.2% | 0.2% |
| LP-HE10 | 0.2% | 0.2% | 0.2% |

Comparison between anchor and proposal for 4 wavefronts

The Excel sheets representing the results presented in this section are attached along with this document.

# Conclusion

CPDB offers the following benefits in exchange for a tiny overhead:

- Latency reduction at both encoder and decoder compared to existing methods

- Very low buffering requirements at both encoder and decoder

- Decoder parallelism improvement since all parallel decoders can start at the same time

- Start code scanning can be skipped on payload data of CPDB

- Removal of emulation prevention bytes can be skipped for CPDBs not consumed by the decoder

- Removal of the need for multiple methods targeting decoder-level parallelism for tiles

We propose for the CPDB scheme to be adopted into the HEVC draft design in place of the current two methods – namely entry point offsets and tile markers.

The necessary syntax modifications are made on top of the latest WD text and attached along with this document.

# References

- Gary J. Sullivan, “Category-prefixed data batching for HEVC wavefront and PIPE/V2V/V2F coding”, JCTVC-G815, 7th meeting, Geneva, November 2011.

- Hendry et. al, “AHG4: Harmonized Method for Signalling Entry Points of tiles and WPP Substreams”, JCTVC-H0556, 8th meeting, San Jose, February 2012.

- Kiran Misra and Andrew Segall, “New results for parallel decoding for Tiles”, JCTVC-F594, 6th meeting, Torino, July 2011.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).