| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0428r1 |
| --- | --- |

| Title: | Fast forward and inverse DST |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jian Lou Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

In the current HEVC, 4x4 discrete sine transform (DST) is adopted for some Intra prediction modes. This document proposes fast algorithm for 4x4 forward and inverse DST in HEVC.

# Introduction

Discrete Sine Transform (DST) is adopted in HEVC for some Intra prediction modes [1]. In HEVC, the integer 4x4 DST matrix is

DSTHEVC4×4=2955748474740−7484−29−745555−8474−29

The fast forward 4x4 DST in HEVC is implemented as follows

void fastForwardDst(short block[4][4],short coeff[4][4],int shift)  // input block, output coeff

{

int i, c[4];

int rnd_factor = 1<<(shift-1);

for (i=0; i<4; i++)

{

// Intermediate Variables

c[0] = block[i][0] + block[i][3];

c[1] = block[i][1] + block[i][3];

c[2] = block[i][0] - block[i][1];

c[3] = 74* block[i][2];

coeff[0][i] =  ( 29 * c[0] + 55 * c[1]         + c[3]               + rnd_factor ) >> shift;

coeff[1][i] =  ( 74 * (block[i][0]+ block[i][1] - block[i][3])      + rnd_factor ) >> shift;

coeff[2][i] =  ( 29 * c[2] + 55 * c[0]         - c[3]               + rnd_factor ) >> shift;

coeff[3][i] =  ( 55 * c[2] - 29 * c[1]         + c[3]               + rnd_factor ) >> shift;

}

}

The fast inverse 4x4 DST in HEVC is implemented as follows

void fastInverseDst(short tmp[4][4],short block[4][4],int shift)  // input tmp, output block

{

int i, c[4];

int rnd_factor = 1<<(shift-1);

for (i=0; i<4; i++)

{

// Intermediate Variables

c[0] = tmp[0][i] + tmp[2][i];

c[1] = tmp[2][i] + tmp[3][i];

c[2] = tmp[0][i] - tmp[3][i];

c[3] = 74* tmp[1][i];

block[i][0] = Clip3( -32768, 32767, ( 29 * c[0] + 55 * c[1]     + c[3]               + rnd_factor ) >> shift );

block[i][1] = Clip3( -32768, 32767, ( 55 * c[2] - 29 * c[1]     + c[3]               + rnd_factor ) >> shift );

block[i][2] = Clip3( -32768, 32767, ( 74 * (tmp[0][i] - tmp[2][i]  + tmp[3][i])      + rnd_factor ) >> shift );

block[i][3] = Clip3( -32768, 32767, ( 55 * c[0] + 29 * c[2]     - c[3]               + rnd_factor ) >> shift );

}

}

The inverse DST description in HEVC draft is

A set of intermediate values c0, c1, c2 and c3 is calculated as follows:

c0 = x0 + x2											(8-276)
c1 = x2 + x3											(8-277)
c2 = x0 – x3											(8-278)
c3 = 74 * x1											(8-279)

The output values yi with i = 0..3 are then specified as follows:

y0 = 29*c0 + 55*c1 + c3									(8-280)
y1 = 55*c2 − 29*c1 + c3									(8-281)
y2 = 74*( x0 - x2 + x3 )										(8-282)
y3 = 55*c0 + 29 * c2 – c3									(8-283)

The fast forward 4x4 DST needs 8 multiplications and 11 additions for one 4x1 or 1x4 forward transform. The fast inverse 4x4 DST needs 8 multiplications and 11 additions for one 4x1 or 1x4 inverse transform.

In [2], some other fast forward and inverse DST implementations have been proposed. It was reported that both fast forward 4x4 DST and fast inverse 4x4 DST need 6 multiplications and 10 additions, or 4 multiplications, 2 shift and 13 additions. However, 3-stage decomposition introduces more delay.

In [3], new DS coefficients were proposed to reduce the computational complexity.  The DST matrice is changed to

DSTHEVC4×4New=2856748474740−7484−28−745656−8474−28

In [3], it was reported that both fast forward 4x4 DST and fast inverse 4x4 DST need 5 multiplications, 3 shift and 11 additions. In [2], it was reported that both fast forward 4x4 DST and fast inverse 4x4 DST need 4 multiplications, 2 shift and 13 additions, or 5 multiplications, 1 shift and 10 additions, or 4 multiplications, 2 shift and 11 additions with 3-stage decomposition.

# Proposed fast forward and reverse 4x4 DST in HEVC

The floating point 4x4 DST matrix is

DSTfloat=23sin1∗1∗π923sin1∗2∗π923sin1∗3∗π923sin1∗4∗π923sin3∗1∗π923sin3∗2∗π923sin3∗3∗π923sin3∗4∗π923sin5∗1∗π923sin5∗2∗π923sin5∗3∗π923sin5∗4∗π923sin7∗1∗π923sin7∗2∗π923sin7∗3∗π923sin7∗4∗π9

And the integer DST matrix is derived by left shift 7 bits and then rounded to the nearest integers.

Notice that the 4x4 DST matrix could be represented as

M=ABCDCC0−CD−A−CBB−DC−A where A+B=D.

With the matrix symmetry and characteristics, the matrix multiplication Y = MT*X, where Y = [y0, y1, y2, y3]T and X = [x0, x1, x2, x3]T could be decomposed as

c0 = ( x0 – x3 ) * A										
c1 = ( x0 + x2 ) * B										
c2 = ( x2 + x3 ) * D										
c3 = x1 * C

y0 = c0 + c2 + c3										
y1 = c1 – c2 + c3										
y2 = ( x0 - x2 + x3 ) * C									
y3 = c0 + c1 – c3

The matrix multiplication Y = M*X, where Y = [y0, y1, y2, y3]T and X = [x0, x1, x2, x3]T could be decomposed as

c0 = ( x0 + x3 ) * A										
c1 = ( x1 + x3 ) * B										
c2 = ( x0 – x1 ) * D										
c3 = x1 * C

y0 = c0 + c1 + c3										
y1 = ( x0 + x1 – x3 ) * C										
y2 = c1 + c2 – c3									
y3 = -c0 + c2 + c3

The proposed fast forward 4x4 DST in HEVC is implemented as follows

void fastForwardDst(short block[4][4],short coeff[4][4],int shift)  // input block, output coeff

{

int i, c[4];

int rnd_factor = 1<<(shift-1);

for (i=0; i<4; i++)

{

// Intermediate Variables

c[0] = (block[i][0] + block[i][3]) * 29;

c[1] = (block[i][1] + block[i][3]) * 55;

c[2] = (block[i][0] - block[i][1]) * 84;

c[3] =  block[i][2]                * 74;

coeff[0][i] =  ( c[0] + c[1] + c[3]                                        + rnd_factor ) >> shift;

coeff[2][i] =  ( c[2] + c[1] - c[3]                                        + rnd_factor ) >> shift;

coeff[3][i] =  ( c[2] - c[0] + c[3]                                        + rnd_factor ) >> shift;

coeff[1][i] =  ((block[i][0] + block[i][1] - block[i][3]) * 74 + rnd_factor ) >> shift;

}

}

The proposed fast inverse 4x4 DST in HEVC is implemented as follows

void fastInverseDst(short tmp[4][4],short block[4][4],int shift)  // input tmp, output block

{

int i, c[4];

int rnd_factor = 1<<(shift-1);

for (i=0; i<4; i++)

{

// Intermediate Variables

c[0] = (tmp[0][i] - tmp[3][i]) * 29;

c[1] = (tmp[0][i] + tmp[2][i]) * 55;

c[2] = (tmp[2][i] + tmp[3][i]) * 84;

c[3] =  tmp[1][i]              * 74;

block[i][0] =  ( c[0] + c[2] + c[3]                                  + rnd_factor ) >> shift;

block[i][1] =  ( c[1] - c[2] + c[3]                                  + rnd_factor ) >> shift;

block[i][3] =  ( c[0] + c[1] - c[3]                                  + rnd_factor ) >> shift;

block[i][2] =  ((tmp[0][i] - tmp[2][i] + tmp[3][i]) * 74 + rnd_factor ) >> shift;

}

}

The proposed inverse DST description in HEVC draft is

A set of intermediate values c0, c1, c2 and c3 is calculated as follows:

c0 = ( x0 – x3 ) * 29										(8-276)
c1 = ( x0 + x2 ) * 55										(8-277)
c2 = ( x2 + x3 ) * 84										(8-278)
c3 = x1 * 74											(8-279)

The output values yi with i = 0..3 are then specified as follows:

y0 = c0 + c2 + c3										(8-280)
y1 = c1 – c2 + c3										(8-281)
y2 = ( x0 - x2 + x3 ) * 74									(8-282)
y3 = c0 + c1 – c3										(8-283)

The proposed fast forward 4x4 DST needs 5 multiplications and 11 additions for one 4x1 or 1x4 forward transform. The fast inverse 4x4 DST needs 5 multiplications and 11 additions for one 4x1 or 1x4 inverse transform.

# Comparisons between various fast algorithms

| Variant | Method | Multiplication | Shift | Addition | Operation Sum | Stages |
| --- | --- | --- | --- | --- | --- | --- |
| [29,55] | HM | 8 | 0 | 11 | 19 | 2 |
|  | B024 | 8 | 0 | 10 | 18 | 2 |
|  | F153-1 | 6 | 0 | 10 | 16 | 3 |
|  | F153-2 | 4 | 2 | 13 | 19 | 3 |
|  | Proposed | 5 | 0 | 11 | 16 | 2 |
| [28,56] | F283 | 5 | 3 | 11 | 19 | 2 |
|  | F153-1 | 5 | 1 | 10 | 16 | 3 |
|  | F153-2 | 4 | 2 | 11 | 17 | 3 |
|  | Proposed | 5 | 0 | 11 | 16 | 2 |

# References

[1] A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction,” JCT-VC document, JCTVC-E125, March, 2011.

[2] C. Yeo, Y.H. Tan, Z. Li, “On fast implementation of 4-point ODST-3 in HM3,” JCT-VC document, JCTVC-F153, July 2011

[3] A. Saxena and F.C. Fernandes, “CE 7: On fast implementation of 4-point DST Type-7 with 5 multiplications,” JCT-VC document, JCTVC-F283, July 2011

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).