| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0436 WG11 Number: m24688 |
| --- | --- |

| Title: | Modified derivation process on motion vector predictor and weighted prediction for HEVC multi-view extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, it was proposed to modify the derivation process on motion vector predictor (MVP) and weighted prediction as hooks for future HEVC multi-view extension. With high-level modification on top of the HEVC CD specification, denominator or numerator on scaling calculation of MVP could become zero for inter-view prediction. In order to solve this issue, additional conditions were proposed to take care of the case that either denominator or numerator becomes zero. Experimental result reportedly showed no loss for all test conditions relative to HM6.0 with implicit weighted prediction since the denominator and the numerator will never become zero for single-view video coding.

# Introduction

In multi-view video coding, inter-view prediction is helpful to achieve higher coding efficiency (Figure 1). However, if inter-view prediction were introduced on top of the HEVC CD specification only with high-level modifications, scaling calculations for MVP cannot work properly since denominator and numerator, which are derived from POC difference, could become zero. This contribution proposes to modify them without affecting the coding efficiency of single-view video coding.

Figure 1: example of inter-view prediction

# Proposed method

In this contribution, Modified derivation process on motion vector predictor and weighted prediction is proposed as hooks for future HEVC multi-view extension. This modification includes the following three parts:

- Modified derivation process on spatial motion vector predictor

- Modified derivation process on temporal motion vector predictor

- Modified derivation process on weighted prediction parameters

## Modified derivation process on spatial motion vector predictor

In current CD text, a spatial motion vector predictor is calculated by using the scaling calculation with POC differences between current picture, current reference picture and reference picture of neighbor PU, if the current reference picture would not be equal to the one of neighbor PU. This scaling calculation is conducted with below expressions from (8-136) to (8-140). In single-view video coding, the variable td, which is calculated by expression (8-139), could not become zero because the POC of the current picture (PicOrderCntVal) could not be equal to the POC of the reference picture of neighbor PU (PicOrderCnt( RefPicListListA( refIdxA ) )). Likewise, the variable tb, which is calculated by expression (8-140), could not become zero because the POC of the current picture (PicOrderCntVal) could not be equal to the POC of the current reference picture (PicOrderCnt( RefPicListLX( refIdxLX ) )).

----- 8.5.2.1.6 Derivation process for motion vector predictor candidates -----

- When availableFlagLXA is equal to 1, mvLXA is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td								(8-1)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )					(8-2)

mvLXA = Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvLXA ) * 			
		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )					(8-3)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListA( refIdxA ) ) )		(8-4)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )			(8-5)

-------------------------------------------------------------------------------------------

Considering HEVC multi-view extension, the variable td and tb could become zero in the case of inter-view prediction, because the POC of the current picture could be equal to the POC of the current reference picture or the reference picture of neighbor PU. For example, if inter-view prediction were used in neighbor PU and inter prediction were used in current PU, the scaling calculation would be conducted in the current HEVC specification, because the current reference picture is not equal to the one of neighbor PU. However, in this case, td could become zero because inter-view prediction is used in neighbor PU. Therefore, the scaling calculation could not work properly due to division by zero.

To avoid such kind of situations in HEVC multi-view extension, it is proposed to add below sentences (highlighted ones) before the scaling calculation on spatial motion vector predictor. It would not affect the coding efficiency of single-view video coding since td and tb will never become zero for single-view video coding.

----- 8.5.2.1.6 Derivation process for motion vector predictor candidates (Modified) -----

- If  PicOrderCntVal – PicOrderCnt( RefPicListListA( refIdxA ) ) is equal to 0, or PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) is equal to 0, availableFlagLXA is set equal to 0.

- When availableFlagLXA is equal to 1, mvLXA is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td								(8-6)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )					(8-7)

mvLXA = Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvLXA ) * 			
		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )					(8-8)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListA( refIdxA ) ) )		(8-9)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )			(8-10)

……..

- If  PicOrderCntVal – PicOrderCnt( RefPicListListB( refIdxB ) ) is equal to 0, or PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) is equal to 0, availableFlagLXB is set equal to 0.

- When availableFlagLXB is equal to 1 and PicOrderCnt( RefPicListListB( refIdxB ) ) is not equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), mvLXB is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td								(8-11)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )					(8-12)

mvLXB =Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvLXA ) * 
		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )						(8-13)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListB( refIdxB ) ) )			(8-14)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )			(8-15)

-------------------------------------------------------------------------------------------

## Modified derivation process on temporal motion vector predictor

In current CD text, a temporal motion vector predictor is also calculated by using the scaling calculation with POC differences between current picture, current reference picture, colPic and reference picture of colPic. This scaling calculation is conducted such as below expressions from (8-153) to (8-158). In single-view video coding, the variable td, which is calculated by expression (8-157), could not become zero because the POC of the colPic (PicOrderCnt( colPic )) could not be equal to the POC of the reference picture of colPic (RefPicOrderCnt( colPic, refIdxCol, listCol ) )). Likewise, the variable tb, which is calculated by expression (8-158), could not become zero because the POC of the current picture (PicOrderCntVal) could not be equal to the POC of the current reference picture (PicOrderCnt( RefPicListLX( refIdxLX ) )).

----- 8.5.2.1.7 Derivation process for temporal luma motion vector prediction -----

- If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) is equal to PicOrderCntVal - PicOrderCnt( RefPicListLX ( refIdxLX ) ),

mvLXCol = mvCol										(8-16)

- Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + ( Abs( td ) >>1 ) ) / td								(8-17)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )					(8-18)

mvLXCol =  Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvCol ) * 		
		( (Abs( DistScaleFactor * mvCol ) + 127 ) >> 8 ) )						(8-19)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) )		(8-20)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX ( refIdxLX ) ) )			(8-21)

-------------------------------------------------------------------------------------------

Considering HEVC multi-view extension, the variable td could become zero in the case of inter-view prediction, because the POC of the colPic could be equal to the POC of the reference picture of the colPic. Likewise, the variable tb could also become zero, because the POC of the current picture could be equal to the POC of the current reference picture. For example, if inter-view prediction were used in colPu and inter prediction were used in current PU, the scaling calculation would be conducted in the current HEVC specification, because td is not equal to tb. However, in this case, td could become zero because inter-view prediction is used in colPu. Therefore, the scaling calculation could not work properly due to division by zero.

To avoid such kind of  situations in HEVC multi-view extension, it is also proposed to add below sentence (highlighted one) before the scaling calculation on temporal motion vector predictor. It would not affect the coding efficiency of single-view video coding since td and tb will never become zero for single-view video coding.

----- 8.5.2.1.7 Derivation process for temporal luma motion vector prediction (Modified) -----

- If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) is equal to PicOrderCntVal - PicOrderCnt( RefPicListLX ( refIdxLX ) ),

mvLXCol = mvCol										(8-22)

- Otherwise, if PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) is equal to 0, or PicOrderCntVal – PicOrderCnt( RefPicListLX ( refIdxLX ) ) is equal to 0, availableFlagLXCol is set equal to 0.

- Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + ( Abs( td ) >>1 ) ) / td								(8-23)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )					(8-24)

mvLXCol =  Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvCol ) * 		
		( (Abs( DistScaleFactor * mvCol ) + 127 ) >> 8 ) )						(8-25)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) )		(8-26)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX ( refIdxLX ) ) )			(8-27)

-------------------------------------------------------------------------------------------

## Modified derivation process on weighted prediction parameters

In current CD text, implicit weighted prediction parameters are also calculated by using the scaling calculation with POC differences between current picture and current reference picture for L0/L1. This scaling calculation is conducted such as below expressions from (8-224) to (8-231). In single-view video coding, the variable tb, which is calculated by expression (8-224), could not become zero because the POC of the current picture (PicOrderCntVal) could not be equal to the POC of the current reference picture for L0 (PicOrderCnt( RefPicListL0( refIdxL0 ) ) )). Moreover, the variable td, which is calculated by expression (8-225), could not be equal to tb because the POC of the current picture (PicOrderCntVal) could not be equal to the POC of the current reference picture for L1 (PicOrderCnt( RefPicListL1( refIdxL1 ) )).

----- 8.5.2.2.3.2 Weighted sample prediction process -----

The variable WeightScaleFactor is derived from the values currPoc, refIdxL0 and refIdxL1 as follows:

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )			(8-28)

td = Clip3( −128, 127, PicOrderCnt( RefPicListL1( refIdxL1 ) ) 
		 – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )							(8-29)

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td								(8-30)

WeightScaleFactor = Clip3( −1024, 1023, ( tb * tx + 32 ) >> 6 )					(8-31)

The variables w0C and w1C are derived as follows.

–	If PicOrderCnt( RefPicListL0( refIdxL0 ) ) is equal to PicOrderCnt( RefPicListL1( refIdxL1 ) ) or ( WeightScaleFactor >> 2 ) < −64 or ( WeightScaleFactor >> 2 ) > 128, the following applies.

w0C=32											(8-32)

w1C=32											(8-33)

–	Otherwise;

w0C = 64 – (WeightScaleFactor >> 2)								(8-34)

w1C = WeightScaleFactor >> 2									(8-35)

-------------------------------------------------------------------------------------------

Considering HEVC multi-view extension, the variable tb could also become zero in the case of inter-view prediction, because the POC of the current picture could be equal to the POC of the reference picture for L0. Likewise, the variable td could be equal to tb because the POC of the current picture could be equal to the POC of the current reference picture for L1. In these cases, the variables w0C or w1C could be equal to zero. Therefore, implicit weighted prediction for inter-view prediction would not work properly.

To avoid such kind of situations in HEVC multi-view extension, it is also proposed to add below sentence (highlighted one) before creating weighted sample prediction. It would not affect the coding efficiency of single-view video coding since tb will never become zero and td will never become tb for single-view video coding.

----- 8.5.2.2.3.2 Weighted sample prediction process (Modified) -----

The variable WeightScaleFactor is derived from the values currPoc, refIdxL0 and refIdxL1 as follows:

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )			(8-36)

td = Clip3( −128, 127, PicOrderCnt( RefPicListL1( refIdxL1 ) ) 
		 – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )							(8-37)

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td								(8-38)

WeightScaleFactor = Clip3( −1024, 1023, ( tb * tx + 32 ) >> 6 )					(8-39)

The variables w0C and w1C are derived as follows.

–	If PicOrderCnt( RefPicListL0( refIdxL0 ) ) is equal to PicOrderCnt( RefPicListL1( refIdxL1 ) ) or ( WeightScaleFactor >> 2 ) < −64 or ( WeightScaleFactor >> 2 ) > 128 or tb is equal to td or tb is equal to 0, the following applies.

w0C=32											(8-40)

w1C=32											(8-41)

–	Otherwise;

w0C = 64 – (WeightScaleFactor >> 2)								(8-42)

w1C = WeightScaleFactor >> 2									(8-43)

-------------------------------------------------------------------------------------------

# Experimental results

Proposed method was implemented on HM6.0 software and its performance relative to the HM6.0 software was evaluated under the common test condition described in JCTVC-H1100 [1].

Table 1 shows RD performance and execution time of the proposed method relative to HM6.0 with implicit weighted prediction. In this case, proposed method shows no loss on average in all test conditions. The difference in the encoding and decoding time is not obvious. It shows that proposed modifications would not affect the coding efficiency of single-view video coding.

Table 1: experimental result of proposed method relative to HM6.0 with implicit weighted prediction

# Conclusion

In this contribution, it is proposed to modify the derivation process on motion vector predictor (MVP) and weighted prediction as hooks for future HEVC multi-view extension. With high-level modification on top of the HEVC CD specification, denominator or numerator on scaling calculation of MVP could become zero for inter-view prediction. In order to solve this issue, additional conditions are proposed to take care of the case that either denominator or numerator is zero. Experimental result reportedly shows no loss for all test conditions relative to HM6.0 with implicit weighted prediction since the denominator and the numerator will never become zero for single-view video coding.

Inter-view prediction is one of the most important technologies for HEVC multi-view extension to achieve high coding efficiency, but it cannot be applied without low-level modifications. As our proposal can solve the issue without any impact on the coding efficiency of HEVC single-view video coding, we suggest the proposed method be adopted in the DIS of HEVC.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100, San Jose, February 2012.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Proposed changes on Committee Draft

##### Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A, or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-2 – Spatial motion vector neighbours

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP − 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line.

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- If one of the following conditions is true, the variable isScaledFlagLX is set equal to 1.

- the prediction unit covering luma location ( xA0, yA0 ) is available and PredMode is not MODE_INTRA.

- the prediction unit covering luma location ( xA1, yA1 ) is available and PredMode is not MODE_INTRA.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 − MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xAk ][ yAk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- If  PicOrderCntVal – PicOrderCnt( RefPicListListA( refIdxA ) ) is equal to 0, or PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) is equal to 0, availableFlagLXA is set equal to 0.

- When availableFlagLXA is equal to 1, mvLXA is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-44)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-45)

mvLXA = Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvLXA ) * 		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-46)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListA( refIdxA ) ) )	(8-47)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )	(8-48)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0− MinPuSize , xB2 = xP − MinPuSize and yBk = yP − 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and its extended line.

- If yP−1 is less than yCtb, the following applies.

xB0 = (xB0>>Log2MinCbSize)<<Log2MinCbSize) + ((xB0>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-49)
xB1 = (xB1>>Log2MinCbSize)<<Log2MinCbSize) + ((xB1>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-50)
xB2 = (xB2>>Log2MinCbSize)<<Log2MinCbSize) + ((xB2>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-51)

where MinPuSize = 1 << ( Log2MinCbSize − 1)

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPSW, xB1 = xB0 − MinPuSize , and xB2 =  xP − MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xBk ][ yBk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- When isScaledFlagLX is equal to 0 and availableFlagLXB is equal to 1,  mvLXA is set equal to mvLXB and refIdxA is set equal to refIdxB and availableFlagLXA is set equal to 1.

- When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- If  PicOrderCntVal – PicOrderCnt( RefPicListListB( refIdxB ) ) is equal to 0, or PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) is equal to 0, availableFlagLXB is set equal to 0.

- When availableFlagLXB is equal to 1 and PicOrderCnt( RefPicListListB( refIdxB ) ) is not equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), mvLXB is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-52)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-53)

mvLXB =Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvLXA ) * 
	( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )		(8-54)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListB( refIdxB ) ) )	(8-55)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )	(8-56)

##### Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( picX, refIdx, LX ) returns the picture order count PicOrderCntVal of the reference picture with index refIdx from reference picture list LX of the picture picX and is specified as follows.

RefPicOrderCnt( picX, refIdx, LX ) = PicOrderCnt(RefPicListLX( refIdx ) of the picture picX)	(8 141)

Depending on the values of slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the picture that contains the collocated partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList1[ collocated_ref_idx ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList0[ collocated_ref_idx ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- The variable colPu is derived as follows

yPRb = yP + nPSH	(8-57)

  - If ( yP >> Log2CtbSize ) is equal to ( yPRb >> Log2CtbSize ), the horizontal component of the right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-58)

and the variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

  - Otherwise ( ( yP >> Log2CtbSize ) is not equal to ( yPRb >> Log2CtbSize ) ), colPu is marked as "unavailable".

- When colPu is coded in an intra prediction mode or colPu is marked as "unavailable", the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 )	(8-59)

yPCtr = ( yP + ( nPSH >> 1 )	(8-60)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If one of the following conditions is true, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

  - colPu is coded in an intra prediction mode.

  - colPu is marked as "unavailable".

  - colPic is marked as "unused for temporal motion vector prediction".

  - enable_temporal_mvp_flag is equal to 0.

- Otherwise, the motion vector mvCol, the reference index refIdxCol, and the reference list identifier listCol are derived as follows.

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to MvL1[ xPCol ][ yPCol ], RefIdxL1[ xPCol ][ yPCol ], and L1, respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to MvL0[ xPCol ][ yPCol ], RefIdxL0[ xPCol ][ yPCol ], and L0, respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

      - If PicOrderCnt( pic ) of every picture pic in every reference picture lists is less than or equal to PicOrderCntVal, mvCol, refIdxCol, and listCol are set equal to MvLX[ xPCol ][ yPCol ], RefIdxLX[ xPCol ][ yPCol ] and LX, respectively with X being the value of X this process is invoked for.

      - Otherwise (PicOrderCnt( pic ) of at least one picture pic in at least one reference picture list is greater than PicOrderCntVal, mvCol, refIdxCol and listCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxLN[ xPCol ][ yPCol ] and LN, respectively with N being the value of collocated_from_l0_flag.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) is equal to PicOrderCntVal - PicOrderCnt( RefPicListLX ( refIdxLX ) ),

mvLXCol = mvCol	(8-61)

  - Otherwise, if PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) is equal to 0, or PicOrderCntVal – PicOrderCnt( RefPicListLX ( refIdxLX ) ) is equal to 0, availableFlagLXCol is set equal to 0.

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + ( Abs( td ) >>1 ) ) / td	(8-62)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-63)

mvLXCol =  Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvCol ) * 
	( (Abs( DistScaleFactor * mvCol ) + 127 ) >> 8 ) ) (8-64)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) )	(8-65)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX ( refIdxLX ) ) )	(8-66)

###### Weighted sample prediction process

Inputs to this process are:

–	the same as specified in subclause Error: Reference source not found.

–	variables for weighted prediction logWDC, logWDC, w0C, w1C, o0C, o1C with C being replaced by L for luma samples and, when chroma_format_idc is not equal to 0, Cb and Cr for chroma samples.

Outputs of this process are:

–	the same as specified in subclause Error: Reference source not found.

The prediction sample predSamples[ x, y ] with x = 0..(nPSW)−1 and y = 0..(nPSH)−1, with H being replaced by Y for luma samples and by C for chroma samples are derived as follows:

–	If the predFlagL0 is equal to 1 and predFlagL1 is equal to 0, the prediction samples are derived as follows:

if( logWDC >= 1 ) 
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL0 [ x, y ] * w0C + 2logWDC − 1 ) >> logWDC ) + o0C ) 	 (8-67)
else
	predSamples[ x, y ] = Clip1H( predSamplesL0 [ x, y ] * w0C + o0C ) 	(8-68)

–	Otherwise, if the predFlagL0 is equal to 0 and predFlagL1 is equal to 1, the final predicted sample values predSamples [ x, y ] are derived by

if( logWDC >= 1 )
	predSamples[ x, y ] = Clip1H( ( ( predSamplesL1 [ x, y ] * w1C + 2logWDC − 1 ) >> logWDC ) + o1C ) 	(8-69)
else
	predSamples[ x, y ] = Clip1H( predSamplesL1 [ x, y ] * w1C + o1C ) 	(8-70)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1), if RefPicOrderCnt( currPic, refIdxL0, L0) is equal to RefPicOrderCnt( currPic, refIdxL1, L1) and mvL0 is equal to mvL1, the final predicted sample values predSamples [ x, y ] are derived by

predSamples[ x, y ] = Clip1H( ( predSamplesL0 [ x, y ] * ( w0C + w1C ) + 
			 ( ( o0C + o1C + 1 ) << logWDC ) ) >> ( logWDC + 1 ) )	(8-71)

–	Otherwise, the final predicted sample values predSamples[ x, y ] are derived by

predSamples[ x, y ] = Clip1H( ( predSamplesL0 [ x, y ] * w0C + predSamplesL1 [ x, y ] * w1C + 
			 ( ( o0C + o1C + 1 ) << logWDC ) ) >> ( logWDC + 1 ) )	(8-72)

Where the variables logWDC, o0C, o1C, and w0C, w1C are derived as follows.

–	If weighted_bipred_idc is equal to 2 in B-slices, implicit mode weighted prediction is used as follows:

logWDc = 5+shift1		(8-73)

o0C = 0				(8-74)

o1C = 0				(8-75)

The variable WeightScaleFactor is derived from the values currPoc, refIdxL0 and refIdxL1 as follows:

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )	(8-76)

td = Clip3( −128, 127, PicOrderCnt( RefPicListL1( refIdxL1 ) ) 
		 – PicOrderCnt( RefPicListL0( refIdxL0 ) ) )	(8-77)

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-78)

WeightScaleFactor = Clip3( −1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-79)

The variables w0C and w1C are derived as follows.

–	If PicOrderCnt( RefPicListL0( refIdxL0 ) ) is equal to PicOrderCnt( RefPicListL1( refIdxL1 ) ) or ( WeightScaleFactor >> 2 ) < −64 or ( WeightScaleFactor >> 2 ) > 128 or tb is equal to td or tb is equal to 0, the following applies.

w0C=32				(8-80)

w1C=32				(8-81)

–	Otherwise;

w0C = 64 – (WeightScaleFactor >> 2)		(8-82)

w1C = WeightScaleFactor >> 2		(8-83)

–	Otherwise, if both weighted_bipred_idc and ref_pic_list_combination_flag are equal to 1 in B slice, combined explicit mode weighted prediction is used as follows:

–	If C is equal to L for luma samples,

logWDc = luma_log2_weight_denom+ shift1		(8-84)

w0C = LumaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ]	(8-85)

w1C = LumaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ]	(8-86)

o0C = luma_offset_lc[ RefIdxLCToRefIdxLx[ refIdxL0 ] ] * ( 1 << ( BitDepthY − 8 ) )	(8-87)

o1C = luma_offset_lc[ RefIdxLCToRefIdxLx[ refIdxL1 ] ] * ( 1 << ( BitDepthY − 8 ) )	(8-88)

–	Otherwise (C is equal to Cb or Cr for chroma samples, with iCbCr = 0 for Cb, iCbCr = 1 for Cr),

logWDc = ChromaLog2WeightDenom + shift1	(8-89)

w0C = ChromaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ][ iCbCr ]	(8-90)

w1C = ChromaWeightLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ][ iCbCr ]	(8-91)

o0C = ChromaOffsetLC[ RefIdxLCToRefIdxLx[ refIdxL0 ] ][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-92)

o1C = ChromaOffsetLC[ RefIdxLCToRefIdxLx[ refIdxL1 ] ][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-93)

–	Otherwise (weighted_pred_flag is equal to 1 in P slice or ref_pic_list_combination_flag is equal to 0 in B-slice) explicit mode weighted prediction is used as follows:

–	If C is equal to L for luma samples,

logWDc = luma_log2_weight_denom+ shift1		(8-94)

w0C = LumaWeightL0[refIdxL0]		(8-95)

w1C = LumaWeightL1[refIdxL1]		(8-96)

o0C = luma_offset_l0[refIdxL0] * ( 1 << ( BitDepthY − 8 ) )	(8-97)

o1C = luma_offset_l1[refIdxL1] * ( 1 << ( BitDepthY − 8 ) )	(8-98)

–	Otherwise (C is equal to Cb or Cr for chroma samples, with iCbCr = 0 for Cb, iCbCr = 1 for Cr),

logWDc = ChromaLog2WeightDenom + shift1	(8-99)

w0C = ChromaWeightL0[refIdxL0][ iCbCr ]		(8-100)

w1C = ChromaWeightL1[refIdxL1][ iCbCr ]		(8-101)

o0C = ChromaOffsetL0[refIdxL0][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-102)

o1C = ChromaOffsetL1[refIdxL1][ iCbCr ] * ( 1 << ( BitDepthC − 8 ) )	(8-103)