| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0439 |
| --- | --- |

| Title: | On Explicit Weighted Prediction Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seungwook Park Yongjoon Jeon Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com seungwook.park@lge.com yongjoon.jeon@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution claims that the current weighted prediction signalling may have problem for signalling weighted prediction for LC. In the current scheme, when weighted prediction is signalled for LC, the weighted prediction for L0 and L1 are derived and the number of loop for signalling weight and offset for reference pictures in LC is controlled by num_ref_idx_lc_active_minus1. A problem may occur when encoder sets num_ref_idx_lc_active_minus1 to less than the number of unique reference pictures that are exists in L0 and L1, so that derivation of weighted prediction for L0 and L1 might not be possible. Also, the current signalling method is claimed to be complicated so that simplification might be needed.

The proposals in this contribution can be summarized as follow:

- When weighted prediction information is signalled for reference pictures in reference picture list combination (LC), the number of loop for signalling weight and offset should not use num_ref_idx_lc_active_minus1, rather, the number of loop should be controlled by separately signalled number, that is, num_wp_present_minus1.

- The current signalling of weighted prediction is simplified as follow:

  - If the slice is a P slice, then signal weighted prediction information for L0.

  - Else if the slice is a B slice, then weighted prediction information for LC.

The proposed item 2 is similar to the option 3 proposed by JCTVC-I0335.

The proponents apologize for registering and uploading the contribution late

# Introduction

As per Committee Draft of HEVC specification (JCTVC-H1003), the signalling of explicit weighted prediction is as follow:

- If the slice is a P slice, then weighted prediction information for reference picture list 0 (i., L0) is signalled

- If the slice is a B slice, depending on the value of ref_pic_list_combinaion_flag,

  - If the flag is 0, weighted prediction information for reference picture list 0 (i., L0) and reference picture list 1 (i.e., L1) are signalled

  - Else, weighted prediction information for reference picture list combination (i., LC) is signalled. Weighted prediction information for L0 and L1 are then derived from the signalled weighted prediction information for LC.

We have identified two possible problems in the current signalling design.

Problem #1 – Dependency to num_ref_idx_lc_active_minus1

As described above, when the slice is a B slice and rep_pic_list_combination_flag is 1, then weighted prediction information is only signalled for LC. The number of signalled weighted prediction is controlled by num_ref_idx_lc_active_minus1.Such technique may have problem since does not really reflect the real number of reference pictures in L0 and L1. Encoder may assign smaller value for num_ref_idx_lc_active_minus1 than the actual number of different reference pictures in L0 and L1.

Consider the situation illustrated in the following figure. L0 contains reference pictures with POC 4, POC 2, and POC 1 and L1 contains reference pictures with POC 6, POC 4, and POC2. Therefore, under normal condition, encoder would assign num_ref_idx_lc_active_minus1 as 3 since there are 4 different reference pictures exist in L0 and L1, that is, reference pictures with POC 4, POC 6, POC 2, and POC1 as illustrated in Figure 1(a). However, there is no restriction that govern that encoder cannot assign smaller number for num_ref_idx_lc_active_minus1, if it wants to. For example as shown in Figure 1(b), it is possible that encoder assigns num_ref_idx_lc_active_minus1 to 2, meaning that it only wants to use 3 reference pictures for unidirectional motion prediction.

When situation such as illustrated in Figure 1(b) occurs, that is encoder assigns num_ref_idx_lc_active_minus1 with some value smaller than the actuall number of different reference pictures in L0 and L1 - 1, then the current explicit weighted prediction signalling may have problem for it cannot correctly derive weighted prediction information for L0 and L1 from LC.

    - Ideal case. There are 4 different reference pictures in LC, then num_ref_idx_lc_active_minus1 is set to 3

- Ideal case. There are 4 different reference pictures in LC, then num_ref_idx_lc_active_minus1 is set to 3

Figure 1 – Possible problem with explicit weighted prediction signalling with fix num_ref_idx_lc_active_minus1

Problem #2 – Complicated Signaling Process

As described above, the signalling of explicit weighted prediction information is complicated since there are parameters (slice type, ref_pic_list_combination_flag, num_ref_idx_lc_active_minus1, etc) to control to which weighted prediction is attached to (i.e. to L0 only, to L0 and L1, to LC).

It will be much better if the signalling of explicit weighted prediction information is simplified.

# Proposed Solutions

## For problem #1

To overcome problem #1, we propose:

- When weighted prediction information is signalled for reference pictures in reference picture list combination (LC), the number of weighted prediction information should be made independent from num_ref_lc_active_minus1

- When weighted prediction information is signalled for reference pictures in reference picture list combination (LC), the number of weighted prediction information should be signalled separately, that is by a new syntax element called ‘num_wp_present_minus1’.

The changes in syntax and semantics that are applied to current syntax and semantics of weighted prediction are as follow:

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( slice_type = = P ¦ ¦ ( slice_type = = B && ref_pic_list_combination_flag = = 0 ) ) { |  |
| … |  |
| } |  |
| if( slice_type = = B ) { |  |
| if( ref_pic_list_combination_flag = = 0 ) { |  |
| … |  |
| } else { |  |
| num_wp_present_minus1 | ue(v) |
| for( i = 0; i <= num_wp_present_minus1; i++ ) { |  |
| luma_weight_lc_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_lc[ i ] | se(v) |
| delta_luma_offset_lc[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_lc_flag | u(1) |
| if( chroma_weight_lc_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_lc[ i ][ j ] | se(v) |
| delta_chroma_offset_lc[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

num_wp_present_minus1 + 1 specifies number of weighted prediction information present. The value of delta_num_wp_present shall be in the range of 0 to num_ref_idx_l0_minus1 + num_ref_idx_l1_minus1 + 1, inclusive.

## For problem #2

The proposed signalling of weighted prediction is as follow:

- If the slice is a P slice, then weighted prediction information for reference picture list 0 (i., L0) is signalled

- If the slice is a B slice, then weighted prediction information for reference picture list combination (i., LC) is signalled

  - Weighted prediction information for L0 and L1 are then derived from the signalled weighted prediction information for LC.

Compared to current signalling method, the proposed signalling method is much simpler since there are only two choices, that is: signal weighted prediction information for L0 or signal weighted prediction information for LC.

The changes in syntax and semantics that are applied to current syntax and semantics of weighted prediction are as follow:

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( slice_type = = P ¦ ¦ ( slice_type = = B && ref_pic_list_combination_flag = = 0 ) ) { |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| luma_weight_l0_flag | u(1) |
| if( luma_weight_l0_flag ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l0_flag | u(1) |
| if( chroma_weight_l0_flag ) |  |
| for( j =0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| Else if( slice_type = = B ) { |  |
| for( i = 0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |
| luma_weight_lc_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_lc[ i ] | se(v) |
| luma_offset_lc[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_lc_flag | u(1) |
| if( chroma_weight_lc_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_lc[ i ][ j ] | se(v) |
| delta_chroma_offset_lc[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

luma_log2_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors. The value of luma_log2_weight_denom shall be in the range of 0 to 7, inclusive.

delta_chroma_log2_weight_denom is the difference of the base 2 logarithm of the denominator for all chroma weighting factors.

The variable ChromaLog2WeightDenom is specified by luma_log2_weight_denom + delta_chroma_log2_weight_denom and it shall be in the range of 0 to 7, inclusive.

luma_weight_l0_flag equal to 1 specifies that weighting factors for the luma component of list 0 prediction are present. luma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

delta_luma_weight_l0[ i ] is the difference of the weighting factor applied to the luma prediction value for list 0 prediction using RefPicList0[ i ].

The variable LumaWeightL0[ i ] is specified by (1 << luma_log2_weight_denom ) + delta_luma_weight_l0[ i ]. When luma_weight_l0_flag is equal to 1, the value of LumaWeightL0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, LumaWeightL0[ i ] is inferred to be equal to 2luma_log2_weight_denom for RefPicList0[ i ].

delta_luma_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ]. The value of luma_offset_l0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, luma_offset_l0[ i ] is inferred as equal to 0 for RefPicList0[ i ].

chroma_weight_l0_flag equal to 1 specifies that weighting factors for the chroma prediction values of list 0 prediction are present. chroma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

delta_chroma_weight_l0[ i ][ j ] is the difference of the weighting factor applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaWeightL0[ i ][ j ] is specified by ( 1 << ChromaLog2WeightDenom ) + delta_chroma_weight_l0[ i ][ j ]. When chroma_weight_l0_flag is equal to 1, the value of ChromaWeightL0[ i ][ j ] shall be in the range of −128 to 127, inclusive.  When chroma_weight_l0_flag is equal to 0, ChromaWeightL0[ i ][ j ] is inferred to be equal to 2ChromaLog2WeightDenom for RefPicList0[ i ].

delta_chroma_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaOffsetL0[ i ][ j ] is specified as follows:

shift = 1 << ( BitDepthC − 1 )

ChromaOffsetL0[ i ][ j ] = (delta_chroma_offset_l0[i][j] – 
				( (shift*ChromaWeightL0[ i ][ j ]) >> ChromaLog2WeightDenom ) − shift )

The variable ChromaOffsetL0[ i ][ j ] shall be in the range of −127 to 128, inclusive. When chroma_weight_l0_flag is equal to 0, ChromaOffsetL0[ i ][ j ] is inferred to be equal to 0 for RefPicList0[ i ].

luma_weight_lc_flag, delta_luma_weight_lc, delta_luma_offset_lc, chroma_weight_lc_flag, delta_chroma_weight_lc, delta_chroma_offset_lc have the same semantics as luma_weight_l0_flag, delta_luma_weight_l0, luma_offset_l0, chroma_weight_l0_flag, delta_chroma_weight_l0, delta_chroma_offset_l0, respectively, with l0, list 0, and List0 replaced by lc, list combination, and List combination, respectively.

Note:

- This simplification is the same as the option 3 in [2].

- The proposals in 2.1 and 2.2 can easily be combined.

# Conclusion

In this contribution, two possible problems are identified and solutions for each problem are given. Further, We recommend that the identified problems are discussed during and proposed solutions are to be considered to be adopted for overcome the problems.

# References

[1] JCTVC-H1003, “High efficiency video coding (HEVC) text specification draft 6,” 8th JCT-VC Meeting, San Jose, US.

[2] JCTVC-I0335, “On weighted prediction signaling,” 9th JCT-VC Meeting, Geneva, CH.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).