| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0440 |
| --- | --- |

| Title: | Cross-verification of JCTVC-I0186 on intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of JCTVC-I0186 on intra mode coding. The cross-verification task was successfully completed and it is confirmed that proponent's results are accurate. It is recommended that Method2 or Method3 is considered for adoption in DIS.

- Analysis and simulation results

The software provided by Canon was inspected and it was confirmed that the following methods are implemented in the software according to the JCTVC-I0186 description.

- Method1 is to set the left CU intra prediction mode to MPM0. This method enables codec to keep the DC and Planar modes in the first two MPMs and determine the MPM0 with less computation.

- Method2 is to set Planar to MPM2 and modify the position of Angular mode checking. This method reduces the number of postponed MPM sorting operations significantly because of the fixed MPM2 mode.

- Method3 is similar to Method2: it sets DC instead of Planar to MPM2; the rest is the same as Method2.

The software provided by Canon was successfully compiled and run on Window-based computing platform. Tables 1, 2, and 3 shows the simulation results of Method1, Method2, and Method3, respectively. It was confirmed that all results are identical to Canon's results.

Table1: Anchor versus Method1.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.01% | -0.14% | -0.09% | 0.01% | -0.07% | -0.06% |
| Class B | 0.03% | -0.03% | -0.02% | 0.04% | 0.02% | -0.01% |
| Class C | 0.00% | -0.02% | -0.04% | 0.00% | 0.01% | 0.00% |
| Class D | 0.00% | 0.03% | 0.00% | 0.01% | -0.02% | 0.00% |
| Class E | 0.02% | -0.02% | 0.04% | 0.03% | 0.03% | 0.09% |
| Overall | 0.01% | -0.04% | -0.03% | 0.02% | -0.01% | 0.00% |
|  | 0.01% | -0.04% | -0.03% | 0.02% | -0.01% | 0.00% |
| Class F | -0.01% | 0.01% | -0.07% | 0.02% | 0.10% | -0.05% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table2: Anchor versus Method2.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.12% | -0.13% | -0.19% | -0.09% | -0.24% | -0.14% |
| Class B | -0.06% | -0.10% | -0.05% | -0.03% | -0.11% | -0.05% |
| Class C | -0.07% | -0.02% | 0.02% | -0.07% | -0.05% | 0.01% |
| Class D | -0.09% | -0.02% | -0.07% | -0.09% | -0.04% | -0.02% |
| Class E | -0.01% | 0.03% | -0.05% | 0.03% | -0.05% | 0.01% |
| Overall | -0.07% | -0.06% | -0.07% | -0.05% | -0.10% | -0.04% |
|  | -0.07% | -0.05% | -0.07% | -0.05% | -0.09% | -0.04% |
| Class F | -0.10% | -0.01% | -0.01% | -0.04% | -0.10% | -0.13% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table3: Anchor versus Method3.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.05% | -0.07% | -0.06% | -0.03% | -0.05% | 0.03% |
| Class B | -0.04% | -0.12% | -0.08% | -0.02% | -0.12% | -0.10% |
| Class C | -0.06% | -0.06% | -0.02% | -0.06% | -0.06% | -0.06% |
| Class D | -0.08% | -0.04% | -0.08% | -0.09% | -0.03% | -0.03% |
| Class E | 0.11% | 0.10% | -0.12% | 0.13% | -0.08% | -0.15% |
| Overall | -0.03% | -0.05% | -0.07% | -0.02% | -0.07% | -0.06% |
|  | -0.03% | -0.04% | -0.07% | -0.02% | -0.05% | -0.06% |
| Class F | -0.10% | 0.27% | 0.01% | -0.05% | 0.06% | -0.05% |
| Enc Time[%] | 97% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

# Patent rights declarations

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).