| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0445 |
| --- | --- |

| Title: | Cross-check report of JCTVC-I0149 on transform split pattern of NSQT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yongbing Lin | Tel: Email: | +86 10 60612142 yblin@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This document reports cross-check results of JCTVC-I0149 on coherent transform split pattern of NSQT. The coding order of transform units in a 64x64 CU is unified with other CU sizes. It is confirmed that all bit-rate and PSNR values match exactly with those reported by proponent, and encoding/decoding run time has similar character.

# Introduction

This document reports cross-check results of JCTVC-I0149 on coherent transform split pattern of NSQT[1]. NSQT unit is always split into four non square transform units. The splitting continues until the split non square transform is applicable to the current CU size. Therefore the coding order of transform units in a 64x64 CU is unified with other CU sizes. At the meantime, CBF for transform unit at the layer of Tr Depth=1 is not coded since 64x16 transform is not applicable under common test conditions.

# Test results

We got source code from proponent and independently compiled the code with VS2005, and ran in environment of Windows server 2003 R2 64bits. The common test conditions and software reference configurations are followed in our simulation [2]. HM6.1 is used as anchor. The test results are summarized in Table 1. It shows coding efficiency of 0.0%, -0.1%, 0.0% in RA-HE10 and 0.1%, -0.1%, -0.3% in LB-HE10. It is confirmed that all bit-rate and PSNR values match exactly with those reported by proponent, and encoding/decoding run time has similar character.

Table 1. Test results of the proposed method, comparing to HM6.1

|  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.2% | 0.0% |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | -0.3% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.2% |
| Class E |  |  |  | 0.1% | -0.6% | -0.4% |
| Overall | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% | -0.3% |
|  | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% | -0.2% |
| Class F | 0.0% | -0.1% | 0.0% | 0.1% | -0.5% | -0.3% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

# Recommendation

The test results reported in JCTVC-I0149 is confirmed.

# References

- Jungsun Kim, Byeongmoon Jeon, “Coherent transform split pattern for non square transform units,” JCT-VC Document, JCTVC-I0149, Geneva, CH, April, 2012

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-H1100, San Jose, US, Feb. 2012