| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0447 |
| --- | --- |

| Title: | Crosscheck of template-based modeling for coefficient coding in JCTVC-I0383 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang }@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the parallel context processing for significant_coeff_flag , coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag in all TUs proposed by Qualcomm in JCTVC-I0383. In this contribution, the HM-6.0 mask-based context modelling in large TUs are applied to all TUs. The number of nonzero coefficients is utilized for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag context formation. The mask pattern is modified when horizontal and vertical scan are applied for 2-bins parallel context assignment. The verification task was done successfully, and the results match exactly those provided by Qualcomm.

# Verification

Qualcomm provided the HM source code of their proposed codec described in JCTVC-I0383 [1]. Two tests are evaluated. One is for normal QP (QP = 22, 27, 32, 37), and the other is for low QP (QP = 12, 17, 22, 27). The verifications are carried out by setting the following parameters in the provided codes:

#define SIGMAP_LEVEL_MOD        1

Six configurations (AI-Main, AI-HE10, RA-Main, RA-HE10, LB-Main, LB-HE10) were run for the proposed template-based context assignment with the provided code. The running of the tests includes encoding, decoding, and collecting the run time of encoding and decoding processes.

# Results

The BD-rates from our experimental results match exactly those provided by Qualcomm. The encoding time and decoding time are roughly the same to those provided by Qualcomm. The detailed results are listed in JCTVC-I0447_normal_QP.xls, JCTVC-I0447_low_QP.xls.

# Conclusion

The results presented by Qualcomm in JCTVC-I0383 are confirmed.

# References

- W. -J. Chien, J. Chen, J. Sole, M. Karczewicz, “Template-based context modeling for coefficient coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0383, April, 2012.