| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0449 |
| --- | --- |

| Title: | AHG7: Cross verification of “Bi-pred restriction for small PUs” (JCTVC-I0297) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki Sony City Osaki, 2-10-1 Osaki Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for JCTVC-I0297 [1]. In this contribution, the source code and simulation results were checked. And the cross-verification task was successfully completed and it is confirmed that proponent's results are accurate.

# The software

We checked the software that is provided by proponent. It was modified as below:

- For common modification, Small PU block (width + height <= 12) with bi-prediction is restricted.

  - Actually, the bi-prediction 4x8 and 8x4 are restricted.

- Proposal #1

  - The marge candidates are replaced from bi-prediction to uni-prediction when the small PU block after merge candidate process.

- Proposal #2

  - In motion compensation process, it is processed as uni-prediction when the PU is bi-pred 4x8 or 8x4.

# Cross check simulation

## Test conditions

The common test condition [2] is followed for this cross check.

## Simulation results

The simulation results are shown in Table 1 and Table 2. It was checked that those results were matched with proponent’s results.

### Proposal #1

Table 1 simulation results for proposal #1

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.4% | 0.0% | 0.2% | 0.1% |
| Class B | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.4% | 0.4% | 0.4% | 0.3% | 0.3% | 0.2% |
| Class D | 0.6% | 0.5% | 0.7% | 0.5% | 0.5% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 0.3% | 0.4% | 0.2% | 0.3% | 0.2% |
|  | 0.3% | 0.3% | 0.4% | 0.2% | 0.3% | 0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 93% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -0.2% | -0.1% | 0.2% | -0.1% | 0.0% |
| Class C | 0.5% | 0.6% | 0.4% | 0.4% | 0.6% | 0.5% |
| Class D | 0.6% | 0.3% | 0.8% | 0.6% | 0.2% | -0.2% |
| Class E | 0.2% | 0.3% | 0.0% | 0.1% | 0.0% | 0.1% |
| Overall | 0.4% | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% |
|  | 0.4% | 0.3% | 0.3% | 0.3% | 0.2% | 0.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 91% |  |  | 93% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

### Proposal #2

Table 2 simulation results for proposal #2

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.0% | 0.2% | 0.1% | 0.2% | 0.0% |
| Class B | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.4% | 0.5% | 0.4% | 0.3% | 0.3% | 0.2% |
| Class D | 0.6% | 0.4% | 0.6% | 0.5% | 0.5% | 0.5% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 0.3% | 0.4% | 0.3% | 0.3% | 0.2% |
|  | 0.3% | 0.3% | 0.4% | 0.3% | 0.3% | 0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 93% |  |  | 95% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -0.2% | -0.1% | 0.1% | 0.2% | 0.3% |
| Class C | 0.4% | 0.6% | 0.3% | 0.4% | 0.6% | 0.7% |
| Class D | 0.6% | 0.6% | 1.0% | 0.6% | 0.1% | 0.5% |
| Class E | 0.2% | 0.0% | 0.4% | 0.1% | 0.0% | 0.0% |
| Overall | 0.4% | 0.2% | 0.4% | 0.3% | 0.3% | 0.4% |
|  | 0.4% | 0.3% | 0.4% | 0.3% | 0.3% | 0.4% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 91% |  |  | 93% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusions

This contribution reported the result of cross check for JCTVC-I0297. The software is checked. And the simulation results were verified.

# References

[1] S. Fukushima, “AHG7: Bi-pred restriction for small PUs”, JCTVC-I0297, JCT-VC 9th meeting, Geneva 2011

[2] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100, JCT-VC 8th meeting, Geneva 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).