| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0450 |
| --- | --- |

| Title: | Crosscheck report of Simplification on default quantization matrix signalling (JCTVC-I0101) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the crosscheck result of JCTVC-I0101, proposal of Simplification on default quantization matrix signalling by ETRI.  It has been verified that the results presented in JCTVC-I0101 are accurate.

# Cross Check Procedure

The source code corresponds to JCTVC-I0101 was provided by ETRI, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.

Using the provided software, all the referenced data are generated with the following macro setting:

#define ETRI_SCALING_LIST_DEFAULT_SIGNALING    0

while all the tested data are generated with the following macro setting:

#define ETRI_SCALING_LIST_DEFAULT_SIGNALING    1

According to the test description in JCTVC-H1100 (common test condition) with modifications to configuration file to use a quantization matrix file, the following performance was measured:

- PSNR and bitrate results

- Quantization matrices coding performance using accompanied quantization matrix files in JCTVC-I0101.

# Results

## Simulation results

Cross check results for JCTVC-I0101 are summarized in Table 1, where accompanied “scaling_list_symmetry1_rd.txt” is used for the quantization matrices to be coded.  Since the difference between reference results and tested results are 12bits per sequence as reported in Table 2 later, the difference is not visible in Table 1.

Table 1: BD-rate results for JCTVC-I0101 using scaling_list_symmetry1_rd.txt

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

## Quantization Matrices Coding Performance

Quantization matrices coding performance using accompanied 3 quantization matrix files are measured as in Table 2.  It has been verified that the numbers reported in Table 2 are exactly identical to the numbers reported in JCTVC-I0101 contribution.

Table 2: Quantization matrices coding results for JCTVC-I0101

| QM bits | symmetry1 modified | symmetry7 modified | asymmetry7 modified |
| --- | --- | --- | --- |
| I0101 reference | 1469 | 2107 | 2459 |
| I0101 tested | 1457 | 2087 | 2439 |

## Theoretical analysis of the influence made by JCTVC-I0101

JCTVC-I0101 contribution also contains analytical study of the influence to coded bits required for quantization matrices coding.  According to JCTVC-I0101, the best case is that all but one quantization matrices use default matrices and its effect is 152bits saving, while the worst case is that all but the first matrices (with matrixID equal to 0) in each size use copy mode and copying from the previous matrices and its effect is 32bits increase.

With HM6 anchor, it requires 10bits to code a default matrix, as it requires 1 bit to code scaling_list_pred_mode_flag (with the value of 1) and 9 bits to code scaling_list_delta_coef (with the value of -8).  In contrast, with the proposed method of JCTVC-I0101, it requires 2bits to code a default matrix, as it requires 1 bit to code scaling_list_pred_mode_flag (with the value of 0) and 1 bit to code scaling_list_pred_matrix_id_delta (with the value of 0).  Accordingly, the proposed method of JCTVC-I0101 is able to save 8bits per default quantization matrix, and it will lead to 152bits saving in total which equals to 8bits times 19 (of 20 quantization matrices) in the best case.

Meanwhile, with HM6 anchor, it requires 2bits to represent a copy mode from previous matrix, as it requires 1 bit to code scaling_list_pred_mode_flag (with the value of 0) and another 1 bit to code scaling_list_pred_matrix_id_delta (with the value of 0).  In contrast, with the proposed method of JCTVC-I0101, it requires 4bits to represent a copy mode from previous matrix, as it requires 1 bit to code scaling_list_pred_mode_flag (with the value of 0) and 3 bits to code scaling_list_pred_matrix_id_delta (with the value of 1).  Accordingly, the proposed method of JCTVC-I0101 can increase 2bits per quantization matrix with copy mode, and it will lead to 32bits increase in total which equals to 2bits times 16 (of 20 quantization matrices) in the worst case.

Hence, it has been verified that the theoretical analysis of the influence presented in JCTVC-I0101 is also accurate.

# Conclusion

It has been verified that the results of “Simplification on default quantization matrix signalling” provided as JCTVC-I0101 are accurate.