| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0451 |
| --- | --- |

| Title: | Cross-check on newer quantization matrices for HEVC (JCTVC-I0126) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Il-Koo Kim | Email: | ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Cross-check report on JCTVC-I0126

In this document, a cross-checking report on JCTVC-I0126 (newer quantization matrices for HEVC) is provided. In the contribution, new quantization matrices for 4x4 and 8x8 are proposed.

A test is conducted to verify effect of proposed quantization matrices compared with current HM matrices.

Software was provided by the proponent of JCTVC-I0126. It is confirmed that the source code has modified according to the description and the results under common test condition Error: Reference source not found are same with the provided results from the proponent as summarized in Error: Reference source not found.

Table I Coding efficiency comparison between HM6.0 quantization matrices and proposed quantization matrices

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.3% | 0.0% | -0.1% | -0.2% |
| Class B | -0.5% | 0.2% | 0.2% | -0.4% | 0.4% | 0.4% |
| Class C | -1.1% | 0.2% | 0.2% | -0.9% | 0.4% | 0.5% |
| Class D | -1.2% | 0.4% | 0.2% | -1.1% | 0.6% | 0.4% |
| Class E | -0.2% | 0.2% | 0.2% | -0.1% | 0.2% | 0.3% |
| Overall | -0.6% | 0.1% | 0.1% | -0.5% | 0.3% | 0.3% |
|  | -0.6% | 0.1% | 0.1% | -0.5% | 0.3% | 0.2% |
| Class F | -1.7% | -0.5% | -0.9% | -1.6% | 0.1% | -0.4% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.5% | -0.8% | 0.0% | -0.2% | -0.6% |
| Class B | -0.3% | 0.2% | 0.2% | -0.2% | 0.2% | 0.3% |
| Class C | -0.9% | 0.0% | 0.0% | -0.8% | 0.0% | 0.0% |
| Class D | -1.0% | 0.1% | 0.2% | -0.9% | 0.5% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.5% | 0.0% | -0.1% | -0.5% | 0.1% | 0.0% |
|  | -0.6% | -0.1% | -0.1% | -0.5% | 0.1% | 0.0% |
| Class F | -1.7% | -1.0% | -1.9% | -1.7% | -0.9% | -1.3% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.3% | 0.8% | -0.1% | 0.6% | 0.0% |
| Class C | -0.7% | 0.4% | 0.5% | -0.6% | 0.5% | 0.1% |
| Class D | -0.8% | 0.0% | 0.6% | -0.6% | 0.0% | 0.6% |
| Class E | -0.2% | -0.7% | -0.3% | -0.1% | -0.7% | -0.3% |
| Overall | -0.5% | 0.1% | 0.5% | -0.4% | 0.2% | 0.1% |
|  | -0.5% | 0.1% | 0.3% | -0.4% | 0.2% | 0.1% |
| Class F | -1.5% | -0.1% | -0.9% | -1.1% | 0.9% | -0.5% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |

# References

- F. Bossen, " Common test conditions and software reference configurations," Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, 8th Meeting: San Jose, CA, USA, 1-10 February, 2012