| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0452 |
| --- | --- |

| Title: | Cross-check on implicit TU split process for asymmetric partitions (JCTVC-I0308) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Il-Koo Kim | Email: | ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Cross-check report on JCTVC-I0308

In this document, a cross-checking report on JCTVC-I0308 (implicit TU split process for asymmetric partitions) is provided. In the contribution, implicit TU split process is proposed for asymmetric partition when QuadtreeTUMaxDepthInter is set to 1. Square transform which is fit inside smaller partition of asymmetric partition mode is chosen and applied to residual block without overlapping.

The first test is conducted to verify effect of proposed implicit TU split process compared with current HM design when a parameter, QuadtreeTUMaxDepthInter, of configuration file is set to 1. The second test is conducted to verify effect of proposed method compared with current HM design when two parameters, QuadtreeTUMaxDepthInter and NSQT, of configuration file are set to 1.

Software was provided by the proponent of JCTVC-I0308. It is confirmed that the source code has modified according to the description and the results under common test condition Error: Reference source not found are same with the provided results from the proponent as summarized in Error: Reference source not found and Error: Reference source not found.

Table I. Coding efficiency comparison between HM6.0 and proposed (when QuadtreeTUMaxDepthInter is set to 1 and NSQT are set to 0 in configuration file)

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.4% | -0.1% | -0.2% | 0.0% |
| Class B | -0.2% | -0.4% | -0.4% | -0.2% | -0.4% | -0.3% |
| Class C | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% |
| Class D | -0.3% | -0.3% | -0.1% | -0.3% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | -0.3% | -0.3% | -0.2% | -0.2% | -0.1% |
|  | -0.2% | -0.4% | -0.3% | -0.2% | -0.2% | -0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -1.4% | -1.1% | -0.3% | -1.1% | -0.8% |
| Class C | -0.4% | -0.8% | -0.7% | -0.4% | -0.5% | -0.2% |
| Class D | -0.5% | -1.5% | -0.9% | -0.5% | -0.8% | -1.2% |
| Class E | -0.2% | -0.7% | 0.2% | -0.1% | -0.1% | -0.4% |
| Overall | -0.4% | -1.1% | -0.7% | -0.4% | -0.7% | -0.7% |
|  | -0.4% | -1.1% | -0.7% | -0.4% | -0.7% | -0.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table II. Coding efficiency comparison between proposed and HM6.0 (when QuadtreeTUMaxDepthInter and NSQT are set to 1 in configuration file)

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.3% | -0.2% | -0.4% | -0.6% |
| Class B | -0.4% | -1.4% | -1.3% | -0.3% | -1.3% | -1.2% |
| Class C | -0.4% | -0.7% | -0.7% | -0.4% | -0.7% | -0.6% |
| Class D | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.7% | -0.7% | -0.3% | -0.7% | -0.7% |
|  | -0.3% | -0.7% | -0.7% | -0.3% | -0.7% | -0.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -2.8% | -2.9% | -0.8% | -2.7% | -2.8% |
| Class C | -0.8% | -1.8% | -1.8% | -0.7% | -1.5% | -1.8% |
| Class D | -0.7% | -0.8% | -0.3% | -0.6% | -0.6% | -0.3% |
| Class E | -1.6% | -2.1% | -2.8% | -1.3% | -2.9% | -1.7% |
| Overall | -1.0% | -1.9% | -2.0% | -0.8% | -1.9% | -1.7% |
|  | -1.0% | -1.9% | -1.9% | -0.8% | -1.9% | -1.8% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# References

- F. Bossen, " Common test conditions and software reference configurations," Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, 8th Meeting: San Jose, CA, USA, 1-10 February, 2012